# 3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215822

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.10.3.1

Source: 	OPPO

Title: 	On measurement without gaps for UEs reporting NeedForGapsInfoNR

Document for:	Approval

1	Introduction

Last RAN4 meeting had an initial discussion on RRM requirements for measurements without gap and identified several issues captured in [1]. We will provide our considerations on these issues.

2	Discussion

- Issue 3-1: [NFG] Whether interruption is expected when UE reports ’no-gap’ < Way forward >: - Option 1: Yes - Option 2: No - Option 3: Introduce additional UE capability to differentiate whether UE needs interruption

When ‘no-gap’ is reported, it means UE could measure target cells without a long gap duration. But whether interruption is needed is not clear and companies’ views seems are equally divided between option 1 and option 2. Sometimes, no interruption is expected. For example, when the target SSB is completely contained in the active BWP. In the other scenarios, RF tuning may still be required as discussed in NCSG. As a compromise, we support option 3 to introduce additional UE capability regarding the interruption.

Proposal-1: Support option 3: Introduce additional UE capability to differentiate whether UE needs interruption.

- Issue 3-2: [NFG] Interruption requirement, if allowed < Way forward >: - Option 1: (Visible) Take NCSG as a starting point. FFS the exact values - Option 2: (Invisible) Adopt deactivated SCell’s interruption requirement as a start point. FFS the interruption ratio (data dropping rate).

When interruption is allowed, we think NCSG pattern could be used as a starting point. For the interruption length, the VIL for NCSG could be reused, i.e. 1ms for FR1 and 0.75ms for FR2. As for the location, use the boundary for the corresponding SMTC as the starting/ending point for the interruption.

Proposal-2: Take NCSG as a starting point to define the interruption:

- For interruption length, reuse VIL

- For interruption location, use the boundary of SMTC as the starting/ending point

- Issue 3-4: [NFG] Requirement for intra-freq measurement without gap < Way forward >: - Option 1: Same as requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) - Option 2: Take requirements NCSG requirements as a starting point - Option 3: Take requirements in Section 9.39 of TS38.133 (inter-freq wo/ gap) as a starting point - Option 4: The related frequency layer should be counted in CSSF outside gap - Option 5: While discussing the measurement period, since similar with the same issue in pre-configured MG, some solution proposed during pre-configured MG discussion can be used for reference. - Issue 3-5: [NFG] Requirement for inter-freq measurement without gap < Way forward >: - Option 1: Take requirements in Section 9.3.9 of TS38.133 (intra-freq w/o gap) as a starting point - Option 2: Take requirements NCSG requirements as a starting point - Option 3: Take requirements in Section 9.3.9 of TS38.133 (inter-freq wo/ gap) as a starting point - Option 4: The related frequency layer should be counted in CSSF outside gap - Option 5: While discussing the measurement period, since similar with the same issue in pre-configured MG, some solution proposed during pre-configured MG discussion can be used for reference.

For defining measurement delay requirements, we prefer option 1 for intra-frequency without gap and option 3 for inter-frequency measurements without gap as a starting point. If using option 2 with NCSG requirements, there is no explicit parameter for measurement occasion pattern like VIRP. And we think the existing CSSF and Kp in section 9.2.5 and 9.3.9 should be followed. However for defining interruption or scheduling availability requirements, the requirements for NCSG are more appreciated.

Proposal-3: To define requirements for measurement without gap:

- Measurement delay requirements:

  - For intra-frequency: take intra-frequency without gap in section 9.2.5 as a starting point

  - For inter-frequency: take inter-frequency without gap in section 9.3.9 as a starting point

- Scheduling availability requirements: take NCSG as a starting point

3	Conclusion

This contribution gave our general views on RRM requirements without gaps and the following proposals:

Proposal-1: Support option 3: Introduce additional UE capability to differentiate whether UE needs interruption.

Proposal-2: Take NCSG as a starting point to define the interruption:

- For interruption length, reuse VIL

- For interruption location, use the boundary of SMTC as the starting/ending pointing

Proposal-3: To define requirements for measurement without gap:

- Measurement delay requirements:

  - For intra-frequency: take intra-frequency without gap in section 9.2.5 as a starting point

  - For inter-frequency: take inter-frequency without gap in section 9.3.9 as a starting point

- Scheduling availability requirements: take NCSG as a starting point

4	Reference

- R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.