3GPP TSG RAN WG4 Meeting #104-bis-e	R4-2215867

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.8.3.1

Source:	vivo

Title:	Further analysis on RRM impacts and general aspects for FR2 multi-Rx chain

Document for:	Discussion

- Introduction

In the RAN4#104-e meeting, there were initial discussions on the RRM impacts and general aspects of FR2 multi-Rx chain DL reception. The following agreements were achieved and captured in the WF [1].

<Agreement >: Issue 1-1-1-2: Number of searchers for cell detection and measurements - 2 searchers are assumed. Issue 1-1-3: Feasibility/necessity of enhancing requirements for L1 measurements - Following L1 measurements related requirements are considered in the multi-Rx chain WI - L1-RSRP measurement - BFD/CBD - RLM - L1-SINR - TCI state switching with dual TCI - Scheduling/measurement restrictions

There were many other issues being identified and discussed. The open issues are also captured in the WF [1]. We provided our views on general RRM requirements in [2] in the last meeting. In this contribution, we further provide our views on RRM impacts and general aspects of FR2 multi-Rx chain DL reception.

- Discussion

- 2.1 General aspects

Since RRM requirements for multi-Rx chain and multi-TRP operation were initially discussed in the last RAN4 meeting, different views and aspects were presented. The main RRM requirements to be discussed in this WI are for L1 measurements, dual TCI state switching and L3 measurements according to scope of the WI. From general aspects perspective, it is independent of specific measurement requirements and aims at scenarios, multi-panel operations etc., or it is common for all the measurements.

Scenarios

It is important to decide firstly what scenarios are considered in the WI. One of the objectives is to specify requirements for reception of 4-layer downlink MIMO with simultaneous reception at the UE from two different directions, which includes RF requirements, demodulation requirements and necessary RRM requirements. Single-DCI can be used to achieve 4-layer DL MIMO by scheduling two SDM PDSCHs from different TRPs with one DCI. Multi-DCI can also be used for 4-layer DL MIMO by scheduling two SDM PDSCHs from different TRPs with two DCIs if UE supports overlapPDSCHsFullyFreqTime-r16. For multi-DCI based scheme, it can be used in either intra-cell multi-TRP deployment and inter-cell multi-TRP deployment. From this perspective, both intra-cell and inter-cell multi-TRP deployment scenarios should be considered.

One of another objectives is to specify necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. Simultaneous DL reception of different QCL TypeD RSs from different directions can be used as support of 4-layer DL MIMO. It can also be used for other purposes, e.g., L1/L2 based mobility, FDM/TDM based multi-TRP operation. Inter-cell multi-TRP could be the use case for the purposes.

Three options for scenarios for Rel-18 multi-Rx chain were proposed and discussed in the last meeting.

Issue 1-1-1-8: Scenarios for Rel-18 multi-Rx DL reception - Proposals - Option 1: support both intra-cell and inter-cell operation with TRPs located within reasonable intercell distance. - Option 2: Working on inter-cell operation with TRPs located within reasonable intercell distance after intra-cell multi-TRP operation work is completed. - Option 3: intra-cell multi-TRP operation only

Based on above analysis, it should be beneficial to support inter-cell multi-TRP operation for multi-Rx chain capable UE. In our view, the work for inter-cell multi-TRP operation and intra-cell multi-TRP operation can be done in parallel.

Proposal 1: Both intra-cell and inter-cell multi-TRP operation are supported for multi-Rx chain UE in the WI.

Receive timing difference

With support of both intra-cell and inter-cell multi-TRP operation, the receive timing difference can be discussed together with deployment scenario and use cases. To support multi-DCI based multi-TRP transmissions and single-DCI based multi-TRP transmissions for 4 layer MIMO in FR2, the receive timing difference between different directions of different QCL Type D RSs should within the CP for SDM scheduling. In addition, for Rel-17 L1/L2 mobility it is also assumed the timing difference of arrival at UE between the SSBs of serving cell and cell with different PCI is less than CP length of the corresponding SCS. Therefore, RTD less than CP should be considered as baseline at least.

In addition, it may not always be possible for NW to guarantee that RTD for configured different QCL Type D RSs is always be within the CP. It would be beneficial to also further discuss if RTD larger than CP can be considered for defining RRM requirements. For example, for FDM PDSCHs scheduling with multi-DCI it is not necessarily assumed that RTD should be within CP. To support L1/L2 mobility with more typical scenarios, RTD between serving cell and neighbor cell can also be larger than CP.

From UE implementation perspective, two FFTs need to be assumed if RTD between two directions of two different Type-D RSs is larger than CP. The UE needs to implement independent timing tracking module. Since receive timing is unknown to UE until it is estimated, it seems UE may have to use independent timing tracking for all the cases, even if RTD is within CP under current operation. There is obvious increase of UE complexity and power consumption.

Proposal 2: Receive time difference for configured different QCL Type D RSs is within CP. FFS use cases that receive time difference for configured different QCL Type D RSs is larger than CP.

TAG assumption for uplink transmission

This WI is focused on multi-Rx chain DL reception and no requirements related to uplink transmission is expected to be specified. Since multi-DCI multi-TRP with two TAs are being discussed in Rel-18 MIMO evo WI, it should not be considered in this WI. Thus, it is reasonable to assume 1 TAG for uplink transmission if it is necessary. It would have no impact to RRM requirements for multi-Rx chain DL reception.

Proposal 3: It is assumed in this WI that uplink transmission is based on 1 TAG.

Antenna panel activation

Whether to define activation delay from a single antenna panel to multi antenna panels was discussed.

Issue 1-1-1-4: Whether to define activation delay from a single antenna panel to multi-antenna panels - Proposals - Option 1: Depending on RF conclusion - Option 2: Study in RRM session

From legacy beam management requirements perspective, it is up to UE implementation how UE satisfy the requirements. UE with multi-Rx chain (multi panels) may switch the panels for beam sweeping. No activation time for panel switching is specified.

For multi-TRP operation, UE may need to keep both panels active for simultaneous PDCCH and/or PDSCH reception from different directions, or simultaneous different Type-D RSs reception. If dual TCI is activated by MAC CE, it may indicate multi-TRP operation implicitly. Then UE may active both panels. If both panels are active, then there is no active delay to be considered. On the other hand, UE may deactivate one panel for power saving when it is not used for a short period. Then the panel need to be activated again when there is simultaneous reception.

If the activation delay means RF chain and antenna panel activation time, then it needs RF discussion to decide the switching time which is also UE implementation dependent. It would take much RF efforts to figure out the switching time together with UE architecture. Since this is not in the scope of the WID, then it is still preferable to leave to UE implementation.

If it means how long the UE is ready for multi-panel PDCCH and/or PDSCH simultaneous reception, then it should be discussed in RRM session. For example, active delay is for UE to complete the dual TCI state switching after MAC CE activates dual TCI state so that UE is ready for simultaneous reception.

Proposal 4: Further clarification is needed on activation delay from a single antenna panel to multi-antenna panels.

Scope

There are two issues related to the scope of the WID or coordination between different WIs.

Issue 1-1-1-6: Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel - Proposals - Option 1: FFS Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction is not the scope of this work item. Issue 1-1-1-9: How to handle FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception - Proposals - Option 1: FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception is handled in this WI, if necessary. - Option 2: FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception is handled in RRM enh3 WI, if necessary.

Regarding the spatial MIMO by using single panel, there was relevant agreements reached in the RF session and captured in [3].

- General deployment assumption for DL split between TRPs to enable 4L reception Agreement: - Proposal: To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE. - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

By using one panel, it is still possible for UE to support 4-layer DL MIMO reception when polarization multiplex is used or polarization multiplex + spatial multiplex as depicted in Figure 1.

Figure 1. Dual-polarized MIMO for single panel

The main point is that for multi-TRP operation, two TCI states should be activated so that different TRPs can use different TCI states for data scheduling. How the two DL signals from different directions is received by UE should be up to UE implementation and not be restricted.

If two independent signals from the same or nearly the same direction is used spatial MIMO, the performance may not be guaranteed. However, it seems implementation issue. For RRM requirements perspective, it is not necessarily to preclude this explicitly.

Observation 1: How Spatial MIMO (either spatial diversity or spatial multiplexing) is used is up to NW and UE implementation. It is not necessary to explicitly preclude spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction from the WI.

Regarding how to handle FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception, it needs coordination between different WIs. In eFeRRM WI, following issue was captured in the WF [4].

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception - FFS whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction.

In our view, whether/how beam sweeping factor is reduced for UE with multi-Rx chain can be discussed in the multi-Rx chain WI. It is helpful to reach consistent conclusion. The conclusion may be made for other requirements than SCell activation. We are open to discuss how it will be handled.

Observation 2: FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception, e.g., beam sweeping factor enhancement, can be handled in this WI.

2.2 UE architecture

From UE architecture perspective, it should be UE implementation specific. Following possible implementations were discussed in the last meeting.

Issue 1-1-7: UE architectures - Proposals - Option 1: The scope of a RX chain architecture includes possible implementations and UE capabilities as below: - Multiple Antenna panel - Multiple Antenna panel + AGC - Multiple Antenna panel + AGC + front-end (time and frequency sync) - Multiple Antenna panel + AGC + front-end + Demod/RRM

For simultaneous DL reception from different directions, independent RF chain would be expected. It is related to other assumptions, e.g., receive timing difference, how timing tracking is performed in baseband. Regarding demodulation and RRM, it should be discussed with requirements, rather than defining general assumption. Therefore, we don’t think it needs to have a general conclusion on multi-Rx chain architecture.

Observation 3: it is not necessary to have a general conclusion on multi-Rx chain architecture.

Beam management

Issue 1-1-8: Beam management - Proposals - P1: RAN4 can consider two options for beam management for UE with multiple RX chains - Option-(i) Reuse independent beam management concept (IBM) from Inter-band CA study - Option-(ii) define panel or RX chain specific behaviors with RX panel control signal for DL - P2: Study independent beam managements for different QCL-D sources with multiple TRPs, and also study other QCL channel property impacts of {average gain, delay shift and doppler shift} for multi-RX chain requirements.

For UE supporting independent beam management (IBM), there is no measurement restriction on measurements of RSs from different directions on the same OFDM symbol. For examples, the measurement restriction requirements for beam failure detection are as follows.

For FR2, if the network configures same or mixed numerology between SSB for BFD measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions on the other band, provided that UE is capable of independent beam management on this FR2 band pair.

In general, such principle can be used to define beam management related requirements for multi-Rx chain UE, e.g., no measurement restrictions for UE to measure different QCL Type-D RSs from different directions on the same OFDM symbol simultaneously.

Observation 4: Principle of defining beam management related requirements for IBM can be extended for multi-Rx chain.

For defining panel or RX chain specific behaviors with RX panel control signal for DL, it has RAN1 impact and needs RAN1 study. So, it is not in the scope of the WI.

Observation 5: It is not in the scope of the WI to define panel or RX chain specific behaviors with RX panel control signal for DL.

Issue 1-1-9: Dependent on RF conclusions - Proposals - Option 1: RRM requirement of simultaneous DL reception from different directions shall be defined based on applicable conditions/architecture to be concluded in UE RF session. - Option 2: RRM requirements discussion and RF discussion are in parallel.

We think both options are valid. RRM requirements should be based on applicable conditions/architecture to be concluded in UE RF session. Meanwhile RRM requirements discussion and RF discussion can be progressed in parallel unless there is clear dependency.

Observation 6: RRM requirement of simultaneous DL reception from different directions shall be defined based on applicable conditions/architecture to be concluded in UE RF session. The RRM and RF discussions can be in parallel.

- 2.2 General considerations on defining requirements

Based on WID, it is to introduce necessary RRM requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL Type-D RSs on a single component carrier. Therefore, simultaneous reception from different directions can only be enabled on a single component carrier. From demodulation point of view, extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined.

Proposal 5: RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier.

Issue 1-1-2-2: Simultaneous L3 measurements and L1 measurements - Proposals - Option 1: RAN4 to revisit the FR2 SMTC use case for ‘L1 and L3’ measurements and ‘serving cell and non-serving cell’ to reduce measurement latency. Rel-18 FR2 multi-RX chain UE may be able to measure RSs simultaneously with different QCL-D sources or with different beam assumptions (i.e. narrow and wide beam assumption) - Option 2: In R18, the enhanced UE in FR2 is still not required to perform L3 measurements and L1 measurements simultaneously - Option 3: The issue is discussed under mobility enhancement WI

Firstly, it doesn’t matter the issue is discussed in this WI or in the mobility enhancement WI. It’s just that duplicated discussions should be avoided. If simultaneous L3 measurements and L1 measurements are based on multi-Rx chain UE which is capable of simultaneous reception from different directions, it should be discussed in this WI. Then, RAN4 should identify the use case for simultaneous L3 measurements and L1 measurements and study the feasibility.

Proposal 6: RAN4 to identify use cases for simultaneous L3 measurements and L1 measurements and study the feasibility.

Regarding RRM requirements are defined depending on UE behaviour for single-DCI and multi-DCI multi-TRP operation or only single-DCI multi-TRP operation is considered, we think that multi-DCI multi-TRP operation should not be precluded from RRM requirements perspective. If there is requirement that cannot be applied for multi-DCI multi-TRP operation, it can be discussed case by case.

Proposal 7: Multi-DCI multi-TRP operation should not be precluded from RRM requirements perspective.

Issue 1-1-2-4: Whether and how to define power saving related requirements - Proposals - Option 1: Power saving can be additional requirements of UE with multiple RX chains - Option 2: Power saving is not part of this study - Option 3: no need to define additional requirements for power saving.

There is no RRM requirements defined by power saving itself. There is RRM relaxation requirements for the purpose of power saving. For both cases, it is no in the scope of the WI. Therefore, no corresponding requirements are specified. Of course, if certain requirements can be specified in a way for the sake of power saving, it can be discussed case by case.

Proposal 8: no power saving specific requirements are considered in the WI.

Proposal 9: Power saving can be one aspect to be considered when specifying RRM requirements.

Issue 1-1-2-7: Accuracy assumption when defining RRM core requirements - Proposals - Option 1: FFS The new improved RRM requirements should aim to reduce the delays during measurements and procedures (e.g., RLM evaluation period, measurement delays, etc.) while maintaining the existing accuracy requirements.

Option 1 should be agreeable. The accuracy requirements should not be compromised when multi-Rx chain is used for simultaneous reception to reduce measurement delay.

Proposal 10: Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements.

Issue 1-1-2-8: Applicability of new requirements to different QCL types - Proposals - Option 1: FFS RAN4 to discuss whether the new requirements will also be applicable when QCL type D is configured together with QCL type A and QCL type C

If QCL type A is configured, UE needs to track and estimate the two different RSs independently. It may not be the case when only QCL type D is configured where single timing tracking may be performed depending on receive timing difference assumption. Therefore, it cannot conclude for now that requirements defined for QCL type-D only are also applicable when QCL type D is configured together with QCL type A/C.

Observation 7: FFS whether requirements defined for QCL type-D only are also applicable when QCL type D is configured together with QCL type A/C.

- 2.3 UE capability

Following UE capability related issues were raised.

Issue 1-1-6-1: UE capability of simultaneous reception of measured RS and data - Proposals - Option 1: Consider simultaneous reception of L1/L3 measured RS and data if UE supports simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. - Option 2: Consider simultaneous reception of L1 measured RS and data if UE supports simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. - Option 3: For scenario 2 (two RX beams used for different cells) with multi-RX chain capability, UE can receive PDCCH/PDSCH/TRS/CSI-RS for CQI (one beam) and on SSB symbols (another beam) to be measured - Option 4: RAN4 to discuss if additional UE capability for indicating supporting of simultaneous reception with different QCL type D RS and PDSCH/PDCCH is needed. Issue 1-1-6-2: Clarification/understanding on existing UE capabilitiy simultaneousRxDataSSB-DiffNumerology - Proposals - Option 1: Clarify whether UE can perform SSB based measurement using multi-antenna panels when UE supports simultaneousReceptionDiffTypeD. - Option 2: The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used for FR2 - Option 3: RAN4 shall not introduce new, but reuse Rel-16 UE capability IE simultaneousReceptionDiffTypeD-r16, to indicate enhanced FR2-1 UEs supporting simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. - Option 4: It is assumed that UE is capable of supporting simultaneous reception from two different directions with two different QCL type D RSs.

Firstly, the existing UE capability simultaneousReceptionDiffTypeD-r16 is defined as follows in TS 38.306.

simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS38.213 [11].

The UE capability only indicates supporting of simultaneous reception with different QCL type D RSs. It means UE can receive signals from different directions. However, Simultaneous reception of different QCL type-D RSs or two PDCCHs/PDSCHs require different baseband processing. Simultaneous reception of different QCL type-D RSs may also targeted for L1/L2 mobility rather than multi-TRP operation for high layer MIMO. It seems RAN4 needs further study on the existing UE capability simultaneousReceptionDiffTypeD and whether new UE capability is needed depending on progress of RRM requirements for multi-Rx chain UE.

Proposal 11: RAN4 to clarify the usage of existing UE capability simultaneousReceptionDiffTypeD and further discuss if additional UE capability is needed depending on progress of RRM requirements.

Issue 1-1-6-3: UE behaviour and capability of handling Rx signal level difference between two channels - Proposals - Option 1: Study UE behaviours and capability of multiple RX chains regarding handling Rx signal level difference between two channels. - Option 2: Discuss in RF session

This issue can be discussed in RF session or demodulation session. From RRM perspective, there would be no impact.

Proposal 12: UE behaviour and capability of handling Rx signal level difference between two channels may be discussed in RF/demodulation session.

- Summary

In this contribution, we provided views on RRM core requirements for NR FR2 multi-Rx chain DL reception WI. Following proposals and observations are present.

Proposal 1: Both intra-cell and inter-cell multi-TRP operation are supported for multi-Rx chain UE in the WI.

Proposal 2: Receive time difference for configured different QCL Type D RSs is within CP. FFS use cases that receive time difference for configured different QCL Type D RSs is larger than CP.

Proposal 3: It is assumed in this WI that uplink transmission is based on 1 TAG.

Proposal 4: Further clarification is needed on activation delay from a single antenna panel to multi-antenna panels.

Proposal 5: RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier.

Proposal 6: RAN4 to identify use cases for simultaneous L3 measurements and L1 measurements and study the feasibility.

Proposal 7: Multi-DCI multi-TRP operation should not be precluded from RRM requirements perspective.

Proposal 8: no power saving specific requirements are considered in the WI.

Proposal 9: Power saving can be one aspect to be considered when specifying RRM requirements.

Proposal 10: Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements.

Proposal 11: RAN4 to clarify the usage of existing UE capability simultaneousReceptionDiffTypeD and further discuss if additional UE capability is needed depending on progress of RRM requirements.

Proposal 12: UE behaviour and capability of handling Rx signal level difference between two channels may be discussed in RF/demodulation session.

Observation 1: How Spatial MIMO (either spatial diversity or spatial multiplexing) is used is up to NW and UE implementation. It is not necessary to explicitly preclude spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction from the WI.

Observation 2: FR2 SCell activation delay reduction by multi-Rx chain simultaneous reception, e.g., beam sweeping factor enhancement, can be handled in this WI.

Observation 3: it is not necessary to have a general conclusion on multi-Rx chain architecture.

Observation 4: Principle of defining beam management related requirements for IBM can be extended for multi-Rx chain.

Observation 5: It is not in the scope of the WI to define panel or RX chain specific behaviors with RX panel control signal for DL.

Observation 6: RRM requirement of simultaneous DL reception from different directions shall be defined based on applicable conditions/architecture to be concluded in UE RF session. The RRM and RF discussions can be in parallel.

Observation 7: FFS whether requirements defined for QCL type-D only are also applicable when QCL type D is configured together with QCL type A/C.

- References

- R4-2214344	WF on FR2 multi-Rx RRM, vivo

- R4-2213054	Discussion on RRM requirements for multi-chain DL reception, vivo

- R4-2214457	WF on multi-Rx chain DL reception, Sony

- R4-2214345	WF on R18 eFeRRM, Apple