# 3GPP TSG-RAN WG4 Meeting # 104bis-e                            R4-2215894

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	On CA band combination of DL CA_n1-n78 with UL PC2 n1

# Source: 	ZTE Corporation, China Unicom

# Agenda item:	5.20.2

# Document for:	Approval

# 1	Introduction

In last meeting, a basket WID on high power UE for FR1 for inter-band NR_CADC_R18_yBDL_xBUL (x=1,2; y=2,3,4,5,6) with power class 2 on single carrier uplink on FDD band was approved in [1], where the following RF requirements should be defined for the requested band combination.

- Specify the band-combination specific RF requirements for cases in the table below, including

  - Maximum output power and Tx power tolerance.

  - Self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

  - Other additional impact on the requirements, if identified.

In this contributions, we give some analysis on the RF requirements for DL CA_n1-n78 with UL PC2 n1

# 2	Discussion

- 1. Maximum output power and Tx power tolerance

Due to only PC2 single band n1 in the UL, the Tx power tolerance for DL CA with single band PC2 band is the same with PC2, i.e. 26+2/-3 for DL CA_n1-n78 with UL PC2 n1.

In the TS38.101-1 specification, the UE maximum output power for Inter-band CA(i.e. sub-clause 6.2A.1.3) is for two bands in the UL, therefore, there is no further modifications on sub-clause 6.2A.1.3.

The configurations for DL CA_n1-n78 with UL PC2 n1 is:

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | n18 n788 CA_n1A-n78A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination

2. ∆TIB, c and ∆RIB, c

The ∆TIB, c and ∆RIB, c values for DL CA_n1-n78 with UL PC2 n1 are the same with the corresponding DL CA_n1-n78 with UL PC3 n1, which are:

ΔTIB,c values

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n78 | 0.3 | 0.8 |

ΔRIB,c values

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n78 | - | 0.5 |

3. REFSEN requirements

For xDL/1UL inter-band NR CA, MSD caused by harmonic issue, Rx harmonic issue or cross band isolation needs to be studied.

For PC3 CA_n1-n78 1UL, there is no harmonic issue and no harmonic mixing issue for the band combination of n1 and n78 in terms of the studies captured in TR38.716-02-00. In addition, there is no MSD defined for cross band isolation. These consensus can also be applied to PC2 CA_n1-n78 1UL.

Therefore, there are no specific REFSENS requirements.

# 3 Conclusion

In this contribution, we give some discussion on the RF requirements for DL CA_n1-n78 with UL PC2 n1.

Observation: No further modifications on sub-clause 6.2A.1.3 in TS38.101-1 specification, the single band maximum output power and Tx power tolerance can be applied.

Proposal 1: Same ∆TIB, c and ∆RIB, c values of PC3 CA_n1-n78 can be applied to DL CA_n1-n78 with UL PC2 n1

Proposal 2: No specific REFSENS requirements (i.e. MSD) defined for DL CA_n1-n78 with UL PC2 n1.

# 4	Reference

[1] RP-222626  New WID: High power UE for FR1 for inter-band NR_CADC_R18_yBDL_xBUL (x=1,2; y=2,3,4,5,6) with power class 2 on single carrier uplink on FDD band, China Unicom