# 3GPP TSG-RAN WG4 Meeting # 104bis-e                            R4-2215895

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	On CA band combination of DL CA_n3-n78 with UL PC2 n3

# Source: 	ZTE Corporation, China Unicom

# Agenda item:	5.20.2

# Document for:	Approval

# 1	Introduction

In last meeting, a basket WID on high power UE for FR1 for inter-band NR_CADC_R18_yBDL_xBUL (x=1,2; y=2,3,4,5,6) with power class 2 on single carrier uplink on FDD band was approved in [1], where the following RF requirements should be defined for the requested band combination.

- Specify the band-combination specific RF requirements for cases in the table below, including

  - Maximum output power and Tx power tolerance.

  - Self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

  - Other additional impact on the requirements, if identified.

In this contributions, we give some analysis on the RF requirements for DL CA_n3-n78 with UL PC2 n3

# 2	Discussion

The configurations for DL CA_n3-n78 with UL PC2 n3 is in Table 1.

Table 1 n3-n78 configuration (UL is PC2 n3)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78A | n38 n788 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

NOTE 8: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination

2. ∆TIB, c and ∆RIB, c

The ∆TIB, c and ∆RIB, c values for DL CA_n3-n78 with UL PC2 n1 are the same with the corresponding  DL CA_n3-n78 with UL PC3 n3, which are defined in table 2 and table 3:

Table 2. ΔTIB,c values

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n3-n78 | 0.6 | 0.8 |

Table 3. ΔRIB,c values

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n3-n78 | 0.2 | 0.5 |

3. REFSEN requirements

For xDL/1UL inter-band NR CA, MSD caused by harmonic issue, Rx harmonic issue or cross band isolation needs to be studied.

For PC3 CA_n3-n78 1UL, there are:

- 2nd harmonic of band n3 will fall into its own n78 Rx band, i.e. 2*fn3_UL is overlapped with 1*fn78

- n78 UL harmonic mixing may fall into its own n3 Rx band, i.e. 1*fn78 is overlapped with 2*fn3_DL

In current 38.101-1 spec, there is no harmonic mixing MSD defined for PC3 CA_n3-n78 inter-band CA but 8.1dB harmonic mixing MSD was defined for PC2 CA_n3-n78 when aggressor UL is PC2 n78. Nevertheless, the harmonic mixing MSD for PC3 n3-n78 inter-band CA should be defined first, where we think it can reused the values of ENDC 3_n78.

Regardless the mistakes of PC3 CA_n3-n78, due to the harmonic mixing is from the aggressor of n78 UL to the victim of band n3 DL, due to the n78 is PC3 in the UL, so there is no need to revisit the harmonic mixing MSD.

For H2 harmonic issue, the same RF architecture as DL CA_n3-n78 with PC3 single band UL was used, where harmonic trap filter (HTF) is adopted for band n3.

Some parameters such as PCB isolation=65dB and antenna isolation = 10dB, and HTF isolation equals to 30dB are adopted.

For the test point of channel bandwidth@n78=10MHz and channel bandwidth@n3 =5MHz, the MSD is calculated in table 4.

Table 4. H2 MSD for n3-n78 (MSD test points: n3= 5MHz @ n78=10MHz)

|  | Main path | Diversity path |
| --- | --- | --- |
| IMD2 (without CF) (dBm) | -64.1 | -67.4 |
| Band n78 Thermal noise@10MHz (dBm) | -92.3 | -92.3 |
| Total Noise (dBm) | -64.1 | -70.4 |
| After MRC sensitivity @ Ant (dBm) | -69.1 |  |
| MSD (dB) | 26.2 |  |

Similarly, for the MSD test points of channel bandwidth@n78=100MHz and channel bandwidth@n3 =10MHz,  the MSD is calculated in table 5.

Table 4. H2 MSD for n3-n78 (MSD test points: n3= 5MHz @ n78=10MHz)

|  | Main path | Diversity path |
| --- | --- | --- |
| IMD2 (without CF) (dBm) | -64.1 | -67.4 |
| Band n78 Thermal noise@100MHz (dBm) | -82.6 | -82.6 |
| Total Noise (dBm) | -64.0 | -67.3 |
| After MRC sensitivity @ Ant (dBm) | -69.0 |  |
| MSD (dB) | 16.6 |  |

# 3 Conclusion

In this contribution, we give some discussion on the RF requirements for DL CA_n3-n78 with UL PC2 n3.

Proposal 1: Same ∆TIB, c and ∆RIB, c values of PC3 CA_n3-n78 can be applied to DL CA_n3-n78 with UL PC2 n3

Proposal 2: No specific REFSENS requirements (i.e. harmonic mixing MSD) defined for DL CA_n3-n78 with UL PC2 n3.

Proposal 3: The harmonic MSD for DL CA_n3-n78 with UL PC2 n3 are defined as:

Table 7.3A.4-1a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 26.2 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

# 4	Reference

[1] RP-222626  New WID: High power UE for FR1 for inter-band NR_CADC_R18_yBDL_xBUL (x=1,2; y=2,3,4,5,6) with power class 2 on single carrier uplink on FDD band, China Unicom