# 3GPP TSG-RAN WG4 Meeting # 104bis-e                               R4-2215899

# Electronic Meeting, Oct. 10-Oct. 19, 2022

# Title: 	RF requirements on CA_n39-n41 supporting simultaneous Rx/Tx

# Source: 	ZTE Corporation

# Agenda item:	5.26.2

# Document for:	Approval

# 1	Introduction

In last meeting, a basket WID for simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18 was approved [1]. It aims to identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values.

Currently, there are following TDD-TDD band combination requested in the basket WID:

1. PC3: CA_n34A-n41A

2. PC3: CA_n39A-n41A

3. PC2&3: CA_n40A-n41A

In this contributions, we give some discussion on the RF requirements on PC3 n39-n41 supporting simultaneous Rx/Tx.

# 2	Discussion

Currently, the requirements for the above three TDD NR CA band combination were derived without supporting non-simultaneous Rx/Tx. In current TS38.101-1, delta Tib,c/Rib,c values for LTE CA_39-41 are reused for CA_n39-n41, respectively. As shown in table 1 and table 2.

Table 1. ΔTIB,c requirements for CA_n39-n41

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | 02 / 0.53 | 02 / 0.53 |
| NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter- band carrier aggregation configurations NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

Table 2. ΔRIB,c requirements for CA_n39-n41

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | 0.22 / 0.23 | 0.22 / 0.23 |
| NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx.. |  |  |

As can be seen from the above two tables, there were two sets of the delta Tib,c/Rib,c values, one for single UL and the other one for dual UL, in which delta Tib,c/Rib,c values of 0dB/0.2dB are for single UL considering there was only diplexer for Rx, while Tib,c/Rib,c values of 0.5dB/0.2dB are for dual UL considering additional diplexer may be used for the Tx chain, while the Rx chain could be reused.

In general, the RF architectures are different for TDD-TDD band combination with or without supporting simultaneous Rx/Tx. Fig 1 gives some examples of the RF architecture for the LTE CA_39-41 band combination with or without supporting simultaneous Rx/Tx [2].

(a) With simultaneous Rx/Tx

(b) Without simultaneous Rx/Tx (Rx share diplexer)

Fig 1. RF architecture of LTE 39-41 with or without supporting simultaneous Rx/Tx

In our understanding, to support simultaneous Rx/Tx for band n39-n41, the RF architecture in Fig1(a) for LTE CA_39-41 can be reused, where diplexer is shared by band 39 and band 41. It should be noted that non-simultaneous Rx/Tx could also be enabled in the implementation of Fig 1(a).

For High-High band combination, the diplexer IL is ~1dB, the delta Tib,c values for band n39 and n41 equal to 0.5dB by using share-pain principle. Seemingly, it is more like LTE UL CA_39-41 where diplexer is used in both Tx and Rx. So it is proposed that the delta Tib,c/Rib,c values for n39-n41 supporting simultaneous Rx/Tx in table 3 and table 4, respectively.

Table 3. ΔTIB,c values

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | [0.5]x | [0.5]x |
| NOTE x: Applicable for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx. |  |  |

Table 4. ΔRIB,c values

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | [0.2]x | [0.2]x |
| NOTE x: Applicable for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx. |  |  |

Next, when simultaneous Rx-Tx is supported for n39-n41, the cross band isolation MSD should be checked. The cross band isolation MSD values are calculated below.

Table 4. Parameters used in the calculation

| Parameters | Values |
| --- | --- |
| Tx PA output power (dB) | 27 |
| n41 Tx PA output power at Bn39 Rx band(dBm/Hz) | -130 |
| n41 Tx Transceiver noise floor at PA output (dBm/Hz) | -150 |
| n39Tx PA output power at Bn41 Rx band(dBm/Hz) | -135 |
| n39Tx Transceiver noise floor at PA output (dBm/Hz) | -145 |
| PA to antenna IL | 4 |
| n39 Rx LO phase noise (dBc/Hz) | -160 |
| n41 Rx LO phase noise (dBc/Hz) | -150 |
| Rx IIP2 (dBm) | 50 |
| LNA to antenna IL (dB) | 3 |
| NF | 9 |
| Cross-band isolation (dB) at Tx (Rx filter) Bn41 Rx rejection @ Bn39 (1880~1920) including diplexer isolation | 40 |
| Cross-band isolation (dB) at Rx (Tx filter) Bn39 Rx rejection @ Bn41 (2496~2690) including diplexer isolation | 55 |

For n41 Tx impact on n39 Rx, the MSD is evaluated as:

Table 5. Cross band isolation MSD for n41->n39

|  | 5MHz | 40MHz |
| --- | --- | --- |
| n39 REFSEN | -100 | -90.6 |
| Total interference noise referred to antenna port (dBm) (Main path) | -98.2 | -90.6 |
| Total interference noise referred to antenna port (dBm) (Div path) | -109.9 | -100.9 |
| MSD (Mian) | 2.5 | 1.8 |
| MSD (Div) | 0.2 | 0.2 |
| MSD(after MRC) | 1.2 | 0.9 |

For n39 Tx impact on n41 Rx, the MSD is evaluated as:

Table 6. Cross band isolation MSD for n39->n41

|  | 10MHz | 100MHz |
| --- | --- | --- |
| n41 REFSEN | -94.8 | -84.7 |
| Total interference noise referred to antenna port (dBm) (Main path) | -73.0 | -72.7 |
| Total interference noise referred to antenna port (dBm) (Div path) | -98.5 | -95.4 |
| MSD (Mian) | 17.9 | 8.6 |
| MSD (Div) | 0.1 | 0.1 |
| MSD(after MRC) | 3.6 | 2.6 |

Therefore, the cross band isolation MSD values for CA_n39-n41A with supporting simultaneous Rx/Tx are proposed in table 7.

Table 7. Cross band isolation MSD for CA_n39-n41A with supporting simultaneous Rx/Tx

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.6 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.6 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 0.9 | >ACLR2 |

# 3 Conclusion

In this contribution, we give some discussion on the RF requirements for CA_n39-n41 supporting simultaneous Rx/Tx. Considering the current RF requirements were derived without simultaneous Rx/Tx, therefore ΔTIB,c /ΔRIB,c and cross band isolation MSD should be re-evaluated for CA_n39-n41 supporting simultaneous Rx/Tx.

Proposal 1. The ΔTIB,c /ΔRIB,c values are proposed:

Table 3. ΔTIB,c values

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | [0.5]x | [0.5]x |
| NOTE x: Applicable for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx. |  |  |

Table 4. ΔRIB,c values

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | [0.2]x | [0.2]x |
| NOTE x: Applicable for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx. |  |  |

Proposal 1. The cross band isolation MSD values are proposed:

Table 7. Cross band isolation MSD for CA_n39-n41A with supporting simultaneous Rx/Tx

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.6 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.6 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 0.9 | >ACLR2 |

# 4	Reference

[1] RP-222648, New WID: Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18, Huawei, HiSilicon

[2] R4-131928, UE requirement structure for CA_B39_B41 way forward, CMCC, Renesas, CATT, Huawei, MediaTek, Qualcomm, ZTE

[3] R4- 135274, Discussion on CA_B39_B41 UE type, CMCC

[4] TR36.860