3GPP TSG-WG RAN4 Meeting #104-bis-e	R4-2215930

# Online, 10th – 19th October, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 				UE REFENS for APT600

Agenda Item: 	5.27.3

Document for:	Approval

# Introduction

A new WI to introduce a 600 MHz band for APT region was approved in RAN#96 [1]. The feasibility study of the band has been already completed as reported in [2]. The band plan to specify is based on option B1 with an assumption that a single FDD duplexer 2x40 MHz for 663-703 MHz uplink and 612-652 MHz. RAN4#104e made lots of agreements [4-11] ready to complete the core part of WI by December 2022

In this contribution an open issue on UE REFSENS is discussed according to the WF agreed [9].

# Discussion

GTW agreement

- For 5MHz, 10MHz, 15MHz, use the requirements of n71 as the starting point, and further discuss whether to add 0.5dB relaxation or to tighten the requirement of n71.

- For larger channel bandwidth, FFS on the improvement of reference sensitivity compared to n71.

Way forward

- APT600 REFSENS UL configurations are the same than for n71

- 5, 10 and 15MHz CBW REFSENS baseline is the following:

| SCS | 5MHz | 10MHz | 15MHz |
| --- | --- | --- | --- |
| 15 | -97.2 | -94.0 | -91.6 |
| 30 |  | -94.3 | -91.9 |

- The following options are discussed for next meeting starting from the GTW agreed baseline

  - Option 1: values are reused as is

  - Option 2: values are relaxed by 0.5dB as follows

| SCS | 5MHz | 10MHz | 15MHz |
| --- | --- | --- | --- |
| 15 | -96.7 | -93.5 | -91.1 |
| 30 |  | -93.8 | -91.4 |

  - Option 3: values are tightened by 0.5dB as follows

| SCS | 5MHz | 10MHz | 15MHz |
| --- | --- | --- | --- |
| 15 | -97.7 | -94.5 | -92.1 |
| 30 |  | -94.8 | -92.4 |

  - Option 4: 0.5dB relaxation is applied to channels overlapping the 612-617MHz range as follows

| SCS | 5MHz | 10MHz | 15MHz |
| --- | --- | --- | --- |
| 15 | -97.2x | -94.0 x | -91.6 x |
| 30 |  | -94.3 x | -91.9 x |
| Note X: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |

- Improvement of REFSENS for channels >15MHz versus n71 is studied for next meeting

WF on UE requirements for APT 600 MHz in RAN4#104-e captured the agreement on REFSENS in the following.

It is already assumed in the WID [1] that UL is restricted to 20 MHz to avoid large MSD, which is aligned with band n71. For -51 MHz duplex in APT600 band n105, the UL frequency is 5 MHz away further than band n71 based on -46 MHz duplex. MSD impact is expected much less than n71 due to the larger separation. Further, we expect band n105 UE typically support band n71 with the same RF components and the insertion loss the same as band n71 should be achieved for the entire range of band n105. Therefore, the REFSENS for n105 is not relaxed from band n71 requirement.

Proposal 1: REFSENS of band n105 with -51 MHz duplex shall not be relaxed from band n71 requirement.

# Conclusion

Considering the co-banding with band n71 and larger duplex for n105 for -51 MHz, it is proposed that REFNSES for n105 shall not be relaxed from band n71.

Proposal 1: REFSENS of band n105 with -51 MHz duplex shall not be relaxed from band n71 requirement.

# Reference

[1] RP-221778 New WID on APT 600 MHz NR band

[2] TR 38.860 Study on extended 600MHz NR band

[3] TR 36.755 US 600 MHz Band for LTE

[4] R4-2214878 Workplan for Introduction of APT 600 MHz band

[5] R4-2214879 Skeleton TR for APT 600MHz NR band

[6] R4-2214880 Text Proposals  for  TR 38.xxx for APT 600MHz NR band

[7] R4-2214433 WF on system parameters for APT 600 MHz, Huawei, HiSilicon

[8] R4-2214434 WF on BS requirements for APT 600 MHz, Nokia, Nokia Shanghai Bell

[9] R4-2214435 WF on UE requirements for APT 600 MHz, Skyworks Solutions Inc

[10] R4-2215090 draft CR to TS38.104 the introduction of APT600MHz, ZTE Corporation

[11] R4-2215102 Introduction of APT 600 MHz band, Qualcomm Incorporated

[12] TR 38.892 v0.0.1

[13] RP-222013 Status report for WI APT 600MHz NR band; rapporteur: Spark NZ Ltd