3GPP TSG-WG RAN4 Meeting #104-bis-e	R4-2215933

# Online, 10th – 19th October, 2022

Source: 				Nokia, Nokia Shanghai Bell

Title: 				Issues on intra-bandENDC-Support

Agenda Item: 	9.3

Document for:	Approval

# Introduction

RAN#97e tasks RAN4 and RAN2 to have more discussion in Q4/2022 to check the inconsistency issue described in RP-222646 [12,13]. At least, two issues should be addressed.

- Whether configurations in Case 3 and Case 4 are valid from RAN4 and RAN2 point of view respectively.

- In the case of configuration in Case 3 and/or in case of configuration in Case 4 are(is) confirmed as valid, whether a solution is necessary in RAN2 to address the ambiguity issue for configurations on some intra-band EN-DC band combinations with more than 2 carriers from Rel-15.

In TS 36.306, a UE capability on intraBandENDC-Support is defined in the following.

intraBandENDC-Support

Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3.

If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination.

This UE capability per BC (band combination) was first introduced to distinguish whether UE supports either DC_(n)41AA and DC_41A_n41A, or both, when the EN-DC band combination of 41 and n41 is signalled based on LS [1] and CR [2,3]. The initial case study was for two carriers only. The above definition is not clear enough what contiguous spectrum or non-contiguous spectrum means for more than 2 carriers, whether if spectrum is about all the carriers or just about two carriers between E-UTRA and NR. This issue has been discussed since RAN4#97e [4-7,9] and RAN#91e [8] without conclusion.

There has been a claim that those Case 3 and Case 4 combinations are not aligned with the fallback principle in TS 36.306 in the following and therefore they are invalid configurations.

Fallback band combination: A Uu band combination that would result from another Uu band combination (parent band combination) by releasing at least one SCell or uplink configuration of SCell, or SCG, or SUL. A PC5 band combination that would result from another PC5 band combination (parent band combination) by releasing at least one sidelink carrier. An intra-band non-contiguous band combination is not considered to be a fallback band combination of an intra-band contiguous band combination. A fallback band combination supports the same channel bandwidth(s) for each carrier as its parent band combination(s).

# Discussion

Ambiguous EN-DC configurations in TS 38.101-3 (Rel-17), which are categorized as Case 3 and Case 4 in RAN4 discussion, are listed below.

(Note that Case 1 and Case 2 are contiguous or non-contiguous only and hence do not need further clarifications.)

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)41AB DC_(n)41CA DC_(n)41DA | DC_41A_n41A |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, it will has two kinds of UL ENDC configurations:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

Use Cases

Channel allocation is dynamic for GAA (General Authorized Access) operation in the CBRS band. GAA can start with a Pcell and can add vacant channels freely as a PScell or Scell. Once a channel is not available anymore, it needs to be dropped. The intra-band combinations would become contiguous and non-contiguous during this dynamic channel allocation process. Thus, those Case 3 and Case 4 configurations for band 48/n48 are necessary due to this nature of GAA operation, even though this may not be aligned with the fallback principle.

Observation 1: Case 3 and Case 4 are needed for the flexibility of CBRS GAA operation.

However, this may not be the case for band 41/n41. Indeed, Case 3 was agreed to be removed from Rel-15 [9,10], however, the CR was not fully implemented and some of them still remain in Rel-15. If there is no use case identified for band 41 and n41, it can be removed also from later releases.

Proposal 1: Consider removing Case 3 for band 41 and n41 if no use case is identified.

In the fallback operation, Scell de-activation should not increase the number of non-contiguous sub-blocks to keep the same number of active RF chains. With this assumption, the capability signalling can be only made for the highest order combination and all fallback combinations can follow the same UE capability. However, there is no strict limitation that Case 3 or Case 4 configuration shall be forbidden, as far as per BC UE capability signalling is well defined.

Observation 2: Fallback is defined to support reduced UE capability signalling for lower order CA/DC and is not restricting Case 3 or Case 4 configuration if there is a demand to support them.

Solutions

In order to sort out the inconsistency issues there are possible solutions in the following.

- Option 1: Remove Case 3 and 4 in RAN4; No change in RAN2.

- Option 2: Introduce more flexible UE capability signalling in RAN2.

- Option 3: Use the existing UE capability to clarify which configurations are supported; No need to introduce a new signalling. Further clarification/modification on the definition of existing UE capability is needed in RAN2.

Option 1 is to remove all ambiguous EN-DC cases, which does not require any change in RAN2. According to our Observation 2, this will restrict the expected CBRS GAA operation.

Option 2 is for RAN2 to provide a sufficiently flexible UE capability signalling. RAN4 would not need any further action until RAN2 completes the spec update. We do not discuss a RAN2 solution in detail in this contribution. Nevertheless, a possible issue is whether such a new signalling can be supported in legacy releases or not.

Option 3 is to use the existing capability signalling but it is redefined to further clarify the ambiguity and inconsistency. This is possibly NBC. These band combos are likely not yet supported in any product. Thus, it would be possible to avoid NBC.

A CR [11] is already agreed to introduce a new table for mixed intra-band contiguous and non-contiguous EN-DC cases, intended for Case 4. Though it is not agreed, the intention is to use intraBandENDC-Support=both for the new table. However, Case 3 is not sorted out yet.

One possible way to keep all Case 3 and Case 4 in the spec is to assign intraBandENDC-Support=both to DL contiguous and UL non-contiguous case, i.e.,

- EN-DC config including a (n) sub-block in both DL and UL is categorized to “contiguous”

- EN-DC config without any (n) sub-block is categorized to “non-contiguous”

- EN-DC config including a (n) sub-block in DL and without it in UL is categorized to “both”

In this way each band combo is defined without any ambiguity. This is summarized in Table 1.

Table 1: EN-DC configuration category (proposed vs current)

| EN-DC configuration | Uplink EN-DC configuration | intraBandENDC-Support (possible new category) | in the latest Rel-17 draft spec (existing category according to Table 5.3B.1.2-1, Table 5.3B.1.3-1 or Table 5.3B.1.3-2) |
| --- | --- | --- | --- |
| DC_(n)48AA | DC_(n)48AA | not present (contiguous) | contiguous |
| DC_(n)48CA | DC_(n)48AA | not present (contiguous) | contiguous |
| DC_(n)48DA | DC_(n)48AA | not present (contiguous) | contiguous |
| DC_(n)48CA | DC_48A_n48A | both | contiguous |
| DC_(n)48DA | DC_48A_n48A | both | contiguous |
| DC_48A_n48A | DC_48A_n48A | non-contiguous | non-contiguous |
| DC_48A-48A_n48A | DC_48A_n48A | non-contiguous | non-contiguous |
| DC_48C_n48A | DC_48A_n48A | non-contiguous | non-contiguous |
| DC_48D_n48A | DC_48A_n48A | non-contiguous | non-contiguous |
| DC_48A_(n)48AA | DC_(n)48AA | not present (contiguous) | mixed contiguous and non-contiguous |
| DC_48A_(n)48AA | DC_48A_n48A | both | mixed contiguous and non-contiguous |

Observation 3: The existing signalling can be reused if EN-DC config including a (n) sub-block in both DL and UL is categorized to “contiguous”, EN-DC config without any (n) sub-block is categorized to “non-contiguous”, “EN-DC config including a (n) sub-block in DL and without it in UL is categorized to “both”

This requires some reorganization of EN-DC combo by moving some combos to appropriate tables.

Proposal 2: It is further discussed if we wait for RAN2 to provide a flexible signalling or indicate RAN2 to implement a CR according to Observation 3.

# Conclusion

In this contribution, the issue about the current definition of intraBandENDC-Support is discussed. It is proposed that

Observation 1: Case 3 and Case 4 are needed for the flexibility of CBRS GAA operation.

Proposal 1: Consider removing Case 3 for band 41 and n41 if no use case is identified.

Observation 2: Fallback is defined to support reduced UE capability signalling for lower order CA/DC and is not restricting Case 3 or Case 4 configuration if there is a demand to support them.

Observation 3: The existing signalling can be reused if EN-DC config including a (n) sub-block in both DL and UL is categorized to “contiguous”, EN-DC config without any (n) sub-block is categorized to “non-contiguous”, “EN-DC config including a (n) sub-block in DL and without it in UL is categorized to “both”

Proposal 2: It is further discussed if we wait for RAN2 to provide a flexible signalling or indicate RAN2 to implement a CR according to Observation 3.

# Reference

[1] R4-1813862 Reply LS on intra-band combination for NR CA and MR-DC, RAN4#88bis

[2] R2-1818899 CR on signaling contiguous and non-contiguous EN-DC capability, RAN2#104

[3] R2-1902625 Miscellaneous corrections, RAN2#105

[4] R4-2015089 Clarification of intra-bandENDC-Support, Nokia, RAN4#97e

[5] R4-2102559 Clarification of intra-bandENDC-Support, Nokia, Nokia Shanghai Bell, RAN4#98e

[6] R4-2102628 On UE capability for intra-band ENDC, Huawei, RAN4#98e

[7] R4-2103130 WF on UE capability on intraBandENDC-Support, Huawei, RAN4#98e

[8] RP-210640 Clarifications on intra-band contiguous and non-contiguous EN-DC combinations, Apple Inc., RAN#91e

[9] R4-2114890 WF on intraBandENDC-support, Nokia, RAN4#100e

[10] R4-2115092 Correction to band combinations for intra-band EN-DC, Ericsson, Nokia, RAN4#100e

[11] R4-2214967 Draft CR for 38.101-3 Rel-16 to correct band combination for intra-band ENDC, Xiaomi, RAN4#104e

[12] RP-222646 Discussion on intra-band EN-DC combination, Huawei, HiSilicon, RAN#97e

[13] RP-222594 Moderator's summary for discussion [97e-34-IntraBand-ENDC-Comb], RAN#97e

[14] R4-2212581 Discussion on intrabandENDC-Support, Xiaomi, RAN4#104e

[15] R4-2212850 Discussion on Intra-Band EN-DC support, Google, RAN4#104e