3GPP TSG-RAN WG4 Meeting #104-bis-e                             R4-2216052

Electronic Meeting, 10 - 19 October, 2022

Source: 	Huawei, HiSilicon

Title:  	Discussion on ATG BS type

Agenda Item:	6.13.4

Document for:	Discussion

Introduction

In the last RAN4 meeting, a WF R4-2214461[1] on ATG BS RF requirements were agreed. One of the open issues is about ATG BS type. A WF was agreed that use BS type 1-H, 1-O as baseline, and FFS for BS type I-C. In this contribution, we give our views on ATG BS type I-C.

Discussion

The work item [2] WID on Air-to-ground (ATG) network for NR was approved in RAN#95e meeting, with the following objectives:

- Specify features to core specifications for ATG BS and UE [RAN4]

  - Scenario:

    - BS on the ground, and the CPE type of UE mounted in the aircraft

    - A direct radio link between BS on the ground and CPE type of UE mounted in the aircraft

    - Note: The deployment characteristics described in the justification section shall be taken as a basis for the technical discussion.

  - Specify core requirements for coexistence between ATG and IMT terrestrial network

    - Example bands include n1, n78 and n79.

    - Perform FR1 co-existence evaluation for ATG network (e.g. ACLR, ACS)

    - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs

      - Aim to reuse existing requirements for BS and UE where possible, e.g.,

        - Reuse TN BS requirements for ATG BS

    - Specify RF requirements for ATG UE/BS

      - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

      - Taking into account identified differences between ATG and fully ground based systems

      - Consider BS type 1-C/1-H/1-O and specify the requirements

      - Consider conductive requirements for UE

    - Specify RRM core requirements for ATG UE

      - Taking into account identified differences between ATG and fully ground based systems

      - Considering the different nature of ATG UEs and their view of the network, increased cell sizes and other relevant aspects

    - Specify new UE/BS type(s) for ATG network if necessary

It is clearly stated in the WID that “Consider BS type 1-C/1-H/1-O and specify the requirements”.

- Observation 1: It is clearly stated in the WID that “Consider BS type 1-C/1-H/1-O and specify the requirements”.

- Observation 2: Based on the current BS specification, the baseline is type 1-C, 1-H and 1-O.

In the last RAN4 meeting, some companies thought that type 1-C cannot have beam capability to serve the service in the air. ATG BS should use the beam rather than fixed antenna. However, we have dispute on this.

- If type 1-C RRUs are used, usually they can provide only wide beams. Therefore, to meet the link budget for remote coverage, antenna gain must be high. A rough link budget in [3] shows 23dBi BS TX antenna gain is enough to get downlink coverage up to 245 km. So it is not impossible for type 1-C BS serve the service in the air.

- Observation 3: Type 1-C can provide only wide beams, however, if it can meet the link budget for remote coverage, it is ok to use type 1-C as ATG BS. The mainstream type of ATG BS is 1-O and 1-H，however type 1-C should not be excluded.

- From implementation point of view, type 1-C has the benefit of cost-effective and test simplification

- Observation 4: From implementation point of view, type 1-C has the benefit of cost-effective and test simplification.

- The example bands include n1 (2100MHz), n78 (3.5GHz) and n79 (5GHz) .For n1, since the frequency is relative low, to implement 1-O, 1-H may have technical challenging, while to implement 1-C is more easy. If type 1-C is excluded, ATG BS operating in n1 is technical challenging.

- Observation 5: If type 1-C is excluded, ATG BS operating in n1 is technical challenging.

Proposal 1: Consider BS type 1-C/1-H/1-O for ATG and specify the requirements as WID described.

- 3 Conclusion

Observation 1: It is clearly stated in the WID that “Consider BS type 1-C/1-H/1-O and specify the requirements”.

Observation 2: Based on the current BS specification, the baseline is type 1-C, 1-H and 1-O.

Observation 3: Type 1-C can provide only wide beams, however, if it can meet the link budget for remote coverage, it is ok to use type 1-C as ATG BS. The mainstream type of ATG BS is 1-O and 1-H，however type 1-C should not be excluded.

Observation 4: From implementation point of view, type 1-C has the benefit of cost-effective and test simplification.

Observation 5: If type 1-C is excluded, ATG BS operating in n1 will be technical challenging.

Proposal 1: Consider BS type 1-C/1-H/1-O for ATG and specify the requirements as WID described.

- 4 References

[1] R4-2214461, WF on ATG BS RF requirements_GTW_v5, RAN4#104-e, August 2022

[2] RP-220962, New WID: Air-to-ground network for NR, CMCC, RAN#95e, March 2022

[3] R4-2211774, General discussion on the RF requirements for ATG BS, Huawei, HiSilicon, RAN4#104-e, August 2022