3GPP TSG-RAN WG4 Meeting #104-bis-e                           	R4-2216054

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP and discussion on triple beat MSD of UL DC_3C_n28A

Agenda Item:	5.1.1

Document for:	Approval

Introduction

In last meeting, the WF [1] on triple beat MSD of UL DC_3C_n28A was approved with MSD test configuration. Companies provided some RF architecture and parameters assumptions in contribution [2]. In this paper we’d like to provide our analysis on triple beat MSD of UL DC_3C_n28A.

Discussion

Test configuration

Referring to clause 7.3.3.2 from TR 38.862 [2], it’s known how to detect a triple beat issue in the FDD band DL or a third band DL.

●   Mainly consider the 1st order triple beat α (TX22TX1) of 3rd order non-linearity -> this is shown in the table as triple beat order 1.

-  Detection -> Duplex Offset<1∙ULCAAggBW+TXMBW2 +RXBW2.

For 20MHz+20MHz band 3C, the LTE nominal channel spacing is 19.8MHz and the UL CA aggregated channel bandwidth is 39.8MHz.

Since UL configuration of band n28 was restricted as 25RB for each DL bandwidth REFSENS requirement, the UL RB allocation for band n28 can be 25RB, but closet to DL Rx band. It’s agreed to assume 25MHz channel bandwidth for band n28 in WF R4-2214426. Thus, the following MSD test configuration and value are proposed as below.

Table 2.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | FFS | FDD | 1st order triple beat α (TX22TX1) i.e. IMD3 |
| DC_3C_n28A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 |  |  |  |

RF architecture and parameters assumption

Referring to the WF [3], it’s proposed to assume the RF architecture of DC_3C_n28A as below. And the attenuation and isolation values are assumed in table 2.2-2.

Figure 1 RF architecture of DC_3C_n28A

Table 2.2-1 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 | Main antenna to diversity antenna |
| Diplexer ISO | 10 |  |
| PCB ISO | 60 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| Band n28 Duplexer Tx rejection at B3 Tx | 30 |  |
| Band n28 Duplexer Tx rejection at n28 Rx | 43 |  |
| Band B3 Duplexer Tx rejection at n28 Tx | 32 |  |
| Band B3 Duplexer Tx rejection at n28 Rx | 32 |  |
| Band n28 filter/Duplexer Rx rejection at n28 Tx | 50 |  |
| Band n28 filter/Duplexer Rx rejection at B3 Tx | 33 |  |

MSD analysis

In this case, the MSD requirements due to 1st order triple beat α (TX22TX1) of 3rd order non-linearity is dominated by band n28 LNA non-linearity performance. Table 2-3 summarize the MSD results for UL DC_3C_n28A.

Table 2-3 MSD calculation for victim Rx of band n28

| Component | 1st order triple beat α (TX22TX1) (dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| Band n28 PA Reverse mixing interference | -126.5 |  |
| Band n28 PA Forward mixing interference | -157.5 |  |
| Band B3 PA Reverse mixing interference | -95.0 |  |
| Band B3 PA Forward mixing interference | -164.5 |  |
| Band n28 LNA IMD interference via filter | -65.0 | -76.0 |
| Total | -65.0 | -76.0 |
| REFSENS specified @30MHz of band n28 | -78.5 |  |
| SNR | -1 |  |
| Noise (Thermal Noise + interference from n28 UL) | -77.5 |  |
| Noise in each path (Thermal Noise + interference from n28 UL) | -74.5 | -74.5 |
| Noise + triple beat IMD interference | -64.5 | -72.2 |
| MRC Rx Noise | -72.9 |  |
| MSD | 8.2dB |  |

- References

[1] R4-2214426, WF on triple beat MSD of UL DC_3C_n28A, Huawei, HiSilicon

[2] R4-2213132, Discussion on triple beat MSD of UL DC_3C_n28A, Huawei, HiSilicon

[3] R4-2210557, WF on criteria for triple beat MSD analysis, Qualcomm Incorporated, Skyworks Solutions, Inc., ZTE Corporation

- Text proposal

- <<Start of TP for TR 37.718-11-11>>

- 6.1.X	DC_3_n28

- 6.1.X.1	Configuration for DC

Table 6.1.X.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_3C_n28A | DC_3C_n28A | No |

As DL_3C_n28A_UL_3A_n28A has been completed since Rel-16, in Rel-18 we aim to introduce configuration DL_3C_n28A_UL_3C_n28A.

- 6.1.X.2	Maximum output power for DC

Table 6.1.X.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3C_n28A | 23 | +2/-3 |

- 6.1.X.3	Spurious emission band UE co-existence for DC

For DC_3_n28, the corresponding spurious emission band UE co-existence for DC have been specified in TS 38.101-3-h60. There is no need to further study it for DL_3C_n28A_UL_3C_n28A.

- 6.1.X.4	MSD analysis for DC

Referring to clause 7.3.3.2 from TR 38.862 [Y], it’s known how to detect a triple beat issue in the FDD band DL or a third band DL.

●   Mainly consider the 1st order triple beat α (TX22TX1) of 3rd order non-linearity -> this is shown in the table as triple beat order 1.

-  Detection -> Duplex Offset<1∙ULCAAggBW+TXMBW2 +RXBW2.

For 20MHz+20MHz band 3C, the LTE nominal channel spacing is 19.8MHz and the UL CA aggregated channel bandwidth is 39.8MHz.

Since UL configuration of band n28 was restricted as 25RB for each DL bandwidth REFSENS requirement, the UL RB allocation for band n28 can be 25RB, but closet to DL Rx band. It’s agreed to assume 25MHz channel bandwidth for band n28 in WF R4-2214426. Thus, the following MSD test configuration and value are proposed as below.

Table 6.1.X.4-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | 8.2 | FDD | 1st order triple beat α (TX22TX1) i.e. IMD3 |
| DC_3C_n28A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 |  |  |  |

- 6.1.X.5	∆TIB and ∆RIB values

For DC_3_n28, the corresponding ∆TIB and ∆RIB values have been specified in TS 38.101-3-h60. There is no need to further study it for DL_3C_n28A_UL_3C_n28A.

- <<End of TP>>