3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216126

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	vivo

Title: 	Discussion on test setup of multi-Rx DL reception

Agenda Item:	6.8.2.2

Document for:	Discussion

- Introduction

In the last meeting, several agreements on multi-Rx were reached [1]:

Agreement:

- Proposal: To support 4L DL MIMO reception at the UE when configured with 2 active TCI states, polarization multiplex (2 layers/direction) + spatial multiplex (2 directions) is assumed at the UE.

  - Note: This proposal is for general deployment assumption, not aimed at UE RF assumption

- Proposal: UE RF requirements for simultaneous reception from different directions shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

- Proposal: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline.

- FFS whether the concept of panel should not be explicitly used in core requirements and test configurations.

- FFS whether the single panel should be excluded.

- Further discuss on the on the candidate AoA pairs for setting the UE RF requirement

  - One Fixed AoA1 (e.g. Peak) + Full set AoA2.

  - Multiple AoA1 + Full set AoA2.

  - Fixed offset between the two AoAs, both probes swept simultaneously.

  - Full set AoA1 + Full set AoA2

  - Other solutions are not precluded. Companies are also encouraged to bring the analysis on how to quantify the Refsens value when receiving multiple signals.

In this contribution, we provide our views on test setup of multi-Rx DL reception.

- Discussion

In the last meeting, several test setups were discussed. Before we start to discuss the details of test setup, we prefer to discuss the range of 2 AoAs first. Currently, the typical mm-wave UE has equipped with multiple panels but only one can be activated at one time and actually UE with single panel is hard to pass current spherical coverage requirement. In [2], we provide a initial evaluation for 2 panels working simultaneously, and the performance may be worse than 2 panel switching due to the mutual impact between panels. In addition, when two AoAs is close and only one panel to receive 2-layer data, 3dB polarization gain will be lost. All the analysis shows when multi-Rx activated simultaneously, the coverage performance is not definitely better than multiple panels switching.

Observation 1: The coverage performance of multiple panels work simultaneously is not definitely to be better than multiple panels switching.

So, we think current spherical coverage should be the baseline for multi-Rx and only need to verify the region that meet the requirement, e.g., top 50% of whole sphere for PC3.

Proposal 1: The selected 2 AoAs only need to locate within the region that meet the single carrier spherical coverage requirement, e.g., top 50% of whole sphere for PC3.

In the following part, we further discuss the potential test setup that raise in the last meeting except the Full set AoA1 + Full set AoA2 case, because this case is the most complicated one and should be the last choice.

- One Fixed AoA1 (e.g. Peak) + Full set AoA2.

one potential way to reduce the test complexity is to fix one of the AoAs. In this contribution, we further evaluate whether it is feasible. Simlar to the above, 3D EM simulation is used and the fixed AoA is the peak direction. The results are shown in below.

Figure 1 Simulation results of fixed peak (AoA1) + fullset (AoA2)

It is obviously that fixing one of the AoA will lead to the results far worsen than the actual UE performance, and about 3.5 dB difference exist at 50%-tile CDF. This is because the performance of the panel, which is fixed to point to the peak direction, is not throughly verified, and this is can not be avoided as long as one of the AoAs is fixed.

Observation 2: Fixing one of the AoAs will cause some panels’ performance not to be verified thoroughly.

- Fixed offset between the two AoAs, both probes swept simultaneously.

Another way is that two probe scan sphere simultaneous with fixed offset between 2 AoAs, and advantage of this method is that, when two probe scan simultanously, both panel still can change its beam to the optimized direction, which means the performance of each panel can be reflect thoroughly.

However, the problem for this method is that even though we fix the offset between two AoAs, there are still infinite choice exist. If we take AoA1 as reference point, the AoA2 still can be placed at any point on the circle with AoA1 as the center and fixed offset as the radius. In addition, we still need a set of AoA pair to accommodate different UE behavior. For example, if the fixed offset is small, UE can only work with single panel and dual panel performance cannot be verified. So, this method is flexible but it seems hard to reduce the complexity.

Observation 3: Fixed offset can not reduce the test complexity of multi-Rx effectively.

- Multiple Fixed AoA1 + Full set AoA2.

This method means we should perform multiple test procedure for each fixed AoA1.For this method, the key issue is what is the principle for selecting the multiple AoA1. Generally, we take two multiple AoA1 as starting point and there are two types of cases: 2 AoA1 within one panel and 2 AoA1 within different panels, as show in Figure 2 and Figure 3

Figure 2 2 fixed AoA1 within one panel

Figure 3 2 fixed AoA1 within different panels

When 2 fixed AoA1 within one panel, the situation is quite similar to only 1 AoA1 fixed. The panel#1 can point to another direction but the performance is still restricted. When 2 fixed AoA1 within different panels, each panel have chance to show its full capability and the performance is closer to the ideal situation. To further verify the two cases, we provide simulation results on the antenna gain of the whole sphere, as show in Figure 4

Figure 4 antenna gain of whole sphere for two cases

We can find that when two fixed AoA1 locate in different panels, the performance is closer to the ideal situation as we expected, and only 0.6 dB difference exist at 50%-tile of CDF. In addition, this method also can reduce the test complexity (reduce the test points from N*N to 2*N) and have little impact on test system.

Observation 4: The performance of 2 fixed AoA1 locates in different panel+Fullset AoA2 can be close to the ideal situation.

However, we cannot identify each panel during test, so it is hard to ensure the 2 AoA1 locate in different panels. There are several possible ways to overcome this problem, e.g., increase the number of AoA1 or the 2AoA should be the farthest apart within the top 50% (for PC3) spherical coverage. Anyway, all these details can be further discussed in future meetings.

Proposal 2: Taking multiple Fixed AoA1+Fullset AoA2 as baseline for further discussion.

- FFS on number of AoA1

- FFS on the principle of multiple AoA1 selection.

- Conclusion

In this contribution, we provide our views on the test setup of multi-Rx:

Observation 1: The coverage performance of multiple panels work simultaneously is not definitely to be better than multiple panels switching.

Proposal 1: The selected 2 AoAs only need to locate within the region that meet the single carrier spherical coverage requirement, e.g., top 50% of whole sphere for PC3.

Observation 2: Fixing one of the AoAs will cause some panels’ performance not to be verified thoroughly.

Observation 3: Fixed offset cannot reduce the test complexity of multi-Rx effectively.

Proposal 2: Taking multiple Fixed AoA1+Fullset AoA2 as baseline for further discussion.

- FFS on number of AoA1

- FFS on the principle of multiple AoA1 selection.

- References

- R4-2214457, WF on FR2 multi-Rx chain DL reception, Sony, RAN4#104e

- R4-2216125, Discussion on UE implementation assumption of multi-Rx DL reception, vivo, RAN4#104e-bis