# 3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216134

# Electronic Meeting, 10th - 19th October, 2022

Source: 	vivo

Title:             Further discussion on self-interference modelling for full duplex from BS aspect

Agenda Item:	6.17.2.2.1

Document for:	Approval

Introduction

In the last meeting, the discussion for SI NR_Duplex_Evo started in RAN4, focusing on interference modelling for self-interference at gNB, co-channel inter-subband and adjacent channel gNB-gNB/UE-UE CLI. Several high-level agreements were achieved for interference modelling [1] and a reply LS providing partial answers on these aspects [2] was sent out to RAN1. In this contribution, we provide our further thoughts on self-interference modelling at gNB side.

Discussion

Self-interference modelling at gNB side

RSI value

For SBFD capable gNB, RSIC capability is quite critical to enable full duplex operation. In the last meeting, the following value range of RSIC [2] was summarized in Table 1 according to the inputs from source companies in RAN4#104-e.

Table 1: value range of RSIC

| Parameter | FR1(Frequency Range 1) | FR2(Frequency Range 2) |
| --- | --- | --- |
| Spatial isolation | 50~80dBc | 80-120 dBc |
| Frequency isolation | 45 dBc | 22.5~30 dBc |
| Beam nulling /isolation | 0~40 dBc | 0~40 dBc |
| Digital IC | 0~50 dBc | 0~50 dBc |
| Overall RSIC capability | 95 ~185 dBc | 102.5~ 205 dBc |

It should be noted that the value range of RSIC is just for information and no analysis on these ranges for FR1 and FR2 was performed in the last meeting. These four isolation schemes can result different RSIC capabilities with different implementation assumption, thus the range of overall RSIC capability are vast with 90 dB difference for FR1 and 102.5 dB difference for FR2. This information for value range of RSIC was sent to RAN1 for simulator development and calibration. However, if different companies pick different RSIC value from the range, (e.g. Company A chooses 95 dB RSIC, Company B choose 185 dB for FR1), different simulation results for self-interference level may be derived. Thus, it is suggested to further refine the value range of RSIC.

In the last meeting, we agreed the criteria on gNB UL receiver sensitivity degradation due to self-interference as follows:

- Taking 1dB sensitivity degradation due to self-interference of DL transmission as starting point for system level evaluation and feasibility study

  - Other values lower than 1dB e.g. 0.1dB/0.8dB not precluded pending on the feasibility study

  - Final values used in co-existence evaluation shall be aligned with feasibility analysis conclusion.

Stacking the self-interference from DL subband to UL subband, the noise level would go up for BS receiving UL subband, thus resulting the REFSENS degradation as shown in Figure 1.

Figure 1. REFSENS degradation due to self-interference

According to 1dB REFSENS degradation criteria, the required value for self-interference cacellation for FR1 can be calculated as Table 2. According to Table 2, it can be observed that for different types of BS the required RSIC values  are different for meeting 1dB REFSENS degradation, due to different output power assumption for different BS types. It is beneficial for RAN1 if the BS type can be provided alongside with corresponding RSIC range. The RSIC range summarized in Table 1 may not always be enough for meeting the 1dB REFSENS degradation. It is suggested to further refine the RSIC range according to Table 2.

Table 2. Required RSIC value for meeting 1dB REFSENS degradation for FR1

| BS Type Power Level | Wide Area BS | Medium Range BS | Local Area BS | Notes |
| --- | --- | --- | --- | --- |
| - UL subband (MHz) | 20 | 20 | 20 |  |
| - Output Power for DL subband(s) (dBm) | 53 | 38 | 24 |  |
| - BS Noise Figure (dB) | 5 | 5 | 5 |  |
| - BS Rx Thermal Noise (dBm) | -101 | -101 | -101 | -174dBm/Hz+10lg(ULBW) |
| - Criteria for REFSENS degradation (dB) | 1 | 1 | 1 |  |
| - Residual Self-interference to Noise (dB) | -6 | -6 | -6 | I-N=-6dB |
| - Noise level (dBm) | -96 | -96 | -96 | c)+d) |
| - Residual Self-interference level (dBm) | -102 | -102 | -102 | g)+f) |
| - Required RSIC (dB) | 155 | 140 | 126 |  |

Observation 1: The required RSIC value for meeting 1dB REFSENS degradation is related to BS type.

Observation 2: The summarized value range for RSIC may not always meet 1dB REFSENS degradation criteria.

Proposal 1: It is suggested to mention the BS type for each corresponding RSIC value range in the reply LS to RAN1.

Proposal 2: To define a threshold for RSIC satisfying 1dB REFSENS degradation and further refine the RSIC range.

RSI scaled to subband/RB level

The definition for RSI, denoted as αSIm,n,  was given by the following equation:

αSIm,n=The total power transmitted on a frequency unit mThe residual self-interference received on a different frequency unit n

According the RSI model, it is the ratio of transmitted power on the DL subband to the residual self-interference on the UL subband. For SBFD DUD configuration (D40M+U20M+DL20M), the RSI denotes the ratio of transmitted power for DL 40M to residual self-interference power level on UL 20M; however, with SBFD DU configuration (D80M+U20M), the RSI denotes the ratio of transmitted power for DL 80M to residual self-interference power level on UL 20M. The RSI value 80M DL is expected to be different from 40M DL since the self-interference situation is changed. Therefore, the RSI value is related to SBFD configuration.

Proposal 3: It is suggested to clarify the SBFD configuration assumption for each RSIC range for gNB.

In the reply LS to RAN1, RSI can be modeled as (almost) frequency flat. Then it means that the power spectral density is flat in aggressor frequency unit m and victim frequency unit n as shown in Figure 3. The power level for Frquency m and n can be scaled to subband/RB since the PSD is flat. Accordingly, the RSI model can be scaled to subband/RB level, which is up to RAN1.

Figure 3. Illustration of RSI model as frequency flat

Observation 1：RSI model is (almost) frequency flat, then it can be scaled to subband/RB level.

Another issue is the scalability for RSI model at gNB side. From our understanding, the RSIC value range provided by RAN4 is for subband level. It needs further clarification on the SBFD configuration alongside with RSIC value. Assume DL subband m with P1 Power with residual self-interference P2 received on UL subband n, the RSI αSIm,n can be expressed in P1/P2; if DL subband m is configured with a RB, UL subband n is configured with b RB, the RSI αSIm,n can be scaled to RB level, that is (P1/a)(P2/b). This is initial understanding for scaling RSI model to RB level. However, the scaling methods needs further confirmation in RAN4.

Proposal 4: It is suggested to clarify how to scale the RSI model to subband or RB level for gNB.

RSI scaled to subband/RB level

In the last meeting, it is clarified by BS vendors that ACLR can be used between subbands. It is also quite aligned that frequency isolation (ACLR) 45dB is assumed among all the companies. However, ACLR definition already implies minimum guard band. In our understanding, if frequency isolation is used for RSIC, then gurard band needs to be considered. However, if no frequency isolation technique is used, then the guradband is not a must. It is suggested that guardbands can be defined as optional for subbands based on the minimum GB requirements as shown in Table 5.3.3-1 and Table 5.3.3-2.

Proposal 5: Consider guardbands between subbands as optional taking minimum guardbands requirement as starting point.

Other

In the last meeting, it is clarified by BS vendors that ACS can be used between subbands, which implies the subband-level selectivity can be assumed for the Rx side at gNB. It is beneficial for mitigating self-interference at gNB Rx side with a degree of subband level selectivity. Thus, it is suggested to discuss whether gNB ACS can be used for receiving UL subband for mitigating self-interference at gNB side.

Proposal 6: To discuss whether gNB ACS can be used for receiving UL subband for mitigating self-interference at gNB side.

Conclusion

This contribution discusses self-interference issues at gNB side and gNB-gNB and UE-UE co-channel CLI issues for full duplex. The following observations and proposals are made:

Observation 1: The required RSIC value for meeting 1dB REFSENS degradation is related to BS type.

Observation 2: The summarized value range for RSIC may not always meet 1dB REFSENS degradation criteria.

Proposal 1: It is suggested to mention the BS type for each corresponding RSIC value range in the reply LS to RAN1.

Proposal 2: To define a threshold for RSIC satisfying 1dB REFSENS degradation and further refine the RSIC range.

Proposal 3: It is suggested to clarify the SBFD configuration assumption for each RSIC range for gNB.

Proposal 4: It is suggested to clarify how to scale the RSI model to subband or RB level for gNB.

Proposal 5: Consider guardbands between subbands as optional taking minimum guardbands requirement as starting point.

Proposal 6: To discuss whether gNB ACS can be used for receiving UL subband for mitigating self-interference at gNB side.

- References

[1] R4-2214377, WF on feasibility study from RF perspective, Samsung, RAN4#104-e.

[2] R4-2214376, Reply LS on interference modelling for duplex evolution, Samsung, RAN4#104-e.