3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216192

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Repeater stimulus signal

Agenda item:	4.1.5.1.1

Document for:	Discussion

Introduction

During RAN4#104-e meeting TPs to Repeater conformance specifications conducted TS 38.115-1 and radiated TS 38.115-2 were agreed in [1] and [2]. However, details of stimulus signals in both specifications remain in brackets. In this contribution we further discuss stimulus signals issue for NR repeaters.

Discussion

Last RAN4 meeting discussion on stimulus signal was captured in email thread 305 [4], where contributions [1], [2], and [3] were submitted for this topic.

As first step there was agreed updates to TS 36.141 in R-090574 [6] where 4 types of repeater stimulus signals were introduced, all based on test model E-TM1.1 – two for downlink test, one for the case where the repeater under test has a pass band narrower than 2.8 MHz, and second one for the case where the repeater under test has a pass band wider than 2.8 MHz, and two for uplink test, with similar split for case with pass band narrower or wider than 2.8 MHz. This is also presented below in table 1 below.

Table 1. E-UTRA repeater stimulus signals based on TS 36.143

| Signal type | Direction of test | Repeater pass band bandwidth |
| --- | --- | --- |
| Repeater stimulus signal 1 | For UL test | For repeater pass band wider than 2.8 MHz |
| Repeater stimulus signal 2 | For DL test | For repeater pass band wider than 2.8 MHz |
| Repeater stimulus signal 3 | For UL test | For repeater pass band narrower than 2.8 MHz |
| Repeater stimulus signal 4 | For DL test | For repeater pass band narrower than 2.8 MHz |

In NR repeaters the narrowest channel bandwidth is 5MHz, thus currently in specifications TS 38.115-1 and TS 38.115-2 only 2 stimulus signals are introduced: one for DL tests and second for UL tests.

For further discussions of stimulus signals for NR repeaters it would be worth to recall, discussion that had taken place when E-UTRA repeaters were designed. As it is described in [5], repeater stimulus signals presented in table 1 above were introduced in TS36.143 in Annex D:

“These signals are intended to be used for unwanted emission testing of repeaters and uplink maximum output power. [...] In [R4-090574], the repeater stimulus signals were designed to achieve Gaussian peak to average power characteristics on signals generated in accordance with test model descriptions or uplink reference channel descriptions. This was also achieved through subsequent refinement of the stimulus signals in the following RAN4 meetings. However, the spectral properties of the stimulus signals were overlooked. As a result, stimulus signals generated as prescribed in the specification may contain strong unwanted emission components. This may result in perfectly good repeaters not passing the unwanted emission test due to unfortunate repeater stimulus signal characteristics. This is particularly noticeable for operating band unwanted emission testing in the repeater pass band, where the filtering of the repeater cannot be applied. (Note that stimulus signals with benign characteristics in this respect may also be created without violating the prescribed signal creation method.)”

Based on above discussion it was stated clearly:

“There is a clear need for specifying the spectral properties of the stimulus signal.”

Further, contribution [5] discuss spectral purity of stimulus signals details, taking into account also unwanted emission testing:

“The large gain variation between repeaters makes it impractical to require an absolute spectral purity in power levels of the stimulus signal that accounts for the highest gain. The maximum output power of repeaters tend to vary substantially less and hence, it is more reasonable to assume a maximum repeater output power, and define the repeater stimulus signal relative to this – with some margin for added repeater effects. Using this approach, the spectral purity is defined in relative spectral density requirements.”

In summary [5] proposed to introduce to stimulus signals spectral purity requirements presented in table 2 below.

Table 2: Repeater stimulus signal spectral purity requirements [5]

| Frequency offset of measurement filter -3dB point, Df | Frequency offset of measurement filter centre frequency, f_offset | Minimum relative spectral density suppression | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz £ Df < 1.4 MHz | 0.05 MHz £ f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz £ Df < 2.8 MHz | 1.45 MHz £ f_offset < 2.85 MHz |  | 100 kHz |
| 2.8 MHz £ Df £ Dfmax | 2.85 MHz £ f_offset < f_offsetmax | 60 dBc (Note1) | 100 kHz |

In further works details of signal spectral purity were updated a bit and are included in Repeater conformance specification TS 36.143.

For NR Repeater stimulus signal, spectral purity requirements should be design similar way, and experience there could be reused.

Proposal 1: For NR Repeater stimulus signal, spectral purity requirements should be design similar way, and experience there could be reused.

Conclusion

In this contribution we further discuss stimulus signals issue for NR repeaters. We provided some background of repeater stimulus signal spectral purity requirements based on work done for E-UTRA Repeater. This can be useful for discussion on NR repeaters stimulus signal spectral purity.

Proposal 1: For NR Repeater stimulus signal, spectral purity requirements should be design similar way, and experience there could be reused.

- References

- R4-2213976	TP to TS 38.115-1 Annex A Repeater stimulus signals, Nokia, Nokia Shanghai Bell

- R4-2213977	TP to TS 38.115-2 Annex A Repeater stimulus signals (OTA),	Nokia, Nokia Shanghai Bell

- R4-2213714	Discussion on NR repeater Stimulus signals,	ZTE Corporation

- R4-2214294	Email discussion summary for [104-e][305] NR_Repeater_RFConformance_Part1

- R4-092950 	Repeater stimulus signals Andrew Wireless Systems, Powerwave Technologies

- R4-090574	TS36.143 Introducing E-TM into operating band unwanted emission measurements, Andrew Wireless Systems; Powerwave Technologies