3GPP TSG-RAN WG4 Meeting #104-bis-e                                                    	R4-2216212

Online, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 37.718-31-11: DC_3-20-32_n7

Agenda item:			5.5.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-31-11 to include DC_3-20-32_n7.

The TIB,c and RIB values provided are derived from the relevant subsets already present in 36.101, 38.101-1 and 38.101-3.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

### 5.1.x	DC_3-20-32_n7

#### 5.1.x.1	Configuration for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-20A-32A_n7A | DC_3A_n7A DC_20A_n7A |

#### 5.1.x.2	∆TIB and ∆RIB values

Table 5.1.x.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-20A-32A_n7A | 3 | 0.7 |
|  | 20 | 0.3 |
|  | n7 | 0.7 |

#### 5.1.1.3	Reference sensitivity exceptions

No additional exceptions required compared to fallbacks.

<End of Text Proposal>