3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216220

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on impact from NCD-SSB time offset

Agenda item:	4.6.3.4   Others   [NR_redcap-Core]

Document for:	Discussion

Introduction

The discussion on RedCap core requirements related to NCD-SSB time offset was continued at RAN4 #104-e related to the reply LS to RAN2 [1]. As part of this discussion, the following issue was treated but not agreed [2].

Issue 3-1-2: NCD-SSB time offset impact - Proposals - Option 1: When the SSB for intra-frequency measurement is fully-partially overlapping with the MG due to SSB offset, UE is required to perform intra-frequency measurement and drop the configured MG. (Ericsson) - Option 2: Up to NW configuration to address this issue (Apple vivo Huawei MTK) - Option 3: FFS (Huawei xiaomi) Agreements: No

In this contribution, we discuss the above open issue from last meeting.

Discussion

The above open issue from last meeting is discussed in this section.

Impact from NCD-SSB time offset

The impact of NCD-SSB time offset on measurement requirements was raised in [3]. The case with NCD-SSB time offset = 5ms is referred, which can be configured by RRC signalling. In this case, if NCD-SSB in the dedicated BWP is used for intra-frequency measurements in the second half frame, it is proposed that gap-assisted CD-SSB measurements in the initial BWP for serving cell or inter-frequency neighbour cells in the first half frame are dropped due to fully partial overlapping. Figure 1 of [3] is replicated below.

Figure 1: CD-SSB and NCD-SSB configured with time offset [3].

At RAN4 #104-e, some companies argued that it is a corner case scenario. As NCD-SSB time offset = 5ms can be configured by RRC signalling, we think it is not a corner case scenario and hence should be considered.

The following proposal is made.

RAN4 to consider the scenario NCD-SSB time offset = 5ms and CD-SSB in a different BWP to NCD-SSB with MG for CD-SSB being applied.

In fact, the proposed drop of CD-SSB measurements according to option 1 in this fully partial overlapping scenario represents a severe limitation for use of this SSB offset configuration. Option 2 proposes to leave this to NW configuration and option 3 proposes to study the issue further.

In our view, it should be first investigated, if there is a principal issue with this SSB offset configuration. It is preferable to keep existing measurement requirements while not introducing limitations such as MG dropping. To this purpose, option 2 is further analyzed.

In fact, SSB bursts are transmitted only in slots #0 to #3 of half frames according to 38.213, hence the measurement spans over 4ms rather than 5ms, as already commented during RAN4 #104-e [1].

Figure 1 shows the overlapping of the configured MG for CD-SSB with NCD-SSB. Assuming MGP #0 or #1 is used with MGL= 6ms, there is overlapping of at least half slot between MG and NCD-SSB (using MGTA=0.5ms). For this MG configuration, NCD-SSB cannot be properly received. On the other side, if MG is further advanced, overlapping can be avoided. To achieve this, network can configure the highest gap offset for MG to enforce that MG for CD-SSB is further advanced by 1ms such that there is no overlapping with NCD-SSB in this case.

The following observation is made.

MG configuration with appropriate combination of gap offset and MGTA, selected by network, can resolve the depicted issue to preclude UE having to drop MG assisted CD-SSB inter-frequency measurements.

The following proposal is made.

RAN4 to not further treat the scenario with NCD-SSB offset = 5ms, as there is no specification impact. Network can select appropriate MG configuration based on gap offset and MGTA to preclude UE having to drop MG assisted CD-SSB inter-frequency measurements.

Conclusion

The open issue from last meeting on measurement requirements impact from NCD-SSB time offset = 5ms has been addressed in this contribution.

The following observation and proposals are made.

- MG configuration with appropriate combination of gap offset and MGTA, selected by network, can resolve the depicted issue to preclude UE having to drop MG assisted CD-SSB inter-frequency measurements.

- RAN4 to consider the scenario NCD-SSB time offset = 5ms and CD-SSB in a different BWP to NCD-SSB with MG for CD-SSB being applied.

RAN4 to not further treat the scenario with NCD-SSB offset = 5ms, as there is no specification impact. Network can select appropriate MG configuration based on gap offset and MGTA to preclude UE having to drop MG assisted CD-SSB inter-frequency measurements.

- References

- R4-2214275, “Email discussion summary for [104-e][224] NR_redcap_RRM_2”, Moderator (vivo)

- R4-2215162, “WF on eDRX and RRM measurement relaxations requirements for Redcap UE”, vivo

- R4-2212755,” Discussions on replay LS about NCD-SSB for RedCap”, Ericsson