3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216221

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title: 	Simulation results for RedCap PDSCH

Agenda item:	4.6.5.1.1

Document for:	Discussion

Introduction

The last eMeeting’s discussion and WF for UE demodulation and CSI requirements for RedCap [1] are captured in the summary [2] and WF [3].

In this contribution, we present a summary of our PDSCH simulation results based on the agreed configurations in [3][4].

PDSCH simulation results summary

The agreed configurations for PDSCH demodulation performance were revised in the last meeting [3] and are depicted in [3][4]. Results are presented both for alignment and for specification of requirements. New results related to 256QAM MCS20 are marked.

PDSCH simulation results for alignment

Simulation results for alignment are reported in this section for agreed simulation assumptions contained in [3][4].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New | 0.8 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New | 9.2 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New | 13.9 |
| 1Rx UE | 10MHz / 15kHz | 256QAM MCS20 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 but uses other MCS | New | 21.9 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse | -2.6 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New | 5.0 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse | 17.6 |
| 2Rx UE | 10MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse | 20.9 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New | 0.8 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New | 9.8 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New | 13.8 |
| 1Rx UE | 20MHz / 30kHz | 256QAM MCS20 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 but uses other MCS | New | 21.8 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New | -2.6 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New | 4.9 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New | 18.2 |
| 2Rx UE | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New | 21.4 |

TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse | -2.6 |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse | 11.3 |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse | 15.3 |

PDSCH simulation results with impairments

Simulation results including impairments are reported in this section for agreed simulation assumptions contained in [3][4].

FDD 15kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | New | 3.3 |
| 1Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New | 11.7 |
| 1Rx UE | 10MHz / 15kHz | 64QAM 0.50 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | New | 16.4 |
| 1Rx UE | 10MHz / 15kHz | 256QAM MCS20 Rank 1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 but uses other MCS | New | 24.4 |
| 2Rx UE | 10MHz / 15kHz | QPSK 1/3 Rank 1 | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-1 | Reuse | -0.1 |
| 2Rx UE | 10MHz / 15kHz | 16QAM 0.48 Rank 1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-4 | New | 7.5 |
| 2Rx UE | 10MHz / 15kHz | 64QAM 0.5 Rank 2 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-4 Test 2-1 | Reuse | 20.1 |
| 2Rx UE | 10MHz / 15kHz | 256QAM 0.82 Rank 1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.1.1-3 Test 1-3 | Reuse | 23.4 |

TDD 30kHz FR1

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 Table | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New | 3.3 |
| 1Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New | 12.3 |
| 1Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New | 16.3 |
| 1Rx UE | 20MHz / 30kHz | 256QAM MCS20 Rank 1 | FR1.30-1 | TDLA30-10 | 2x1 low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 but uses other MCS | New | 24.3 |
| 2Rx UE | 20MHz / 30kHz | QPSK 1/3 Rank 1 | FR1.30-1A | TDLB100-400 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-1 | New | -0.1 |
| 2Rx UE | 20MHz / 30kHz | 16QAM 0.48 Rank 1 | FR1.30-1 | TDLC300-100 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-4 | New | 7.4 |
| 2Rx UE | 20MHz / 30kHz | 64QAM 0.50 Rank 2 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-4 Test 2-1 | New | 20.7 |
| 2Rx UE | 20MHz / 30kHz | 256QAM 0.82 Rank 1 | FR1.30-1 | TDLA30-10 | 2x2 ULA low | 70% of peak rate | 5.2.2.2.1-3 Test 1-3 | New | 23.9 |

TDD 120kHz FR2

| 1Rx UE or 2Rx UE | CBW / SCS | MCS and rank | TDD UL/DL pattern | Propagation condition | Antenna configuration | Metric | Reference from TS38.101-4 | New / Reuse | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2Rx UE | 100MHz / 120kHz | QPSK 1/3 Rank 1 | FR2.120-1A | TDLC60-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-3 Test 1-1 | Reuse | 0.5 |
| 2Rx UE | 100MHz / 120kHz | 16QAM 0.48 Rank 2 | FR2.120-1 | TDLA30-300 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-2 | Reuse | 14.4 |
| 2Rx UE | 100MHz / 120kHz | 64QAM 0.46 Rank 2 | FR2.120-2 | TDLA30-75 | 2x2 ULA low | 70% of peak rate | 7.2.2.2.1-4 Test 2-6 | Reuse | 18.4 |

Conclusion

In this contribution we have provided a summary of our PDSCH simulation results for agreed configurations in [3] [4] both for alignment and for requirements specification.

The following proposal is made.

Include PDSCH results for alignment and for requirements specification into the simulation summary.

- References

- R4-220966, "Revised WID on support of reduced capability NR devices", Ericsson, RAN #95e

- R4-2214317 "Email discussion summary for [104-e][328] NR_RedCap_Demod", Moderator (Ericsson)

- R4-2214394 "WF on RedCap demodulation and CQI reporting requirements", Ericsson

- R4-2210672 "WF on RedCap demodulation and CQI reporting requirements", Ericsson