3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216233

Electronic Meeting, 10 October – 19 October 2022

Agenda Item:	4.6.5.2.1

Source:	Ericsson

Title:	Open issues on CSI reporting requirements for RedCap

Document for:	Discussion

# 1	Introduction

RAN4#104-e agreed with the way forward on RedCap UE demodulation and CQI reporting requirements [1]. This contribution discusses the remaining open issues on the CSI reporting requirements for RedCap UE.

# 2	CSI reporting requirements

## 2.1	CQI reporting delay

Agreement: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD - Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD: - CSI-RS periodicity and offset: 10/1 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: [14ms] - Interested companies are encouraged to evaluate the performance difference between CQI delay 14ms and 10ms in RAN4#104-bis-e. If significant performance degradation is observed compared with CQI delay 10ms, RAN4 will revisit the CQI/RI/PMI delay - If CQI delay 10ms is applied, consider CSI periodicity and offset is set to 10/5.

RAN4 agreed to use the common CSI-RS scheduling and CSI report scheduling for both FD-FDD and HD-FDD (DDDSU), but some companies had concern on the performance, e.g., throughput ratio, because the different CQI delay between FD-FDD and HD-FDD.

Table 1 and Table 2 summarize the simulation results of CQI fading test for 1Rx and 2Rx, respectively, where we compared the performance of different CQI delay options: 14 slots and 10 slots. Figure 1 also shows the throughput ratio, i.e., gamma, with different CQI delay options.

Table 1	Simulation results of CQI fading teste (1Rx).

|  | CQI delay: 14 slots |  |  | CQI delay: 10 slots |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Alpha (>0.20) | BLER (>0.02) | Gamma (>1.05) | Alpha (>0.20) | BLER (>0.02) | Gamma (>1.05) |
| 0 | 0.33 | 0.18 | 1.22 | 0.34 | 0.14 | 1.31 |
| 2 | 0.40 | 0.15 | 1.28 | 0.41 | 0.10 | 1.37 |
| 4 | 0.51 | 0.14 | 1.32 | 0.50 | 0.10 | 1.42 |
| 6 | 0.56 | 0.17 | 1.30 | 0.54 | 0.11 | 1.42 |
| 8 | 0.57 | 0.19 | 1.24 | 0.55 | 0.12 | 1.37 |
| 10 | 0.57 | 0.20 | 1.21 | 0.58 | 0.13 | 1.33 |
| 12 | 0.57 | 0.20 | 1.30 | 0.56 | 0.16 | 1.39 |
| 14 | 0.55 | 0.22 | 1.21 | 0.54 | 0.16 | 1.32 |
| 16 | 0.52 | 0.22 | 1.20 | 0.53 | 0.17 | 1.31 |
| 18 | 0.41 | 0.21 | 1.24 | 0.42 | 0.16 | 1.32 |
| 20 | 0.25 | 0.19 | 1.29 | 0.26 | 0.15 | 1.37 |

Table 2	Simulation results of CQI fading teste (2Rx).

|  | CQI delay: 14 slots |  |  | CQI delay: 10 slots |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Alpha (>0.20) | BLER (>0.02) | Gamma (>1.05) | Alpha (>0.20) | BLER (>0.02) | Gamma (>1.05) |
| 0 | 0.49 | 0.18 | 1.30 | 0.48 | 0.14 | 1.40 |
| 2 | 0.52 | 0.19 | 1.29 | 0.52 | 0.14 | 1.41 |
| 4 | 0.51 | 0.21 | 1.24 | 0.51 | 0.16 | 1.34 |
| 6 | 0.53 | 0.22 | 1.20 | 0.52 | 0.18 | 1.29 |
| 8 | 0.53 | 0.24 | 1.24 | 0.53 | 0.19 | 1.34 |
| 10 | 0.50 | 0.24 | 1.20 | 0.50 | 0.21 | 1.26 |
| 12 | 0.48 | 0.23 | 1.22 | 0.49 | 0.19 | 1.28 |
| 14 | 0.40 | 0.20 | 1.27 | 0.40 | 0.17 | 1.33 |
| 16 | 0.23 | 0.20 | 1.32 | 0.23 | 0.16 | 1.39 |
| 18 | 0.10 | 0.16 | 1.17 | 0.11 | 0.13 | 1.21 |
| 20 | 0.04 | 0.11 | 1.08 | 0.03 | 0.10 | 1.10 |

| (a) 1Rx | (b) 2Rx |
| --- | --- |

Figure 1	Throughput ratios with different CQI reporting delay options.

If we focus on the agreed SNR test points (SNR=9/10/15/16dB for 1Rx UE and SNR=6/7/12/13dB for 2Rx), it is not observed the significant difference of PDSCH BLER and alpha between the options, but it is observed the throughput ratio (gamma) with 14 slots is lower than 14 slots. However since the throughput ratio values are larger than the agreed gamma (1.05) with enough margin, we propose to set CQI/RI/PMI delay to 14ms for FDD CQI reporting test in fading condition.

Observation 1: Throughput ratio with CQI delay 14 slots is higher than the agreed gamma (1.05) with enough margin.

Proposal 1: Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD:

- CSI-RS periodicity and offset: 10/1

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 14ms

## 2.2	Static channel matrix

Way forward: Static channel matrix used for 1Rx UE and SNR test point offset for CQI reporting tests - Interested companies are encouraged to evaluate the options for the static channel matrix and SNR test point offset. - Option 1: Set the static channel matrix in the frequency domain as H1RX=1j. Set SNR test point X=[0]dB lower than 2Rx test case. - Option 2: Keep the previous agreement on the static channel matrix in the frequency domain, that is, H1RX=11. Set SNR test point X=3dB lower than 2Rx test case.

In RAN4#103-e, it has agreed to configure H1RX=11 for CQI reporting tests in static condition for 1Rx UE, however RAN4#104-e discussed another channel matrix H1RX=1j, which is derived from the channel matrix with 2 or more receive antennas, defined in TS38.101-4 B.1.

According to the way forward, we have run the simulation of CQI reporting test in static condition with different channel matrix options and the simulation result is shown in Figure 2.

Figure 2	Simulation results of CQI reporting test in static condition with different channel matrix.

It is observed from the simulation results that the median of the reported CQI indexes are same for 2Rx rank 2 case and 1Rx rank 1 with H1RX=1j, below SNR=16dB. On the other hand, the reported CQI indexes for 1Rx rank 1 with H1RX=11 is large and difference is around 3dB, as discussed in last RAN4 meetings.

Observation 2: 1Rx UE can report the same CQI index as 2Rx UE if static channel is configured as H1RX=1j.

We think both the channel matrix options can meet the purpose of CQI reporting test in static condition, and we have no strong view which channel matrix is used. If RAN4 agree to set H1RX=1j, we propose to set the same test point for both 1Rx UE and 2Rx UE tests, according to the simulation results.

Proposal 2: If RAN4 configures H1RX=1j as the channel matrix for 1Rx UE CQI reporting test in static condition, the SNR test points for 1Rx UE are same as 2Rx UE, that is, SNR=8/9dB and SNR=14/15dB. If RAN4 configures H1RX=11 as the channel matrix for 1Rx UE CQI reporting test in static condition, the SNR test points for 1Rx UE are 3dB lower than 2Rx UE, that is, SNR=5/6dB and SNR=11/12dB.

# 3	Summary

Observation 1: Throughput ratio with CQI delay 14 slots is higher than the agreed gamma (1.05) with enough margin.

Proposal 1: Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD:

- CSI-RS periodicity and offset: 10/1

- CSI-Report periodicity and offset: 10/9

- CQI/RI/PMI delay: 14ms

Observation 2: 1Rx UE can report the same CQI index as 2Rx UE if static channel is configured as H1RX=1j.

Proposal 2: If RAN4 configures H1RX=1j as the channel matrix for 1Rx UE CQI reporting test in static condition, the SNR test points for 1Rx UE are same as 2Rx UE, that is, SNR=8/9dB and SNR=14/15dB. If RAN4 configures H1RX=11 as the channel matrix for 1Rx UE CQI reporting test in static condition, the SNR test points for 1Rx UE are 3dB lower than 2Rx UE, that is, SNR=5/6dB and SNR=11/12dB.

# References

- R4-2214394 “WF on RedCap UE demodulation and CQI reporting requirements”, Ericsson.