3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2216236

Electronic meeting, October 10 – October 19, 2022

Source: 	Huawei, HiSilicon

Title: 	Further discussion on SIB1 signaling and CBW configuration

Agenda Item:	6.1.2

Document for:	Approval

- Introduction

Based on the discussion at previous RAN4 meetings, consensus on SIB1 signaling behaviour and the UE specific channel bandwidth configuration is needed prior to method evaluations can be concluded.

It was discussed in last RAN4#104-bis-e meeting. In this contribution, we provide further study on the remaining open issues.

- Discussion

From RAN2 specification TS 38.331, the carrier bandwidth signalled in SIB1 can be any number of RBs less than 275 RBs. Meanwhile for UE specific channel bandwidth it is clear in the spec that network only configures channel bandwidth that corresponds to the channel bandwidth values defined in TS 38.101, and if these fields are absent, UE will use the carrier bandwidth signalled in the SIB1. The intention to introduce UE specific channel bandwidth is to allow the UE doesn’t support the cell-specific channel bandwidth can set the channel bandwidth in order to operate in a regulatory compliant way. Network should configure the cell-specific channel bandwidth in the SIB1 and UE specific channel bandwidth in connected mode properly to allow UE use the channel bandwidth and meet the regulatory requirements. If some of UEs support cell-specific channel bandwidth and some don’t, then network configure both carrier bandwidth in SIB1 and UE dedicated channel bandwidth that corresponds to 38.101 defined UE CBWs. If none of UE support the cell-specific channel bandwidth, the carrierBandwidth in SIB1 can be set to be different to the 38.101 defined UE CBWs, UE access the cell via initial BWP and re-configured with UE-specific channel bandwidth in connected mode.

In last meeting RAN4#104-e, several agreements were made at the GTW discussion.

Can SIB1 carrier bandwidth take any value below 275?

Agreement:

- RAN4 common understanding are

  - In the current specification

    - SIB1 carrierBandwidth corresponds to BS transmit bandwidth configurations, which is not mandated to be the maximum BS transmission bandwidth configuration specified in TS38.104 and can be any values in number of PRBs.

    - The dedicated channel BWP has to be configured within resource grid (refer to Clause 4.4 and Clause 4 in 38.211).

      - FFS whether the resource grid can be changed by signalling such that it is different from the SIB1 carrierBandwidth

    - Check how UE selects the channel bandwidth for the case when SIB1 carrierBandwidth is not supported by UE.

      - Check it for the initial access and connected mode separately.

  - In Rel-18 or later release, a new UE capability may be needed to indicate that a UE can be configured with a channel BW wider than the carrier Bandwidth in SIB1.

- Can the UE BW be allocated outside the SIB1 carrier bandwidth?

Agreement:

- UE BW can be allocated outside the SIB1 carrier bandwidth from signalling perspective.

- The legacy UE is not required to support it.

FFS on 100 kHz channel raster

- RAN4 common understanding for channel raster for Rel-15/16/17 on the band with 100KHz channel raster specified

  - UE channel bandwidth is aligned with 100KHz channel raster

    - Negative: Intel, Ericsson, China Telecom

  - UE BWP is allowed not to be aligned with 100KHz channel raster

    - BWP is subset of channel bandwidth.

  - SIB 1 channelBandwidth is aligned with 100KHz channel raster for RAN4 requirements for both BS and UE

    - Negative: ZTE, Huawei

The SIB1 carrier bandwidth is given by the higher-layer parameter carrierBandwidth in the SCS-SpecificCarrier IE. It can be can be any values in number of PRBs as defined in the RAN2 specification and agreed as RAN4 common understanding. The resource grid is to determine the location and bandwidth of the carrier. It is from UE perspective and can be changed, e.g. set by a dedicated channel bandwidth in connected mode if UE does not support the SIB1 carrier bandwidth.

–	SCS-SpecificCarrier

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier or the carrier bandwidth. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier information element

-- ASN1START

-- TAG-SCS-SPECIFICCARRIER-START

SCS-SpecificCarrier ::=             SEQUENCE {

offsetToCarrier                     INTEGER (0..2199),

subcarrierSpacing                   SubcarrierSpacing,

carrierBandwidth                    INTEGER (1..maxNrofPhysicalResourceBlocks),

...,

[[

txDirectCurrentLocation         INTEGER (0..4095)                                       OPTIONAL            -- Need S

]]

}

-- TAG-SCS-SPECIFICCARRIER-STOP

-- ASN1STOP

The other issue on SIB1 carrierBandwidth is how UE selects the channel bandwidth for the case when SIB1 carrierBandwidth is not supported by UE. From RAN2 specification TS 38.331, for the initial access, UE can select a supported channel bandwidth which is wider than or equal to the initial BWP and smaller than or equal to the SIB1 bandwidth.

The other open issue is channel raster. In order to co-existence with LTE, FR1 low-frequency bands have a channel raster in steps of 100 KHz, which is 20x the global raster. To support better BWP bandwidth, UE might support 5 KHz steps. While strictly according to specification, the UE channel should be on the 100 kHz raster. For SIB1 carrier bandwidth, as discussed above, it can be any values in number of PRBs and it could be a case that none of UE support the SIB1 channel bandwidth. In such a case there is no need to be aligned with 100kHz raster and can give the freedom to network planning.

In summary, in our understanding,

  - In the current specification

    - SIB1 carrierBandwidth corresponds to BS transmit bandwidth configurations, which is not mandated to be the maximum BS transmission bandwidth configuration specified in TS38.104 and can be any values in number of PRBs.

    - When SIB1 carrierBandwidth is not supported by UE,

      - For the initial access, UE can select a supported channel bandwidth which is wider than or equal to the initial BWP and smaller than or equal to the SIB1 bandwidth.

      - In connected mode, it will be re-configured with UE-specific channel bandwidth

    - 100 kHz channel raster

      - UE channel bandwidth is aligned with 100KHz channel raster

      - UE BWP is allowed not to be aligned with 100KHz channel raster

      - SIB 1 carrierBandwidth is allowed not to be aligned with 100KHz channel raster

  - In Rel-18 or later release, a new UE capability may be needed to indicate that a UE can be configured with a channel BW wider than the carrier Bandwidth in SIB1.

- Conclusion

In this contribution, we provide our view to the remaining open issues on SIB1 signaling and CBW configuration.

Proposal

  - In the current specification

    - SIB1 carrierBandwidth corresponds to BS transmit bandwidth configurations, which is not mandated to be the maximum BS transmission bandwidth configuration specified in TS38.104 and can be any values in number of PRBs.

    - When SIB1 carrierBandwidth is not supported by UE,

      - For the initial access, UE can select a supported channel bandwidth which is wider than or equal to the initial BWP and smaller than or equal to the SIB1 bandwidth.

      - In connected mode, it will be re-configured with UE-specific channel bandwidth

    - 100 kHz channel raster

      - UE channel bandwidth is aligned with 100KHz channel raster

      - UE BWP is allowed not to be aligned with 100KHz channel raster

      - SIB 1 carrierBandwidth is allowed not to be aligned with 100KHz channel raster

  - In Rel-18 or later release, a new UE capability may be needed to indicate that a UE can be configured with a channel BW wider than the carrier Bandwidth in SIB1.