GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216264

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.3.5.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on LBT impacts on RRM requirements for FR2-2

Document for:	Discussion

# Introduction

In last RAN4 meeting, most of the remaining issues are completed for RRM requirements for FR2-2 with agreement captured in [1]. In this paper, we further provide our views on some remaining issues.

# Discussion

In RAN4#103e meeting, a LS [3] is sent to RAN1 and RAN2 to ask whether and how UE can acquire the channel access mode information of neighbour cell. RAN2 has discussed the issue and the LS reply [3] is sent to RAN4 with following information:

1. Overall Description: RAN2 would like to thank RAN4 for their LS on CCA configuration question of FR2-2 Neighbour cells. RAN2 has concluded that it is feasible to indicate whether to apply CCA for neighbour cell measurements in Rel-17 and has made draft CRs (as attached) to show how it can be done. As the RRC_CONNECTED state signaling in the attached CR includes neighbour cell list which indicates used channel access mode (CCA) for each cell, RAN2 would like to request RAN3 to evaluate the feasibility of the following options for obtaining the CCA information of neighbour cells: Option 1: rely on O&M to handle CCA information provisioning for neighbour cells. Option 2: enhance backhaul signaling to exchange cell CCA information between gNBs. 2. Actions: To RAN4, RAN1 group. ACTION: RAN2 respectfully asks RAN4 to take above information into account in their decision on CCA handling for neigbour cells on FR2-2. To RAN3 ACTION: RAN2 respectfully asks RAN3 to consider the above two options and inform RAN2 of their conclusion.

Based RAN2 CR, explicit indication of neighbour cell’s channel access mode is introduced. For IDLE mode neighbour cell measurement, channelAccessMode2-r17 is introduced in SIB3 and SIB4 to indicate the information for intra-frequency and inter-frequency neighbour cell measurement. For CONNECTED mode neighbour cell measurement, channelAccessMode2-r17 is added in MeasObjectNR.

Observation 1: RAN2 introduce explicit indications of neighbour cell’s channel access mode for IDLE mode and CONNCECTED mode neighbour cell measurement.

Thus, it is straightforward that UE shall follow NW’s indication accordingly when performing neighbour cell measurement. It is suggested to add following clarifications in the spec. Take CONNCECTED mode intra-frequency measurement requirements as example, the proposed text are as follows:

### 9.2A.1 Introduction The requirements in clause 9.2.A apply for intra-frequency measurements on carrier frequency with CCA. When the intra-frequency measurement is performed on a neighbour cell in FR2-2 with shared spectrum channel access, UE shall determine the CCA mode of the neighbour cell according to channelAccessMode2-r17of the cell configured in MeasObjectNR. If channelAccessMode2-r17 of the cell is enabled, UE shall assume that CCA applies to the cell and perform measurement accordingly, and the requirements in clause 9.2A shall apply; otherwise, UE shall assume that CCA does not apply to the cell and perform measurement accordingly, and requirements in 9.2 shall apply.

We have a companied CR to reflect the changes in [4].

Proposal 1: UE determines the CCA mode of a neighbour cell according to the indication of channelAccessMode2-r17, and requirements for CCA apply when channelAccessMode2-r17 is enabled.

For scheduling restriction in RSSI measurement, the following scheduling restrictions were introduced for RSSI measurement in FR2-2. For 480 kHz and 960 kHz, considering the impact of beam sweeping, the scheduling restriction is introduced on the first and the last symbols in RMTC.

#### 9.2A.7.4 Scheduling restriction during RSSI measurements in FR2-2 When the UE performs intra-frequency RSSI measurements in unlicensed spectrum, the following restrictions apply due to RSSI measurements: - The UE is not expected to transmit PUCCH/PUSCH/SRS on UL symbols which are overlapping in time with the RSSI measurement symbols configured by RMTC. - The UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on symbols which are overlapping in time with RSSI measurement symbols configured by RMTC if the RSSI measurement resources are not QCL-ed with typeD to the DL RS in the active TCI state of PDCCH/PDSCH. - For 480 kHz and 960 kHz, The UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on symbols which are overlapping in time with the first and last RSSI measurement symbols configured by RMTC. When intra-band carrier aggregation in unlicensed spectrum is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with the aforementioned restricted symbols.

However, according to the definition of RSSI/CO measurement, the RSSI measurement is performed on per OFDM symbols basis, and the CO is calculated by counting the number of symbols on which the RSSI is above channelOccupancyThreshold.

Observation 2: RSSI/CO measurement is performed on per OFDM symbols basis.

Thus, for RSSI/CO measurement with SCS of 480 and 960 kHz, the beam sweeping may occupy partial of the OFDM symbols when the sweeping time is larger than CP. It means the measured RSSI can be underestimated for the symbol, and the CO may be also underestimated too. Thus, it is proposed that for RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC.

Proposal 2: For RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC.

About RSSI measurement, the QCL assumption was discussed in last RAN4 meeting with following agreements:

Issue 1-2-1: Intra-frequency QCL-ed assumption Agreement in GTW: - For performing intra-frequency RSSI measurement in FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier in FR2-2. Issue 1-2-2: Inter-frequency QCL-ed assumption Agreement in GTW: - The QCL reference serving cell is determined by serving cell ID indicated in TCI-StateInfo in RMTC-Config, when the TCI state is provided in the RMTC configuration. - For performing inter-frequency RSSI measurement in FR2-2, - if the TCI state is provided in RMTC configuration of FR2-2, UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. - FFS on the following bullet - if no TCI state is provided in the RMTC configuration [or if TCI state is only provided in the RMTC configuration on a non FR2-2 carrier] UE shall assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of [a or the] carrier in FR2-2. on which the RMTC configuration is provided - RAN4 to clarify the case with RAN1 when there is no serving cell in FR2-2. - If a UE has no serving cell in FR2-2, it is not clear if that explicit TCI state should be configured to the UE for FR2-2 RSSI measurement. If explicit TCI state should be configured, how does the UE use such explicit TCI?

There is FFS point for the case when no TCI state is provided. In the CR stage, companies were aligned about the wording of [a or the] carrier in FR2-2. The unsettled part is whether to consider the case when the TCI state is provided on a non FR2-2 carrier, which is related to the LS to RAN1. If there is at least one FR2-2 serving cell, we think it is a rare case that the TCI state is configured associated with a non FR2-2 carrier. For the case when there is no FR2-2 serving cell, RAN4 shall wait for RAN1 feedback on what the beam assumption.

Observation 3: The specification about QCL assumptions when there is no FR2-2 serving cell can be updated with RAN1’s feedback and clarifications.

# Conclusions

Observation 1: RAN2 introduce explicit indications of neighbour cell’s channel access mode for IDLE mode and CONNCECTED mode neighbour cell measurement.

Proposal 1: UE determines the CCA mode of a neighbour cell according to the indication of channelAccessMode2-r17, and requirements for CCA apply when channelAccessMode2-r17 is enabled.

Observation 2: RSSI/CO measurement is performed on per OFDM symbols basis.

Proposal 2: For RSSI/CO measurement with SCS of 480 kHz and 960 kHz, the RSSI/CO measurement results shall be derived based on symbols configured by RMTC except for the first symbol and the last symbol in RMTC.

Observation 3: The specification about QCL assumptions when there is no FR2-2 serving cell can be updated with RAN1’s feedback and clarifications.

# References

[1] R4-2215159 WF on NR extension to 71 GHz RRM requirements (Part 1), Huawei, HiSilicon

[2] R4-2211171 LS on CCA configurations of neighbour cells

[3] R2-2209235 Reply LS on CCA configurations of neighbour cells