GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216275

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.9.3

Source: 	Huawei, HiSilicon

Title: 	Discussion RRM requirements for FR1-FR1 NR-DC

Document for:	Discussion

# Introduction

RRM requirements for FR1+FR1 NR-DC are missing in TS 38.133. The objective of defining corresponding requirements is included in Rel-18 WI [1] and further revised [2]. In RAN4#104e, the scope of the objective and RRM impacts are initially discussed with agreements captured in [3]. In this paper, we further provide our views of RRM impact of the objective.

# Discussion

In last RAN4 meeting, the scope was initially discussed with following way forward:

Issue 3-1-1: How to reflect the applicability of the existing requirement for FR1+FR1 NR-DC Agreement: - Rel-18 RRM requirements need to remove the clarification or limitation of only supporting FR1-FR2 NR-DC - FFS - Proposal 1: Apart from the requirements in R16/R17 mentioned in the WID, RAN4 to discuss the applicability rules of other R16/R17 requirements for FR1+FR1 NR-DC with/without changing on existing requirements. - Proposal 2: - RAN4 to firstly agree on the list of features that shall be included in the table1. - RAN4 to agree on the baseline requirement clause and formulation based on the agreed list of features that included in the table1. - RAN4 shall have separate applicability discussion of the Scheduling availability, CSSF impacted based on the agreed list of features that included in the table1. Issue 3-1-2: Other features with FR1+FR1 NR-DC Agreement: - The impact of features such as NR-U, MG-enhancement and Redcap are not expected unless the WI scope was extended.

Regarding how to reflect the applicability of existing requirements for FR1+FR1 NR-DC, most companies are aligned to focus on the listed requirements in the WID. However, the requirements can be categorized in different aspects. E.g. Requirements listed/not listed in the WID, requirement with/without applicability rules mentioned about FR1+FR2 NR-DC. Based on the discussion in last meeting, some suggestions are worthy considering which can simplify the discussion and minimize the impact to the spec which is proposed as follows:

For requirements explicitly listed in the WID:

If applicability mentioned FR1+FR2 NR-DC in existing spec, add FR1+FR1 NR-DC applicability;

If there is no applicability rules mentioned FR1+FR2 NR-DC, no need to add/modify the applicability;

For requirements not listed in the WID:

Skip the requirements without spec changes unless impact is identified.

Proposal 1:

For requirements explicitly listed in the WID:

If applicability mentioned FR1+FR2 NR-DC in existing spec, add FR1+FR1 NR-DC applicability;

If there is no applicability rules mentioned FR1+FR2 NR-DC, no need to add/modify the applicability;

For requirements not listed in the WID:

Skip the requirements without spec changes unless impact is identified.

Number of serving carriers

Regarding the number of serving carriers, it is straightforward that current requirements shall be updated. The status are summarized as follows:

Issue 3-2-1: Number of serving carriers Agreement: - RRM requirements for supported number of serving carriers for NR-DC need be updated for FR1-FR1 NR-DC according to RF specification. - FFS on exact number of serving carriers for NR-DC. - Option 1: For FR1+FR1 NR-DC, the number of serving carriers in RRM requirement is up to 6 NR DL CCs in total, with 1 UL in PCell, 1 UL in PSCell and 1 UL in SCell. - Option 2: Exact number of serving carriers will be updated until the late phase of Rel-18 when RF spec is in more stable.

From our understanding, there is not much RRM specific issues about the number of serving carriers which is completely decided by the BC introduced in RF spec. Thus, we support to decide the exact number of serving carrier until the late phase of Rel-18 based on RF spec.

Proposal 2: Decide the exact number of serving carrier until the late phase of Rel-18 based on RF spec.

PSCell addition/release delay and conditional PSCell addition delay

Regarding the requirements for PSCell addition/release and conditional PSCell addition, the status are summarized as follows:

Issue 3-2-2: PSCell addition delay requirement for FR1+FR1 NR-DC Agreement: - For NR PSCell in FR1: Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms. - For NR PSCell in FR1: Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 3* Trs ms. Issue 3-2-3: PSCell release delay requirement for FR1+FR1 NR-DC Agreement: - For FR1+FR1 NR-DC case, existing PSCell release delay requirement can be reused (section 8.9.3 in TS38.133). - Whether or not the interruption requirement stated in section 8.9.3 of TS38.133 can be reused for FR1+FR1 NR-DC case is up to issue 3-2-4. Issue 3-2-4: Interruption requirement for PSCell addition/release for FR1+FR1 NR-DC - FFS - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC): For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused. - Option 2(Nokia, Ericsson, vivo): NR-DC interruption requirements need to be updated to support FR1-FR1 NR-DC. - Option 2a(vivo): When defining the interruption requirements, for the number of SCells for FR1-FR1 NR-DC, up to 3 SCell(s) in FR1 of PCell and up to 3 SCell(s) in FR1 of PSCell are configured, deconfigured, activated or deactivated. Issue 3-2-5: conditional PSCell addition delay requirement for FR1+FR1 NR-DC - FFS - Option 1(Apple, Xiaomi, vivo, Intel, OPPO, MTK, QC, Nokia, HW, vivo): For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133) - Option 2 (Ericsson): existing Conditional Pscell addition requirement (section 8.9A.2 TS38.133) can be used as a baseline for starting the discussion - Option 2a: study the potential enhancement for TUE_preparation

One of the remaining issue is about the interruption requirements. The two options are not mutually exclusive actually, where option 1 is for the length of interruption while option 2 is saying about he the number of SCells. From our understanding, the number of SCells can be triggered in similar approach as the number of serving carrier which is to updated accordingly in later phase in Rel-18.

Proposal 3: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused, and the number of SCells can be updated in later phase in Rel-18.

For conditional PSCell addition, there is one remaining point on whether to enhance the TUE_preparation which can be up to 10 ms in current spec. From our understanding, it is the UE implementation margin which is also considered in CHO, and it is not further differentiated for FR1 to FR1 or FR1 to FR2. Thus, it is proposed that existing requirements can apply.

Proposal 4: For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133)

Scheduling and measurement availability

Regarding the scheduling and measurement availability, the status are summarized as follows:

Scheduling availability Issue 3-2-8: Scheduling availability requirement for RLM/BFD/CBD/L1-RSRP in FR1+FR1 NR-DC Agreement: - Scheduling restrictions for UE performing RLM/BFD/CBD/L1-RSRP in case of FR1-FR1 NR-DC needs to be clarified/updated in the specification. - FFS on option 1a and 2 and 3. - Option 1a(Apple, OPPO, Intel, Xiaomi, QC, vivo): For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario. - Option 2(MediaTek, QC): Add inter-band NR-DC scenario in scheduling availability for both FR1+FR1 and FR1+FR2. - Option 3(Nokia): Other solutions are not excluded. Issue 3-2-9: Scheduling availability requirement for L3 measurement in FR1+FR1 NR-DC FFS: - Option 1(Apple, OPPO, Intel, Xiaomi, MTK, HW): - For intra-frequency measurement without MG, - the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario. - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario. - For inter-frequency measurement without MG, - No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA. - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario. Measurement restriction Issue 3-2-10: Measurement restriction in FR1+FR1 NR-DC FFS: - Option 1(MediaTek): Add inter-band NR-DC scenario in measurement restriction for both FR1+FR1 and FR1+FR2.

For scheduling availability requirement for RLM/BFD/CBD/L1-RSRP, after checking the existing reuquirements, there is no scheduling requirements for FR1 inter-band CA. We don’t fully understand the motivation to add scheduling availability for inter-band CA FR1+FR1 and FR1+FR2. If the target scenarios is for inter-band CA in FR2 when UE is not capable of IBM, the scheduling availability may be necessary. But it seems not in the scope of the WI. For FR1+FR1 NR-DC, if the concerns is for UE not support simultaneousRxTxInterBandCA, the corresponding applicability is already defined in clause 3.6.9. Thus, we think, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario.

### 3.6.9 Applicability of requirements for scheduling availability The scheduling availability requirements in clause 8.1.7.3, 8.5.7.3, 8.5.8.3, 9.5.6.3 and 9.10.2.6.2 assumes that: - The UE is not configured with simultaneous UL/DL between two FR2 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA, and - The UE is not configured with mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception with two different numerologies between FR2 CCs in DL. The scheduling availability requirements in clause 8.1.7.1, 8.1.7.2, 8.5.7.1, 8.5.7.2, 8.5.8.1, 8.5.8.2, 9.5.6.1, 9.5.6.2, 9.8.6.1, and 9.8.6.2 assumes that the UE is not configured with simultaneous UL/DL between two FR1 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA. The scheduling availability requirements in clause 8.1.7.4, 8.5.7.4, 8.5.8.4, 9.5.6.4 and 9.8.6.4 assumes that the UE is not configured with simultaneous UL/DL between FR1 and FR2 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA on this band combination.

Proposal 5: For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario.

For scheduling restriction for L3 measurement, the candidate option 1 is reasonable.

Proposal 6:

For intra-frequency measurement without MG,

    - the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

    - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

For inter-frequency measurement without MG,

    - No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

    - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

HO with PSCell

For HO with PSCell for FR1+FR1 NR-DC, the status are summarized as follows:

Issue 3-2-13: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR1 NR-DC) FFS: - Option 1(Apple, Xiaomi, OPPO, vivo, Intel, MTK, QC, HW, vivo): the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused, i.e., for HO with PSCell requirement, Tprocessing= 30 ms for sequential case and Tprocessing= 25 ms for parallel case should be added for FR1-FR1 NR-DC. - Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC Issue 3-2-14: HO with PSCell for FR1+FR1 NR-DC (FR1-FR2 NR-DC to FR1-FR1 NR-DC) FFS: - Option 1(Apple, OPPO, Intel, Xiaomi, QC), HW: the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused - Option 2 (vivo, MTK, QC): Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms. - Option 3(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC Issue 3-2-15: HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR2 NR-DC) FFS: - Option 1(Apple, OPPO, vivo, Intel, Xiaomi, MTK, QC, HW): - Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms. - Option 2(Nokia, Ericsson): HO with PSCell from NR-DC to NR-DC need to be updated to support FR1-FR1 NR-DC

The only unclear part is the length of Tprocessing when considering FR1+FR1 NR-DC. According to the similar logic in Rel-17, where Tprocessing is max (Tprocessing_Pcell, Tprocessing_PSCell) + 5/10 ms. Tprocessing shall be defined accordingly as follows:

Proposal 7:

  - For HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR1 NR-DC), the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused, i.e., for HO with PSCell requirement, Tprocessing= 30 ms for sequential case and Tprocessing= 25 ms for parallel case should be added for FR1-FR1 NR-DC.

  - For HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1), Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

SCG activation/deactivation

For SCG activation/deactivation, the status are summarized as follows:

Issue 3-2-16: RACH based PSCell activation for FR1+FR1 NR-DC Agreements: - SCG activation/deactivation requirements need to be updated to support FR1-FR1 NR-DC - FFS on option 1a and 1b - Option 1a(Apple, Xiaomi, OPPO, MediaTek, vivo, Intel, Ericsson, HW): if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms. - Option 1b(Ericsson): further study the potential enhancement in T∆ Issue 3-2-17: Known condition for PSCell activation for FR1+FR1 NR-DC FFS: - Option 1(Apple, Intel, Xiaomi, OPPO, MTK, QC, HW, vivo): In FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

For the above two issues, it is proposed to follow the same principle as legacy requirements.

Proposal 8:

For RACH based PSCell activation for FR1+FR1 NR-DC:

if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Proposal 9:

In FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

# Conclusions

Proposal 1:

For requirements explicitly listed in the WID:

If applicability mentioned FR1+FR2 NR-DC in existing spec, add FR1+FR1 NR-DC applicability;

If there is no applicability rules mentioned FR1+FR2 NR-DC, no need to add/modify the applicability;

For requirements not listed in the WID:

Skip the requirements without spec changes unless impact is identified.

Proposal 2: Decide the exact number of serving carrier until the late phase of Rel-18 based on RF spec.

Proposal 3: For FR1+FR1 NR-DC case, existing interruption length requirement for NR PSCell addition/release can be reused, and the number of SCells can be updated in later phase in Rel-18.

Proposal 4: For FR1+FR1 NR-DC case, existing conditional PSCell addition requirement can be reused. (section 8.9A TS38.133)

Proposal 5: For RLM/BFD/CBD/L1-RSRP, the existing scheduling availability requirement for FR1 inter-band CA is used for FR1-FR1 NR-DC scenario.

Proposal 6:

For intra-frequency measurement without MG,

    - the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

    - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

For inter-frequency measurement without MG,

    - No need to introduce scheduling availability requirement of UE performing measurements in TDD bands for FR1-FR1 NR-DC scenario unless such requirement is introduced for FR1 inter-band CA.

    - No need to introduce scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario.

Proposal 7:

  - For HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR1 NR-DC), the existing requirement of HO with PSCell in section 6.1.5.4 TS38.133 can be reused, i.e., for HO with PSCell requirement, Tprocessing= 30 ms for sequential case and Tprocessing= 25 ms for parallel case should be added for FR1-FR1 NR-DC.

  - For HO with PSCell for FR1+FR1 NR-DC (FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1), Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, otherwise, Tprocessing = 45 ms.

Proposal 8:

For RACH based PSCell activation for FR1+FR1 NR-DC:

if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms, and if the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Proposal 9:

In FR1, the PSCell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC

[3] R4-2214345 WF on R18 eFeRRM