GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216277

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.8.3.4

Source: 	Huawei, HiSilicon

Title: 	Discussion RRM requirements of TCI state switching for multi-Rx

Document for:	Discussion

# Introduction

In last RAN4 meeting, the RRM impacts of NR FR2 multi-Rx chain DL reception were initially discussed, in terms of L1/L3 measurement enhancement, dual TCI switching with agreement captured in [1]. In this paper, we further provide our views on the scope and RRM impacts of dual TCI switching.

# Discussion

The RRM impact of dual TCI switching was initially discussed in last meeting. Regarding whether Rel-17 Unified TCI framework should be considered or not, companies had different views. The status are summarized as follows:

Issue 1-3-1-2: TCI framework for dual TCIs - Proposals - Option 1: Dual TCI state switching delay requirements shall base on Rel-15/16 TCI framework. - Option 2: To define requirements for TCI switching delay with dual TCI, both R15/R16 TCI framework and R17 TCI framework, i.e., unified TCI, are considered for UE supporting multi-Rx chain simultaneous reception.

As commented in last meeting, Rel-17 Unified TCI was not jointly considered with dual TCI in Rel-17 FeMIMO, which is listed as an objective in Rel-18 MIMO_evo, which is shown as follows. Considering Rel-17 Unified TCI framework, it is possible that dual TCI switch of PDCCH can also trigger TCI switch of PDSCH or vice versa. However, RAN1 had not touched such discussion in Rel-17 though unified TCI is a Rel-17 feature.

The detailed objectives are as follows: RAN1: - Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

Observation 1: Rel-17 unified TCI is not jointly considered with dual TCI, which is listed as an objective in Rel-18 MIMO.

Thus, though companies argued that they thought unified TCI shall also be considered, there lacks of RAN1 specification supporting currently. It is reasonable to define dual TCI state switching delay requirements base on Rel-15/16 TCI framework.

Proposal 1: Define dual TCI state switching delay requirements base on Rel-15/16 TCI framework.

Regarding the scenarios to be considered for dual TCI switching, following cases were proposed in last meeting.

Issue 1-1-2-3: Support of single-DCI and/or multi-DCI multi-TRP operation - Proposals - Option 1: Define RRM requirements depending on UE behaviour for single-DCI and multi-DCI multi-TRP operation - Option 2: Define RRM requirements for single-DCI multi-TRP operation - Option 3: Define dual TCI state switching requirements for following cases: - PDCCH non-SFN: Two MAC CE with one for each TCI state - PDCCH SFN: single MAC CE for two TCI states - PDSCH single DCI: single DCI for two TCI states - PDSCH multiple DCI: Two DCI with one for each TCI state

During discussion in lasting meeting, companies stated that the interested scenarios is to support 4 layer MIMO in single DCI, thus only single DCI should be considered. However, we don’t fully agree with the statement. The RRM objective is to have requirements for simultaneous DL reception from different directions with different QCL type D, and mDCI is also an important scenario to be considered in Rel-16 MIMO. Thus, all the proposed scenarios shall be considered with dual TCI switching.

Proposal 2: Define dual TCI state switching requirements for following cases:

- PDCCH non-SFN: Two MAC CE with one for each TCI state

- PDCCH SFN: single MAC CE for two TCI states

- PDSCH single DCI: single DCI for two TCI states

- PDSCH multiple DCI: Two DCI with one for each TCI state

More specifically, the definition of dual TCI shall be clarified. For instance, it is possible that UE is already configured with (TCI1, TCI2) where TCI1 and TCI2 are with different QCL type D RS. With the UE movement, gNB may trigger TCI state switching to (TCI3, TCI4) where TCI3 and TCI4 are all configured with different QCL type D RS as TCI1 and TCI2. Or it is also possible that gNB commands UE from single TCI to dual TCI or in the other way around. Besides, whether to consider the case if only one of the TCI state is switched while the other one is unchanged. Thus, the definition of dual TCI state switch shall be clarified considering following cases:

- Single TCI to dual TCI

- Dual TCI to single TCI

- Dual TCI with changes of both QCL Type D RSs

- Dual TCI with change of only one of QCL type D RSs.

Proposal 3:

The definition of dual TCI state switch shall be clarified considering following cases:

- Single TCI to dual TCI

- Dual TCI to single TCI

- Dual TCI with changes of both QCL Type D RSs

- Dual TCI with change of only one of QCL type D RS.

The known/unknown conditions were also discussed with following proposals:

Issue 1-3-2: Known/unknown conditions for TCI state switching for multi-Rx chain - Proposals - P1: Study whether TCI state switching delay can be modified or not for multi Rx chain UE, e.g. how to evaluate known/unknown condition by using multi Rx chain, how to define TCI state change delay by the cases that dedicated Rx chain is used, partial Rx chains are used, or all Rx chains are used. - P1a: For MAC-CE based TCI state switch delay with unknown target TCI state, the reduction on switch delay requirements can be considered (i.e. consider the improvement of TL1-RSRP with multiple simultaneous reception) - P1b: For dual TCI state switching delay requirements, which scenarios of triggering, known/unknown TCI state and TCI association to different PCI could be considered and clarified. - P1c: Discuss the known conditions of dual TCI and discuss whether to define requirements for unknown dual TCI state switch.

The targeting scenario in the WID is that UE can perform simultaneous reception with different QCL type D TCI. If gNB configures dual TCI where UE cannot receive simultaneously, it does not make sense to define corresponding requirements. Thus, for dual TCI switching requirements, the conditions shall be considered that the two TCIs with different QCL type D RS shall be the ones that UE can simultaneously receive.

Observation 2: For dual TCI switching requirements, the conditions shall be considered that the two TCIs with different QCL type D RS shall be the ones that UE can simultaneously receive.

In Rel-17 FeMIMO, group based beam management is introduced where UE can report a pair of RSRP from two TRP/resource set, which is shown as follows. But it does not necessarily mean the two RSs are received in different panels. The main purpose of enabling simultaneous reception is for two panels for mTRP, thus dual TCI with QCL typeD switch which targeting the same panel of UE is not the interested scenario of the feature.

TS 38.214 - if the UE is configured with the higher layer parameter groupBasedBeamReporting-r17, the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance nrofReportedRSgroup, if configured, group(s) of two CRIs or SSBRIs selecting one CSI-RS or SSB from each of the two CSI Resource Sets for the report setting, where CSI-RS and/or SSB resources of each group can be received simultaneously by the UE.

In TCI state switching requirements, the performance is verified that UE is able to be scheduled using new TCI right after the required switching delay. In other words, it is indirectly verified by demodulation performance. Thus, for core requirements and performance requirements, it should be guaranteed that two directions with different QCL typeD are received by different UE panels.

Observation 3: The requirements shall apply provided that two directions with different QCL typeD are received by different UE panels.

There was corresponding discussion in RF session in last meeting in [2] where the selection of candidate AoA pairs for RF requirements were discussed. The essential point is same that the selected AoAs shall guarantee that the two directions are received by different panels of UE. Thus, the conclusion of RF session should be considered which shall work as the conditions for dual TCI switching.

Besides, it may also impact whether and how to define the know conditions for dual TCI switching. For instance, UE reports beam pair 1 (SSB 0, SSB 1) and beam part 2 (SSB 0, SSB 5) via group based beam reporting. However, it is possible that beam pair 1 is received within the same panel. If the configured dual TCI is QCL type D to beam pair 1, the switching delay cannot be verified as the demodulation performance will be poor. Based on analysis above, following conditions shall be considered for the known conditions:

- The UE has sent at least one L1-RSRP report for the target TCI states before the TCI state switch command where the associated QCL type D RSs are reported within one group configured by groupBasedBeamReporting-r17.

- The associated QCL type D RSs in target TCI states satisfy the conditions that the RSs are received from different panels, where the conditions shall follow RF conclusion.

Proposal 4: Following conditions shall be considered for the known conditions:

- The UE has sent at least one L1-RSRP report for the target TCI states before the TCI state switch command where the associated QCL type D RSs are reported within one group configured by groupBasedBeamReporting-r17.

- The associated QCL type D RSs in target TCI states satisfy the conditions that the RSs are received from different panels, where the conditions shall follow RF conclusion.

# Conclusions

Observation 1: Rel-17 unified TCI is not jointly considered with dual TCI, which is listed as an objective in Rel-18 MIMO.

Proposal 1: Define dual TCI state switching delay requirements base on Rel-15/16 TCI framework.

Proposal 2: Define dual TCI state switching requirements for following cases:

- PDCCH non-SFN: Two MAC CE with one for each TCI state

- PDCCH SFN: single MAC CE for two TCI states

- PDSCH single DCI: single DCI for two TCI states

- PDSCH multiple DCI: Two DCI with one for each TCI state

Proposal 3:

The definition of dual TCI state switch shall be clarified considering following cases:

- Single TCI to dual TCI

- Dual TCI to single TCI

- Dual TCI with changes of both QCL Type D RSs

- Dual TCI with change of only one of QCL type D RS.

Observation 2: For dual TCI switching requirements, the conditions shall be considered that the two TCIs with different QCL type D RS shall be the ones that UE can simultaneously receive.

Observation 3: The requirements shall apply provided that two directions with different QCL typeD are received by different UE panels.

Proposal 4: Following conditions shall be considered for the known conditions:

- The UE has sent at least one L1-RSRP report for the target TCI states before the TCI state switch command where the associated QCL type D RSs are reported within one group configured by groupBasedBeamReporting-r17.

- The associated QCL type D RSs in target TCI states satisfy the conditions that the RSs are received from different panels, where the conditions shall follow RF conclusion.

# References

[1] R4-2214344, WF on FR2 multi-Rx RRM, vivo

[2] R4-2214457, WF on FR2 multi-Rx chain DL reception, Sony