3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216332

Electronic Meeting, 10 – 19 October, 2022

Title: 	Discussion on RRM test cases for SDT

Source: 	Huawei, HiSilicon

Agenda item:	4.8.2

Document for:	Discussion

Introduction

RRM test cases for SDT are discussed in RAN4#104-e, and the outcomes are captured in [1]. Based on [1], the detailed setup for SDT RRM test cases need to be further discussed.

In this paper we will provide our views on setup for SDT RRM test cases.

Discussion

RAN5 has provided reply LS [2] on the feasibility of triggering UL data for SDT tests. RAN5 agreed that it is feasible to trigger UL transmission at the UE at a specific time based on test loop mode B and timer “T_delay_modeB”. As such, we understand

What can be controlled by the TE on the testing timeline are

- Time when RRCRelease message is transmitted

- Time when UL data is triggered at the UE

- Time when RSRP level is changed

What can be observed by the UE on the testing timeline is

- Time when UE transmits UL data

What cannot be controlled or observed by the TE are

- Time when UE measures RSRP1/2

- Time when UE performs TA validation

In [1] it is agreed to test the cases where UE should transmit with CG-SDT (TA validation passes) and where UE should not transmit with CG-SDT (TA validation fails) in a single test case. As UE only performs TA validation for the first CG-SDT transmission in a SDT session, there would be two SDT sessions in the test. Accordingly, there would be two RRCRelease messages and two UL data triggers.

The first RRCRelease can be transmitted when UE is in CONNECTED. The second RRCRelease can only be transmitted during the subsequent transmission of the first SDT session. Therefore, the first SDT session should be used to verify UE transmits with CG-SDT and the second UE does not transmit.

Figure 1 shows a test setup based on above discussion.

- During T1 UE is in CONNECTED, and T1 ends at the slot #n when the last PDSCH containing RRCRelease message #1 is transmitted, which is denoted as point A. During T1, the RSRP level of the serving cell is changed from rsrp_low to rsrp_high at point X, and point X should be earlier than point A by at least W1. W1 is the size of the measurement window of RSRP1 as defined in clause 5.5.3.

- T2 starts at point A and ends at the slot #m when the last PDSCH containing RRCRelease message #2 is transmitted, which is denoted as point D. During T2 UE is INACTIVE. UE is triggered to transmit UL data at point B, which is [AB] ms later than point A. We do not find clear restriction for length of [AB] and we are open to hear other views. As RSRP level stays at rsrp_high during T2, TA validation will pass and UE should transmit UL data using CG-SDT occasion after point B. Assuming UE performs TA validation ΔT ms after point B, the point C (where TE would observe UE transmits on CG-SDT occasion) should be within (640+ΔT) ms after point B per requirement in clause 5.5.3. Whether and when this assumption holds is discussed later. During the subsequent transmission after point C, TE will transmit RRCRelease message #2 at point D, which is [CD] ms after point C. We do not find clear restriction for length of [CD] and we are open to hear other views.

- T3 starts at point D, i.e. the slot #m when the last PDSCH containing RRCRelease message #2 is transmitted. During T3 UE is INACTIVE. UE is triggered to transmit UL data at point E, which is [DE] ms later than point D. In between point D and E, the RSRP level is changed from rsrp_high to rsrp_low at point Y. The distance between point Y and D should be larger than W1, and the distance between point Y and E should be larger than W2. As such, if UE is implemented correctly, RSRP1 should be rsrp_high, and RSRP2 should be rsrp_low. The gap between rsrp_high and rsrp_low should be set larger than the configured value for the RSRP change threshold plus the margin to account for measurement error, for which we suggest to take L1-RSRP measurement accuracy. The TA validation would fail, and UE should not transmit CG-SDT after point E. Of course, the test cannot last forever and we suggest the TE observes until (640+ΔT) ms after point F, during which there should be no CG-SDT transmission.

Figure 1: suggested timeline for SDT RRM test

Based on above test setup, what is verified are

- UE does not take too early measurement before RRCRelease as RSRP1 – if UE takes RSRP before point X as RSRP1, it will fail the TA validation in T2 and will not transmit with CG-SDT and fail the test.

- UE does not take too late measurement after RRCRelease as RSRP1 – if UE takes RSRP after point Y as RSRP1, it will pass the TA validation in T3 and will transmit with CG-SDT and fail the test.

- UE does not take too early measurement before TA validation as RSRP2 – if UE takes RSRP before point Y as RSRP2, it will pass the TA validation in T3 and will transmit with CG-SDT and fail the test.

- UE does not transmit with CG-SDT too late after TA validation – if point C is (640+ΔT) ms later than point B, UE will fail the test.

Proposal 1: Use the setup in Figure 1 for defining RRM TCs for SDT.

- UE should transmit UL data using a CG-ST occasion within (640+ΔT) ms after point B

- UE should not transmit UL data using CG-SDT occasion within (640+ΔT) ms after point E

As discussed above, the assumption needs to be made regarding when UE performs TA validation, i.e. UE performs TA validation ΔT ms after point B. Strictly speaking, this should be up to UE implementation. Ideally, UE could do TA validation earliest at data arrival (e.g. point B), but in practice it would be delayed. At least some internal processing time is needed before TA validation is triggered, and measurement time is also needed to obtain valid RSRP2.

- For the processing time, we suggest to define it as 10ms, which is re-used from RRC procedure delay.

- For the measurement time, we suggest to define it as 2/16 SMTC periods for FR1/FR2.

Proposal 2: ΔT is defined as (10ms + 2/16 SMTC periods) for FR1/FR2.

Conclusions

In this paper we provided our views on setup for SDT RRM test cases.

Proposal 1: Use the setup in Figure 1 for defining RRM TCs for SDT.

- UE should transmit UL data using a CG-ST occasion within (640+ΔT) ms after point B

- UE should not transmit UL data using CG-SDT occasion within (640+ΔT) ms after point E

Proposal 2: ΔT is defined as (10ms + 2/16 SMTC periods) for FR1/FR2.

Figure 1: suggested timeline for SDT RRM test

Reference

- R4-2214341, Way forward on RRM requirements and test cases for NR SDT, ZTE Corporation, Sanechips

- R5-225279, Reply LS on the feasibility of testing UE initiated SDT data transmission in RRC_INACTIVE, RAN5