3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-221xxxx

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	4.5.3.1.2

Source:	Ericsson

Title:	Discussion on PDCCH repetition requirements

Document for:	Discussion

# 1	Introduction

After the last RAN4 #104 meeting’s discussion, companies agreed on introducing PDCCH repetition requirement without specify the receiver assumption, and additional margin will be added to compensate the performance gap (if any).

Therefore, in this contribution, we are going to share our view on the exact value for additional margin based on the simulation results.

# 2	Discussion

In the last RAN4 #104-e meeting, companies agreed to add an additional margin to compensate the performance gap between different companies’ results. The exact value is under discussion with following options:

- Specify requirements without specific receiver assumption, additional margin will be included: - Additional margin - Option 1: [0.8] dB - Option 2: [1] dB - Other options not precluded

We capture the summarized simulation results from the last meeting with our results included, where ‘wo’ means ‘No soft-combining of received PDCCH symbols’ and ‘w’ means ‘Soft-combining of received PDCCH symbols’.

Table 2-1 So far received simulation results summary

| Duplex | Tx/RX | HW （w） | MTK (w0)-3dB | MTK (w)-3dB | Apple (w) | QC(wo-3dB) | E/// (w) | E/// (wo) | Gap (w) | Gap (w0) | Gap(w wo) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 2x2 | -1.71 | 1.03 | -0.59 | -1.4 | 0.56 | -2.3 | -0.8 | 1.71 | 1.83 | 3.33 |
| FDD | 2x4 | -5.78 | -2.93 | -4.5 | -5.2 | -3.5 | -6.1 | -4.6 | 1.6 | 1.67 | 3.17 |
| TDD | 2x2 | -1.37 | 1.15 | -0.43 | -1.2 | 1.45 | -2.3 | -0.9 | 1.87 | 2.35 | 3.75 |
| TDD | 2x4 | -5.73 | -2.89 | -4.4 | -4.8 | -2.87 | -6.2 | -5.0 | 1.8 | 2.13 | 3.33 |

From the received simulation results we can conclude:

Observation 1: For FDD, 2Rx, the performance gap so far between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB.

Observation 2: For FDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.17dB.

Observation 3: For TDD, 2Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.75dB.

Observation 4: For TDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB.

We are expecting more results received in this meeting. If the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ is larger than 3dB but less than 4dB, we suggest to consider an additional 1dB margin to be added on top of alignment results.

Proposal 1: More results from different companies are needed. Consider an additional 1dB margin to be added on top of alignment results if the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ is larger than 3dB but less than 4dB.

# 3	Summary

In summary, we shared our views on selecting the exact value for the additional margin. We summarize our observations and proposals as follows:

Observation 1: For FDD, 2Rx, the performance gap so far between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB.

Observation 2: For FDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.17dB.

Observation 3: For TDD, 2Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.75dB.

Observation 4: For TDD, 4Rx, the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ so far is 3.33dB.

Proposal 1:  More results from different companies are needed. Consider an additional 1dB margin to be added on top of alignment results if the performance gap between the best result of ‘with soft-combining’ and the worst result of ‘without soft-combining’ is larger than 3dB but less than 4dB.

# References

[1]	R4-2214397: WF on UE demodulation and CSI requirements for FeMIMO, Samsung

# Annex

Simulation assumption for PDCCH repetition

Table 1: Parameter configurations

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15 kHz SCS | TDD 30kHz SCS |
| Repetition transmission schemes | FDM |  |
| CBW | 10 MHz | 40MHz |
| CORESET RB | 24 | 48 |
| CORESET duration | 2 |  |
| Aggregation level | AL=2 |  |
| Propagation Condition | TDLA30-10 |  |
| Antenna configuration | 2x2, 2x4 |  |
| CCE to REG mapping type | Non-Interleaved |  |
| REG bundle size | 6 |  |
| Payload bits(without CRC) | 39 | 41 |
| Test metric | 1% of Pm-dsg (%) |  |
| Operating SNR condition for selection AL | SNR > -4 dB |  |
| SNR setting for each TRP | The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP |  |
| Time offset/Frequency offset for each TRP | timing offset =-0.5us, frequency offset=200Hz | timing offset =-0.25us, frequency offset=300Hz |
| RB allocation | Frequency non-overlapping |  |
| Note: Reuse other test parameters of existing Rel-16 multi-DCI based on TRP transmission test case (Table 5.2.2.1.12-2) with different PCI for TP1 and TP2 |  |  |