3GPP TSG-RAN WG4 Meeting # 104-bis-e 	R4-2216422

Electronic Meeting, October 10-19, 2022

Source:	Ericsson

Title:	Requirements for DRX case

Agenda item:	4.7.1

Document for:	Approval

# 1	Introduction

In RAN4#104 WF [1] it was agreed to:

... Sub-topic 1-4 Shall RAN4 define PDC requirements when DRX is in use. Agreement: - RAN4 will define PDC requirements when DRX is in use Way Forward: - Companies to bring discussion and proposal for next meeting addressing the open issue: How to develop requirements when DRX is in use ...

In this contribution, we share our views on the topic.

# 2	Discussion

For integration with TSN networks then periodic delivery of timing packets are expected and accurate 5GS clock is needed for packet residence time compensation. Accurate 5GS timing would require regular connection to gNB for time and frequency stability.

Observation 1: As pointed out above, accurate TSN clocks are delivered periodically. Will there be time for DRX sleep?

In existing specification [2], we have the following:

### ... 7.1.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH. - when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive. ...

Waking up after DRX is an inaccurate process the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns. For PDC, specifically, it needs to reflect the delay and channel present when the 5GS time was distributed and hence there is a dependency between the 5GS delivery and PDC occasion (when PDC measurement is executed).

Observation 2: Waking up after DRX is an inaccurate process, the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns.

However, if we can make sure that the UE, after waking up after DRX sleep, can acquire the full PRS or TRS bandwidth with the stipulated number of samples, then it is expected that the stipulated minimum performance (without DRX) can be fulfilled. This is already the case for existing positioning requirements for UE Rx-Tx time difference:

“All measurement requirements specified in clause 9.9.2, 9.9.3 and 9.9.4 shall apply without DRX as well as for any DRX configuration specified in TS 38.331”.

Observation 3: Existing positioning requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331.

The same principle can be applied to UE Rx-Tx time difference for PDC.

Proposal 1: Existing PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331.

# 3	Summary

Observation 1: As pointed out above, accurate TSN clocks are delivered periodically. Will there be time for DRX sleep?

Observation 2: Waking up after DRX is an inaccurate process the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns.

Observation 3: Existing positioning requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331.

Proposal 1: Existing PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331.

# References

- R4-2214338, WF on NR_IIOT_URLLC_enh, Nokia

- TS 38-133, Requirements for support of radio resource management