3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216437

Electronic Meeting, 10 – 19 Oct, 2022

Title:	R18 Discussion on 8Rx FWA

Source:	OPPO

Agenda item:	6.6.3

Document for:	Approval

- Introduction

In Rel-18, 8Rx for FWA/CPE/vehicle/industrial devices was one of the objectives for FR1 enhancement as below [1].

Enable 8Rx for CPE/FWA/vehicle/industrial devices - Example bands: - TDD bands: n41, n77/ n78 - FDD bands: n7 - Note 1: the total number of example band should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 8Rx - Study and specify the requirements to support SRS antenna switching for t1r8, t2r8, t4r8 - Discussion on t4r8 shall start after at least one PC for 4Tx is completed - NOTE: Requirements are specified with phase approach. Objectives with 1st priority are considered first.

And in last meeting the WF [4] was approved with open issues captured. This paper is update of paper [5] from last meeting.

- Discussion

2.1 Delta Rib for 8Rx

8Rx for LTE was introduced in Rel-15 and the only change was for Refsens requirement which delta Rib for 8Rx was defined [2]. And the delta RIB,8R is defined as -4dB which considered the different ILs and also potential couplings among Rx paths in the RFFE due to the different antenna locations. This was similar as delta RIB,4R where these issues were also considered but is even worse for 8Rx.

These similar issues were also existing in NR, the difference comparing to LTE is that CPE/FWA will have larger size than smartphone, and some factors could be improved like the coupling among Rx paths, while some factors will be worse like the IL differences. For the factor of antenna correlation, it doesn’t impact the RF requirements. Therefore, overall in our view there is no much room for the improvement comparing to LTE 8Rx, and the value could be around -4.5dB as the delta RIB,8R.

Observation 1:    -4dB for delta RIB,8R was defined for LTE considering the IL differences among different Rx paths and similar issues exist in NR CPE/FWA and some factors could be worse while other factors could be better.

Proposal 1:         Consider delta RIB,8R for NR CPE/FWA as -4.5dB.

2.2 SRS IL

SRS IL for t1r8

When UE indicates the SRS antenna switching capability t1r8, the SRS IL is defined as the relative additional IL of the 2nd, 3rd, 4th, 5th, 6th, 7th, and 8th antenna comparing to the 1st antenna (main antenna). The SRS IL may include the ILs caused by switches, filters, and the PCB trace/RF cable/routing losses, and so on [3]. Besides, the PA impacts were also considered in Rel-16, e.g. TxD caused main antenna Tx power losses, and also the PC2 UE with PC3 PA transmit SRS in antennas other than main antenna caused additional 3dB IL.

Observation 2:    SRS IL analysis should consider the ILs caused by switches, filters, the PCB trace/RF cable/routing losses, and also the PA impacts like TxD and PC2, etc.

Below figure 1 is one of the typical implementation architectures of t1r8 in the FWA, where LPAF is the module includes LNA/PA/Filter/switch and is same for different antennas, and DRX-M is the diversity receive module which includes the LNA and filters. Different antenna locations will lead to different routing losses (PCB trace/RF cable).

Figure 1 UE architecture of t1r8 SRS switch

The SRS IL link budget can be found in table 1 which shows the largest additional IL among Ant 2/3/4/5/6/7/8 comparing to the Ant 1, i.e. SRS to Ant 5/6/7/8 comparing to SRS to Ant 1 with component IL parameters shown in table 2. It can be seen that the additional IL is 3.5dB at 3.5GHz and 4.4dB at 4.9GHz with normal condition, and considering the ETC impacts can be rounded to 4dB@3.5GHz, and 5dB@4.9GHz.

Table 1 Additional SRS IL for t1r8

|  | Additional IL @ 3.5GHz | Additional IL @ 4.9GHz |
| --- | --- | --- |
| Ant 5/6/7/8 – Ant 1 | SP4T + SPDT + PCB trace loss = 3.5dB | SP4T + SPDT + PCB trace loss = 4.4dB |

Table 2 Component IL for 3.5GHz and 4.9GHz

|  | 3.5GHz | 4.9GHz |
| --- | --- | --- |
| DP4T IL | 1.3 | 1.6 |
| DPDT IL | 0.8 | 1.1 |
| SP4T IL | 1 | 1.1 |
| SPDT IL | 0.7 | 0.9 |
| PCB trace IL (SRS path) | 1.8 | 2.4 |
| Note1: Component IL is varying at different frequency point of the band, max IL in the band under normal condition is used. Note2: PCB trace loss used assumes the trace length is 60mm. |  |  |

Proposal 2:         The additional IL for t1r8 is 4dB @3.5GHz, and 5dB @4.9GHz when SRS transmits to the antenna other than the main antenna.

SRS IL for t2r8

When UE SRS antenna switch capability is indicated as t2r8, one of the typical UE architecture could be as shown in figure 2. In this case, each LPAF will transmit to four antennas. And the largest additional IL comparing to main antenna 1 and 3 is from LPAF to Ant 5/6 or Ant 7/8 respectively.

Figure 2 UE architecture of t2r8 SRS switch

The SRS IL link budget can be found in table 3 which shows the largest additional IL among Ant 5/6 comparing to the Ant 1, or among Ant 7/8 comparing to the Ant 3, with component IL parameters shown in above table 2. It can be seen that the additional IL is 2.5dB at 3.5GHz and 3.3dB at 4.9GHz, and can be rounded to 3dB@3.5GHz and 3.5dB@4.9GHz.

Table 3 Additional SRS IL for t2r8

|  | Additional IL @ 3.5GHz | Additional IL @ 4.9GHz |
| --- | --- | --- |
| Ant 5/6 – Ant 1 Or Ant 7/8 – Ant 3 | SPDT + PCB trace loss = 2.5dB | SPDT + PCB trace loss = 3.3dB |

Proposal 3:         The additional IL for t2r8 is 3dB @3.5GHz, and 3.5dB @4.9GHz when SRS transmits to the antenna other than the main antennas.

SRS IL for t2r8 and t1r8

When UE SRS antenna switch capability is indicated as t2r8 and t1r8, the UE implementation probably could be two types, i.e. UE use one PA to transmit SRS to all antennas when configured t1r8, or UE use two PAs to transmit SRS with each cover four antennas, as has been discussed in [3] named as “hardware solution” and “software solution”.

More specifically, the “software solution” is same as Figure 2 above. The two LPAF will transmit in turn when works under t1r8. When NW configure UE with eight SRS resources and each resource contains one antenna port, then in the 1~4 SRS resource activate the first LPAF and in 5~8 SRS resource activate the second LPAF, as illustrated in figure 3. In this case, the SRS IL will be same as the t2r8. However, this relies on UE have the capability of switched PA transmission.

Figure 3 Switched PA transmission when configured with 1 antenna port in each SRS resource

Observation 3:    For UE with t2r8 and t1r8 SRS switch capabilities and also the capability of switched PA transmission, the SRS IL can be same as t2r8 SRS IL, i.e. 3dB @3.5GHz, and 3.5dB @4.9GHz.

Another more general implementation for UE with t2r8 and t1r8 SRS switch capability is using one PA transmit SRS to all the antennas as shown in figure 4. And the largest IL is from LPAF1 to Ant 4/7/8.

Figure 4 One PA transmit SRS to all antennas for UE with t2r4 and t1r4 capability

The SRS IL link budget can be found in table 4 which shows the largest additional IL between Ant 4/7/8 comparing to the Ant 1, with component IL parameters shown in above table 2. It can be seen that the additional IL is 4.5dB at 3.5GHz and 5.8dB at 4.9GHz, and considering ETC impacts it can be rounded to 5dB@3.5GHz and 6dB@4.9GHz.

Table 4 Additional SRS IL for t2r8 and t1r8

|  | Additional IL @ 3.5GHz | Additional IL @ 4.9GHz |
| --- | --- | --- |
| Ant 4/7/8 – Ant 1 | SPDT + DP4T + SPDT + PCB trace loss = 4.5dB | SPDT + DP4T + SPDT + PCB trace loss = 5.8dB |

Observation 4:    For UE with t2r8 and t1r8 SRS switch capabilities but without the capability of switched PA transmission, the SRS IL is 5dB@3.5GHz and 6dB@4.9GHz.

It can be seen that for t2r8 and t1r8 SRS capability, if UE support switched PA transmission the SRS IL is smaller, but this may not be supported by all UEs. Then there probably have two approaches to handle this, one is to define new capability to indicate whether UE supports this switched PA transmission in t1r8 and introduce two requirements, the other is to define one requirement accommodate different implementations. The latter one may be simpler for the spec.

Observation 5:    Two approaches to handle UE with or without switched PA transmission in t2r8 and t1r8:

- One is to define new capability for switched PA transmission in t1r8 and introduce two requirements.

- The other is to define one requirement accommodate different implementations.

Proposal 4:         The additional IL for t2r8 and t1r8 is defined as 5dB@3.5GHz and 6dB@4.9GHz for all UEs.

In summary, the SRS additional IL is shown in table 4 with different SRS transmission capabilities.

Table 4 Summary of additional ILs for different SRS transmission capabilities

| UE capability | Additional IL @ 3.5GHz | Additional IL @ 4.9GHz |
| --- | --- | --- |
| t1r8 | 4 dB | 5 dB |
| t2r8 | 3 dB | 3.5 dB |
| t2r8 and t1r8 | 5 dB | 6 dB |

2.3 Non-zero IL for main antennas

It was captured in the WF [4] that “Further study if the assumption that zero delta is applied for the transmission power calculation for the first SRS resource still valid”. It was raised in last meeting that the ILs are higher at main antennas comparing to 4Rx and 2Rx cases. It is indeed true that higher ILs are caused by more complex frontend design. Usually this is left to UE implementation to solve unless the IL too high like the 3dB caused by TxD in Rel-17. But we are open for this discussion.

Observation 6:    Non-zero IL for main antennas was raised considering the more complexed frontend design for 8Rx.

2.4 Pcmax,h improvement

The potential Pcmax,h improvement issue was also raised in last meeting, i.e. whether it is possible to not reduce the 3dB power level for Pcmax,h when UE supports 2Tx but transmit with 1Tx as seen in below table.

For a UE that supports 2Tx and 1T8R SRS AS, further study whether 3dB power back off at main antenna defined for TxD (ΔPPowerClass) is applied for PCMAX_H,f,c or not.

The 3dB power back off at main antennas was defined in Rel-17 for TxD. And it considers UE support TxD but transmit with 1Tx for SRS signal then only one of the PAs will be activated which caused the 3dB power back off at main antenna. And the 3dB power back off will be applied to both Pcmax,L and Pcmax,H. In our view, this is the normal handling of Pcmax boundaries, however, we are open to the potential enhancement in the improvement of Pcmax,h if UE can transmit higher power.

Observation 7:    3dB power back off was defined in Rel-17 for the case that UE support TxD but only one PA transmit in t1r4 then the max power at main antenna will be reduced by 3dB. And this 3dB will be applied to both Pcmax,L and Pcmax,H.

- Conclusion

In this paper, we discussed requirements for 8Rx, and one is the delta RIB,8R, the other is the SRS IL for t1r8, t2r8 and both. Below are the observations and proposals.

Observation 1:    -4dB for delta RIB,8R was defined for LTE considering the IL differences among different Rx paths and similar issues exist in NR CPE/FWA and some factors could be worse while other factors could be better.

Proposal 1:         Consider delta RIB,8R for NR CPE/FWA as -4.5dB.

Observation 2:    SRS IL analysis should consider the ILs caused by switches, filters, the PCB trace/RF cable/routing losses, and also the PA impacts like TxD and PC2, etc.

Proposal 2:         The additional IL for t1r8 is 4dB @3.5GHz, and 5dB @4.9GHz when SRS transmits to the antenna other than the main antenna.

Proposal 3:         The additional IL for t2r8 is 3dB @3.5GHz, and 3.5dB @4.9GHz when SRS transmits to the antenna other than the main antennas.

Observation 3:    For UE with t2r8 and t1r8 SRS switch capabilities and also the capability of switched PA transmission, the SRS IL can be same as t2r8 SRS IL, i.e. 3dB @3.5GHz, and 3.5dB @4.9GHz.

Observation 4:    For UE with t2r8 and t1r8 SRS switch capabilities but without the capability of switched PA transmission, the SRS IL is 5dB@3.5GHz and 6dB@4.9GHz.

Observation 5:    Two approaches to handle UE with or without switched PA transmission in t2r8 and t1r8:

- One is to define new capability for switched PA transmission in t1r8 and introduce two requirements.

- The other is to define one requirement accommodate different implementations.

Proposal 4:         The additional IL for t2r8 and t1r8 is defined as 5dB@3.5GHz and 6dB@4.9GHz for all UEs.

Observation 6:    Non-zero IL for main antennas was raised considering the more complexed frontend design for 8Rx.

Observation 7:    3dB power back off was defined in Rel-17 for the case that UE support TxD but only one PA transmit in t1r4 then the max power at main antenna will be reduced by 3dB. And this 3dB will be applied to both Pcmax,L and Pcmax,H.

In summary, the SRS additional IL is shown in table 4 with different SRS transmission capabilities.

Table 4 Summary of additional ILs for different SRS transmission capabilities

| UE capability | Additional IL @ 3.5GHz | Additional IL @ 4.9GHz |
| --- | --- | --- |
| t1r8 | 4 dB | 5 dB |
| t2r8 | 3 dB | 3.5 dB |
| t2r8 and t1r8 | 5 dB | 6 dB |

- References

[1] RP-221496, New WID on Further RF requirements enhancement for NR frequency range 1 (FR1)

[2] R4-1803450, CR for 36.101: Reference sensitivity power level, HW

[3] R4-1812719, Further discussion on additional IL caused by SRS switch, OPPO

[4] R4-2214451, WF on FR1 8Rx UE RF requirements, DCM

[5] R4-2213312, R18 Discussion on 8Rx FWA, OPPO