3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216471

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Amendments on cell reselection parameters when not using enhanced mode

Agenda item:	4.2.6.2

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. Following the progress of RAN4 on the topic, it was approved an extension for the WI activities, in order to address eventual open issues. The WI exception list provided in [2] includes RAN4 RRM requirements, with provision for clarifications and addressing possible inconsistencies.  This contribution proposes further clarification for issues regarding the consistency of GNSS-related operation by UEs in NTN. In [3] and [4]several issues are listed, of which we discuss several in the following sections.

Discussion

In previous meetings, RAN4 has agreed, and captured accordingly on the specifications, that the “UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area” (t-Service).

And as this requirement also included the “detection” of the new cell, it was decided that  “This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger”

Based on the definition of the parameter Ttrigger however and its definition this leads to following observation:

Observation 1: Currently,  Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter) when serving cell is below the search threshold, in order to accommodate the detection time of the serving cell.

The definition of the parameters than continues to say that in the formulation above:

-	Tdetect,NR_Intra is HST intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2.2.3-2,

-	Tdetect,NR_Inter  is HST inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2.2.4-2.

However, the following observation need to be made:

Observation 2: In the current definition,  Ttrigger is only considering the enhanced version of the detection time (used  for high speed conditions).

However, RAN4 has also previously discussed that the used of the enhanced parameters, for high mobility, is more relevant for the earth-moving cells. For quasi-earth fixed scenarios, where t-Service is applicable, the enhanced parameters may or may not be applied. If it is not applied, the UE will not have time for detect the cell before Ttrigger .

Proposal 1:  Adapt Ttrigger formulation to use the values of Tdetect,NR_Intra and Tdetect,NR_Inter  also considering the cases UE is not using the enhanced parameters for high mobility.

Another potential problem is on the sentence ““This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started…”.

The UE doesn’t know when the transmission of the t-Service SI has started but, can only know when it started reading it. Moreover, the value of t-Service may be updated by the network during an SI modification period, and it is ambiguous if the new SI modification window should be used as reference instead of the “ SI modification period where the broadcasting of t-Service is started”. We propose to clarify that the requirement does not apply if the UE learns about t-Service within the time defined by Ttrigger.

Proposal 2: For Ttrigger applicability replace the condition:

“This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started…”

By

“This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is first acquired by the UE…”

Conclusion

Observation 1: Currently,  Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter) when serving cell is below the search threshold, in order to accommodate the detection time of the serving cell.

Observation 2: In the current definition,  Ttrigger is only considering the enhanced version of the detection time (used  for high speed conditions).

Proposal 1:  Adapt Ttrigger formulation to use the values of Tdetect,NR_Intra and Tdetect,NR_Inter  also considering the cases UE is not using the enhanced parameters for high mobility.

Proposal 2: For Ttrigger applicability replace the condition:

“This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of t-Service is started…”

By

“This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is first acquired by the UE…”

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-220209, Rel-17 Work Item Exception for NR-NTN-solutions, Thales

- R4-2214471, WF on NR NTN RRM requirements, Qualcomm Inc.

- R4-2214473, WF on performance part for NR NTN, Xiaomi