3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216482

Electronic Meeting, 10 – 19 Aug, 2022

Source:	ZTE Corporation

Title:	Discussion on RRM requirements for joint consideration between pre-MG, concurrent MG and NCSG on NR and MR-DC

Agenda Item:	6.10.2

Document for:	Approval

- Introduction

In RAN plenary 95 meeting, a new WID[1] for Rel-18 was approved to identify further enhancement on NR and MR-DC Measurement Gaps and Measurements without Gaps. Two aspects were included by this WID. The 1st aspect oriented towards the joint consideration of pre-configured MGs, concurrent MGs and NCSG. The 2nd aspect focus on RRM measurements without gap. We discuss the 2nd aspect in another document. For the 1st aspect, the objective in the WID is as follows:

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or multiple concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MGs and multiple concurrent MGs (i.e., concurrent MGs where at least one of the gaps is a pre-configured gap) - Case 2: NCSG and multiple concurrent MGs (i.e., concurrent MGs where at least one of the gaps is NCSG) - Note: Prioritization among other possible combinations of pre-configured MG, concurrent MG and NCSG can be discussed in WI phase

- While during recent RAN plenary 97 meeting, the WID was further revised in [2] as follows:

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in Case 1 and Case 2 - Note 2: The requirement discussions on the scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 will be started after RAN#99. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG, NTN gaps and NCSG can be discussed after RAN#99 - Note 4: This WID does not include any inter-working with MUSIM gaps

- The clarification of scenario was discussed in detail during 104 meeting, now based on the revised WID achieved in RAN plenary 97, it seems that the scenario for current stage is clear, i.e. NTN MG is not considered in Case 1 and Case 2. NCSG is not considered in Case 1 before RAN#99, and pre-configured MG is not considered in Case 2 before RAN#99. Further more, it is clear that MUSIM gap is not considered in this WI.

- Except for the discussion on scenario, during 104 meeting, RAN4 still has discussion about other issues, but in general not multiple detailed agreements achieved in [3]. In this document, we provide some further analysis on the joint consideration of enhancements of pre-configured MGs, multiple concurrent MGs and NCSG.

- Discussion

During Rel-17 phase, three topics i.e. pre-configured MGs, multiple concurrent MGs and NCSG  about measurement gap enhancement were discussed to achieve different aspects of enhancements. To consider more comprehensive, joint of the three topics were proposed during Rel-17 phase. However due to the timeline pressure, the joint consideration of the three topics was deprioritized in Rel-17.

In our opinion, pre-configured MG and NCSG are both enhanced measurement gap. Pre-configured MG can realize flexible activation/deactivation of MG according to the dynamic active BWP switching or some RRC re-configuration. NCSG is a “light” MG which can highly reduce the impact of interruption and scheduling restriction lead by measurement operation.

The enhancements of pre-configured MG and NCSG have respective motivations and can be applied to achieve specific performance gains. At the same time, to keep compatibility with existing legacy MG, supporting multiple concurrent MGs is necessary, so that UE can support pre-configured MG or NCSG, on the base of legacy MG supporting.

Observation 1: Supporting concurrent MG is the precondition to realize performance gain caused by pre-configured MG or NCSG and keep compatibility with existing legacy MG simultaneously.

2.1 Scope and Scenario

No matter for Case 1 or Case 2 proposed in the WID, we believe the multiple concurrent MGs can be the basic framework to allow multiple MGs belonging to same kind or different kinds coexistence at the same time. So the specific MG combinations under Case 1 or Case 2 should be clarified first. In our opinion, the following combinations all should be considered. Further more, the total number of MGs should follow the conclusion approved in the discussion of concurrent MGs in Rel-17.

- Case 1: Joint of pre-configured MGs and concurrent MGs

- C1-1: pre-configured MG + legacy MG

- C1-2: pre-configured MG + pre-configured MG

- Case 2: Joint of NCSG and concurrent MGs

- C2-1: NCSG + legacy MG

- C2-2: NCSG + NCSG

Proposal 1: All the following combinations of MGs should be supported, and the total number of MGs should follow the conclusion approved in Rel-17 concurrent MGs discussion.

- Case 1: Joint of pre-configured MGs and concurrent MGs

- C1-1: pre-configured MG + legacy MG

- C1-2: pre-configured MG + pre-configured MG

- Case 2: Joint of NCSG and concurrent MGs

- C2-1: NCSG + legacy MG

- C2-2: NCSG + NCSG

Under the framework of concurrent MGs, we should pay more attention to two important characteristics, i.e. the association between MG and MO/frequency layer, priority order. In our view, such two characteristics are fundamental for the concurrent MG structure. So it is preferred that for all cases, it is better for NW to configure the association and priority order for each MG so as to avoid the ambiguity.

Proposal 2: For all cases, it is better for NW to configure the association and priority order for each MG so as to avoid the ambiguity.

Since we discuss the joint consideration for NR and MR-DC, at least NR CA and MR-DC should be supported for each case. Especially for C1-1 and C1-2, how to support multiple MGs including pre-configured MG in CA case, which should be identified due to we have limitation on the application of pre-configured MG for CA case during Rel-17. Consequently, the trigger events under the UE autonomous mechanism of pre-configured MG should be further studied under CA and MR-DC cases.

Further more, the structure of concurrent MGs under MR-DC should be further studied, which is not allowed during Rel-17. To support the above combination cases, we should firstly identify how to extend the concurrent MGs structure under MR-DC case. For NCSG, we also have the limitation that not supporting NCSG for MR-DC case during Rel-17.

Proposal 3: Firstly identify how to extend the concurrent MGs structure into MR-DC case, then further study the application of pre-configured MG into CA and MR-DC case, and the application of NCSG into MR-DC case.

Overlapping issue is inevitable for the coexistence of multiple MGs in each case. The proximity condition for overlapping has been identified during concurrent MGs in Rel-17. But refer to NCSG, due to the different components in a NCSG pattern, whether the current proximity condition can be re-used, which should be further discussed.

Proposal 4: Whether the current proximity condition defined in Rel-17 concurrent MGs can be re-used for NCSG, which should be further discussed.

For the measurement related requirements, such as CSSF, scheduling restriction, impact on L1 measurement, it is preferred to re-use the requirements approved in Rel-17 concurrent MGs as far as possible, so as to relieve the workload in this WID.

Proposal 5: It is preferred to re-use the measurement related requirements approved in Rel-17 concurrent MGs as far as possible, so as to relieve the workload in this WID.

Referring to the ePOS gap, it has been approved in last meeting that Rel-17 MGE Pre-MG for RRC-based POS is in the scope of this WI. However whether Rel-17 MAC-CE based POS gap is in the scope or not, still suspending. In our opinion, MAC-CE based ePOS gap is the enhancement of R17 ePOS WI, not in the scope of R17 Pre-MG scope. So it is not in the scope of this Rel-18 WI.

Proposal 6: Rel-17 MAC-CE based ePOS gap is not in the scope of this WI.

2.2 Inherit Rel-17 framework

So as to avoid the discussion on each agreements achieve in Rel-17 phase, in last meeting, we discussed which Rel-17 pre-configured MG principles should be revisited. The following options were kept FFS in the WF:

- FFS: For UE autonomous mechanism, only the measurements associated to the concerned pre-MG are used for the rule checking

- FFS: For Network-controlled mechanism, only the bits corresponding to the concerned pre-MG are used for determining the status

- FFS whether to consider the other potential enhancement

- We agree with the first two bullets. We do not agree to introduce additional rule outside the Rel-17 agreements around pre-configured MG.

- Proposal 7: Around pre-configured MG, the following two rules are acceptable, and other additional rule outside Rel-17 agreements is not recommended by us.

- For UE autonomous mechanism, only the measurements associated to the concerned pre-MG are used for the rule checking

- For Network-controlled mechanism, only the bits corresponding to the concerned pre-MG are used for determining the status

- Around concurrent gap, no matter pre-configured MG, legacy MG or NCSG is configured under the framework of concurrent gap, we believe reusing the priority rule, association rule and number restriction which have been specified under Rel-17 concurrent gap is enough, not need to introduce additional new principles to address the same issue.

- Proposal 8: About to concurrent gap, reusing the priority rule, association rule and number restriction which have been specified under Rel-17 concurrent gap is enough, not need to introduce additional new principles to address the same issue.

- In Rel-17 concurrent gap, RAN 4 has identified the maximum number of concurrent gaps. We can see the further clarification on Case 1 and Case 2 were approved, i.e. now the NTN gap, MUSIM gap are not considered in Case 1 and Case 2. So the maximum number of gaps for Case 1 and Case 2 should comply with the requirements defined in Rel-17 concurrent gap, no obvious cause to extend the maximum number of gaps under the framework of concurrent gap.

- Proposal 9: The maximum number of gaps for Case 1 and Case 2 should still comply with the requirements defined in Rel-17 concurrent gap.

2.3 Collision definition and handling

- In Case 1, directly reusing the definition of collision under Rel-17 concurrent gap is enough, we can not see the necessity of introducing additional new collision definition and handling since pre-configured MG is the gap which can be dynamically or semi-statically activated/deactivated, similar as legacy gap essentially.

- In Case 2, in last meeting, it has been approved that when determining whether a collision occurs between a NCSG instance and another gap instance, the total NCSG duration including both the VILs and the ML should be considered. If one NCSG is configured to UE by NW, which means UE has an idle RF chain besides the RF chain used for data reception, and such idle RF chain can be used to perform NCSG based RRM measurement. But if another legacy gap configured to the UE, it is a bit complicated since the necessity of NCSG is per band. So whether UE can simultaneously perform NCSG based measurement and legacy gap based measurement at the overlapping case, in our opinion, which depends on the band of the associate MO.

- Take the following figure. 1as an example. NCSG+MG are configured for the UE. MO1 and MO2 are associated with the NCSG, therein MO1 is in band A, and MO2 is in band B. UE reports ‘NCSG’ for band A, however reports ‘no NCSG no gap’ for band B. During the three overlapping instance{#0, #1, #2}, in fact for Instance#1, neither of NCSG and MG needs to be canceled since UE has enough RF chains to perform RRM measurement for MO1 and MO3.

- But for Instance #0 and #2, whether one of NCSG and MG should be canceled or not, which should be discussed. From the perspective of NCSG, during ML, UE can simultaneously perform MO2 measurement and data reception, then UE can not perform MO3 measurement since of no RF chain, i.e. the MG should be canceled. While from the perspective of legacy MG, during legacy MG, the interruption of data reception would exist due to MO3 measurement. In fact the NCSG based MO2 measurement degrades to legacy MG based measurement since the interruption exists within the whole ML+2VIL. But neither of MG and NCSG should be canceled. It seems that two different results are both possible, i.e. canceling legacy MG or keeping both NCSG and MG but the degradation of NCSG would happen in fact. Maybe we can decide between the two possibilities depending on the priority. If NCSG has higher priority, which means the handling should come from the perspective of NCSG, i.e. legacy MG should be canceled. While if legacy MG has higher priority, the handling should come from the perspective of legacy MG, i.e. both NCSG and MG is not be canceled but degradation of NCSG happens.

- Figure. 1

- Based on above analysis on Figure.1, we have the following proposals.

- Proposal 10: For Case 2, the collision handling can be further checked since in fact the gap canceling is not always necessary when collision happens since of the necessity of NCSG is per band for the UE capable of NCSG.

- For the collision instance, if no MO needs NCSG, no need to cancel any one between NCSG and MG(NCSG);

- For the collision instance, if at least one MO needs NCSG, there are two possible solutions of collision handling:

  - keep both NCSG and MG(NCSG) at the price of NCSG degradation to legacy MG;

  - Cancel the MG or the lower priority of NCSG.

- Which solution should be applied, it can be decided by the priority order. If the NCSG has higher priority than MG, then cancel the MG; Otherwise, neither of them would be canceled but at the price of NCSG degradation to MG.

- Conclusion

In this contribution, we have the following observation and proposals for joint consideration between pre-MG, concurrent MG and NCSG on NR and MR-DC:

Proposal 1: All the following combinations of MGs should be supported, and the total number of MGs should follow the conclusion approved in Rel-17 concurrent MGs discussion.

- Case 1: Joint of pre-configured MGs and concurrent MGs

- C1-1: pre-configured MG + legacy MG

- C1-2: pre-configured MG + pre-configured MG

- Case 2: Joint of NCSG and concurrent MGs

- C2-1: NCSG + legacy MG

- C2-2: NCSG + NCSG

Proposal 2: For all cases, it is better for NW to configure the association and priority order for each MG so as to avoid the ambiguity.

Proposal 3: Firstly identify how to extend the concurrent MGs structure into MR-DC case, then further study the application of pre-configured MG into CA and MR-DC case, and the application of NCSG into MR-DC case.

Proposal 4: Whether the current proximity condition defined in Rel-17 concurrent MGs can be re-used for NCSG, which should be further discussed.

Proposal 5: It is preferred to re-use the measurement related requirements approved in Rel-17 concurrent MGs as far as possible, so as to relieve the workload in this WID.

Proposal 6: Rel-17 MAC-CE based ePOS gap is not in the scope of this WI.

- Proposal 7: Around pre-configured MG, the following two rules are acceptable, and other additional rule outside Rel-17 agreements is not recommended by us.

- For UE autonomous mechanism, only the measurements associated to the concerned pre-MG are used for the rule checking

- For Network-controlled mechanism, only the bits corresponding to the concerned pre-MG are used for determining the status

- Proposal 8: About to concurrent gap, reusing the priority rule, association rule and number restriction which have been specified under Rel-17 concurrent gap is enough, not need to introduce additional new principles to address the same issue.

- Proposal 9: The maximum number of gaps for Case 1 and Case 2 should still comply with the requirements defined in Rel-17 concurrent gap.

- Proposal 10: For Case 2, the collision handling can be further checked since in fact the gap canceling is not always necessary when collision happens since of the necessity of NCSG is per band for the UE capable of NCSG.

- For the collision instance, if no MO needs NCSG, no need to cancel any one between NCSG and MG(NCSG);

- For the collision instance, if at least one MO needs NCSG, there are two possible solutions of collision handling:

  - keep both NCSG and MG(NCSG) at the price of NCSG degradation to legacy MG;

  - Cancel the MG or the lower priority of NCSG.

- Which solution should be applied, it can be decided by the priority order. If the NCSG has higher priority than MG, then cancel the MG; Otherwise, neither of them would be canceled but at the price of NCSG degradation to MG.

- Reference

- [1] RP-221018 “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation.

- [2] RP-222337 “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation

- [3] R4-2214346 “WF on further enhancements on measurement gaps and measurements without gaps”, MediaTek Inc