3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216504

Electronic Meeting, October 10 – October 19, 2022

Source: 	Ericsson

Title:  	Measurement procedure requirements for NTN

Agenda Item:	4.2.5.1

Document for:	Discussion

# Introduction

In RAN4# 104-e a WF on NTN RRM measurement requirements was approved [1]. In this contribution we develop topics on measurement requirements which were agreed to continue study in RRM measurement part in the WF.

# Discussion

## Fully Overlapping Concurrent MGs

In last meeting, the following agreements are achieved:

### Issue 4. Fully Overlapping Concurrent MGs Agreement: - Option 1: Do not define requirements for fully overlapping concurrent MGs - Option 2: For fully overlapped case, gap sharing rule is applied during the collided gap occasions, and the scaling factor is 2 - Option 2A: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms. - A MG with the lowest ID, i.e. 0, gets priority over the other, and the dropping rule starts from SFN=0, i.e. MG-ID#0 is selected and MG-ID#1 is dropped at the first collision instance after SFN=0, and it alternates afterwards. - [RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only.] - Option 2B: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms. - RAN4 introduce a new UE capability supporting “fully overlapping concurrent MGs” which is limited to NTN-only. - Option 2C: - It is applicable only to the case where both of the concurrent MGs have the longest MGRP, i.e. 160ms.

On top of alternatives, we support option 2 among option 1, 2. Furthermore, if picking 2B and 2C, sharing rule is applied literally, however it poses one critical issue since the vague sharing rule limits scheduling on the resources occupied by MG since no common understanding between network node and UE which MG shall be measured and which MG isn’t measured.

We believe that of the three accessible sub-options, Option 2A is the most appropriate for reducing overall throughput.

Proposal 1: We can support Option 2A, and we’re open to other methods which can avoid scheduling resources on collided gap.

Proposal 2: As per UE capability supporting ‘fully overlapping concurrent MGs’, we have concerns on the usage. We suggest postponing the proposal until practical demand for the capability is available.

## Collision between SMTC and MG

In last meeting, SMTC collision condition was discussed, and agreements are cited in below:

Agreement: - For the case where one SMTC is inside MG and the other SMTC is outside the MG, if the proximity distance between the MG and SMTC outside the MG is smaller than or equal to the proximity distance threshold, i.e. 4ms, the two SMTCs are considered as colliding SMTCs.

The collision or more accurately, proximity condition between SMTC and MG triggers a particular issue in NTN.

Here we use one example to demonstrate the issue. Firstly, assuming a configuration: 1 intra-frequency SMTC and 2MGs (i.e. 2 MG indexes) and MG periodicity is double of intra-frequency SMTC in this case, there are two cases with different proximity conditions.

It shall be noted that we only consider proximity condition. For the case that SMTC already in MG in time domain, it has been covered by current specification.

In case A, illustrated in Figure 1, each SMTC occasion meets proximity condition with MG occasions from 2 MGPs, it results that no intra-frequency SMTC doesn’t meet proximity condition with MG occasions.

Figure 1 One example of proximity condition between intra-frequency SMTC and MGs

In case B, illustrated in Figure 2, some but not all SMTC occasions meet proximity condition with MG occasions from 2 MGPs.

Figure 2 Another example of proximity condition between intra-frequency SMTC and MGs

There are alternative options to deal with the issue on how to measure the SMTC regarding aforementioned two cases. Firstly, we list two possible options:

- Option 1: Scaling factor Kp to drop SMTC upon collision

The method can refer to current Kp definition for intra-frequency measurements requirements with concurrentMeasGap-r17 support in below:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps, Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.2B.3. Kp = 1 when Navailable = 0.

Generally, we can reuse main part of the cited texts besides of two corrections:

- Definition of ‘window’ shall be updated, otherwise there will be various results of counting Navailable if following current window duration and starting point. Also shown in Figure 2, Navailable in the two windows: W1 and W2 is different. The updated window length shall cover (max(SMTC period, MGRP_max)+ 2 proximity conditions(e.g. 4+4 ms)) and window shall start from (any SMTC occasion - proximity condition) to monitor all collided SMTC occasions.

- Define ‘proximity condition’.

Figure 2 Window definition

- Option 2.1: Sharing rule between SMTC and MG, only defining sharing ratio, e.g. 50% SMTC occasions and 50% MG occasions occurring during intra-frequency.

- Option 2.2: Sharing rule, but follow solution in Option 2A in Issue ‘Fully Overlapping Concurrent MGs’. Since there are two MGPs, some minor updates are needed but still follow the basic rationale. Figure 2 shows two possible skipping methods.

  - One rule can be like: Starting SFN#0, if the SMTC occasion meets proximity condition with one MG occasion for the first MGP in window, the MG occasion shall be dropped, and it alternates afterwards; if the SMTC occasion meets proximity condition with one MG occasion for the second MGP in window, the MG occasion shall be dropped, and it alternates afterwards.

  - Another rule can be like: Starting SFN#0, if the SMTC occasion meets proximity condition with one MG occasion for the first MGP in window, the SMTC occasion shall be dropped, and it alternates afterwards; if the SMTC occasion meets proximity condition with one MG occasion for the second MGP in window, the MG occasion shall be dropped, and it alternates afterwards.

  - Or between above two rules.

Figure 3 Dropping rules

Either Option 1 or Option 2 has Pros and Cons, then we suggest adopting both of them but apply to different cases. The main rationale is to keep minimal impact to current specification, meanwhile solve the most critical problem.

Wherein, if no SMTC occasion doesn’t meet proximity condition with MG, it means union of SMTC occasion and MG occasion occupies a rather great portion in time domain, it may severely abbreviate resources for scheduling data, therefore in this case, the motivation of Option 2 is it can mitigate overall throughput.

On the contrary, if there are some SMTC occasions don’t meet proximity condition with MG, we can assume that intra-frequency measurement still can work only with these SMTC occasions, in other words, MG shall be measured upon meeting proximity condition.

Proposal 3: For collision between SMTC and MG:

- If UE is configured with 2 MGPs all the SMTC and MG occasions collide with each other for each of the configured MGPs, the intra-frequency measurement shall apply sharing rule: only defining sharing ratio or explicitly indication of dropping.

- Otherwise, the intra-frequency measurement shall use scaling factor (update from Kp concept) to drop SMTC occasions colliding with MG occasions.

# Summary

Proposal 1: We can support Option 2A, and we’re open to other methods which can avoid scheduling resources on collided gap.

Proposal 2: As per UE capability supporting ‘fully overlapping concurrent MGs’, we have concerns on the usage. We suggest postponing the proposal until practical demand for the capability is available.

Proposal 3:  For collision between SMTC and MG:

- If UE is configured with 2 MGPs all the SMTC and MG occasions collide with each other for each of the configured MGPs, the intra-frequency measurement shall apply sharing rule: only defining sharing ratio or explicitly indication of dropping.

- Otherwise, the intra-frequency measurement shall use scaling factor (update from Kp concept) to drop SMTC occasions colliding with MG occasions.

# References

- R4-2214471	WF on RRM requirements for NTN measurement and mobility	Qualcomm