3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216530

E-meeting,10th Oct – 19th Oct,2022

Agenda item:	5.27.3

Source: 	ZTE Corporation

Title: 	Further discussion on BS RF requirements for APT600MHz

Document for:	Approval

Introduction

The topic of APT 600MHz have already been extensively discussed during R17 time phase. At that time, it was a SID, which mainly focus on feasibility analyses for options B1, B2 and B2a, where the feasibility studies and conclusions can be found in the TR38.860[1]. During last RAN4 meeting, we have reached good progress on BS RF requirements. In this contribution, we want to share some minor updates on top of agreement reached during last RAN4 meeting.

- Specify a new NR FDD operating band to include band numbering and core requirements to operate in APT region by considering that:

  - UE transmit: 663 - 703 MHz, BS transmit: 612 MHz – 652 MHz,

  - To support the following channel bandwidths and SCS as in the Table 2 below

  - UL channel bandwidth is limited to 20 MHz to limit the MSD as was the case with n71

  - The use of UE power class 3 (23dBm).

Table 2: Channel bandwidth and SCS parameters

| SCS (kHz) | channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 5 | 10 | 15 | 20 | 25 | 30 | 35 |  |
| 30 |  | 10 | 15 | 20 | 25 | 30 | 35 |  |

- Develop a technical specification for the APT 600 MHz band for option B1 (as defined in table 1) for the APT region 600 MHz band with a view to developing the normative text on:

- Operating band, channel bandwidth and system parameters- see Table 1 and Table 2

- BS and UE RF core requirement taking into account potential coexistence issues

- RRM requirement

Discussion

##### 6.6.5.2.3	Additional spurious emissions requirements

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band 71 or NR Band n71 | 617 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz |
|  | 663 – 698 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz, since it is already covered by the requirement in clause 6.6.5.2.2. |
| APT 600 MHz | 612 – 652 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz. |
|  | 663 – 703 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n71 or APT 600 MHz, since it is already covered by the requirement in clause 6.6.5.2.2. |

Conclusions

In this contribution, we want to share some further views on BS RF requirements.and proposals are made as following:

Proposal 1: to adopt the changes for additional spurious emission requirement as proposed.

References

- TR38.860, Study on Extended 600 MHz NR band, v17.0.0

- RP-221778, New WID on APT 600 MHz NR band, Spark NZ, Qualcomm Incorporated

- TR36.775, US 600 MHz Band for LTE, v15.0.0

- R4-2214434,WF on BS requirements for APT 600 MHz, Nokia, Approved

- R4-2214235, Email Discussion Summary for [104-e][124] NR_600MHz_APT, Approved.

- R4-2215090,draft CR to TS38.104 the introduction of APT600MHz, ZTE, Approved.