3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216582

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on requirements for concurrent measurement gaps, pre-configured gaps and NCSG

Agenda item:	6.10.2

Document for:	Discussion

Introduction

This paper presents Nokia’s view on RRM aspects related to further enhancements of measurement gaps for Rel-18. In particular, it presents an overview of the corresponding objective of the work item. Based on that, observations and proposals are derived regarding combinations of measurement gap types.

Discussion

The last RAN4 meeting #104 was the first meeting where MG_enh2 was discussed for Rel 18. As part of this discussion many issues were discussed with open points for discussion.

The first open issue we would like to discuss is the prioritization among NR SA and MR-DC, which is shown below [1]:

- Issue 2-3: NR SA and MR-DC < Way forward >: - FFS whether to deprioritize MR-DC in Rel-18

When concurrent operation is concerned, the main problem to be solved in RAN4 is how concurrent, NCSG and preconfigured gaps are combined in the same framework. Given that, we understand that the main focus should be on NR SA for the combination of concurrent gaps with preconfigured gaps or NCSG.

RAN4 to focus on requirements for the combination of Pre-MG, concurrent gaps and NCSG on SA scenarios and if time allows develop requirements for MR-DC.

From last RAN4 meeting [1]:

- Issue 2-9: Which Rel-17 Pre-MG principle(s) should be revisited < Way forward >: - FFS: For UE autonomous mechanism, only the measurements associated to the concerned pre-MG are used for the rule checking - FFS: For Network-controlled mechanism, only the bits corresponding to the concerned pre-MG are used for determining the status - FFS whether to consider the other potential enhancement

When Pre-MG principles are concerned, there were proposals in the last meeting regarding further mechanisms for activation and deactivation of Pre-MGs. Concerning that topic, this work item should focus mostly on how to enable gap priority for Pre-MG such that Pre-MGs can be used together with network-controlled concurrent measurement gaps. Therefore, we propose that the Pre-MG principles for network-controlled mechanism are kept in Rel-18.

Rel-17 Pre-MG principles for configuration and activation/deactivation should be maintained for network-controlled mechanism.

On the other hand, when autonomous Pre-MG activation/deactivation is considered, there might be unclear behavior when such Pre-MG instance is colliding with network-controlled concurrent MG instance. The UE autonomous mechanism thus needs to be modified to handle such collisions.

Further consideration is needed for UE autonomous-Pre-MG activation when a Pre-MG instance collides with a network-controlled concurrent MG instance.

From the last RAN4 meeting [1]:

- Issue 2-10: Which Rel-17 concurrent gap principle(s) should be revisited < Way forward >: - FFS which Rel-17 concurrent gap principle(s) should be revisited

Examples of principles are:

- Proximity

- Collision handling

- Priority

- Overhead

All those principles should be kept unchanged unless it is clearly identified that they need to be revised.

Rel-17 concurrent gaps principles should not be changed for Rel-18 MG_enh2.

From the last RAN4 meeting [1]:

- Sub-topic 2-3: Max number of supported gaps - Issue 2-11: Max # of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs) < Agreement >: - For the max number of gaps for Case 1 (Pre-configured MG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline. < Way forward >: - FFS whether to increase the max number - FFS whether to introduce new UE capability for # of supported Pre-MG - Issue 2-12: Max # of gaps for Case 2 (NCSG and multiple concurrent MGs) < Agreement >: - For the max number of gaps for Case 2 (NCSG and multiple concurrent MGs), the Rel-17 conclusions will be taken as the baseline. < Way forward >: - FFS whether to increase the max number - FFS whether to introduce new UE capability for # of supported NCSG

We prefer to keep the maximum number of supported gaps same as in Rel-17. Here it is also needed to clarify that only the activated pre-configured gaps should be taken into account in addition to concurrent gaps when the maximum number of gaps is evaluated.

Keep the maximum number of gaps as in Table 9.1.8-1 of 38.133, for concurrent operation of concurrent gaps with Pre-MG/NCSG, considering that only activated Pre-MG gaps are counted.

From the last RAN4 meeting [1]:

- Issue 2-13: Detail combinations < Way forward >: - Companies are encouraged to bring high-level principle about the possible gap combinations before going into details

The detail combinations were discussed during the last RAN4 meeting, however there were new agreements at the RAN #97 plenary clarifying the scope of the work for case 1 and case 2. The plenary decision was captured in the revised WID text, with objectives described below:

- Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in Case 1 and Case 2 - Note 2: The requirement discussions on the scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 will be started after RAN#99. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG, NTN gaps and NCSG can be discussed after RAN#99 - Note 4: This WID does not include any inter-working with MUSIM gaps

As part of this decision, the scope of Case 1 and Case 2 was further delimited, and other scenarios are to be considered after RAN#99.

RAN #97 plenary discussion made clear that focus is on Case 1 and Case 2 initially.

Follow scope of new WID and focus on Case 1 and Case 2 initially.

From the last RAN4 meeting [1]:

- Sub-topic 2-4: Collision definition and handling - Issue 2-14: Required changes for Pre-MG on collision < Agreement >: - For Case 1 (Pre-configured MG and multiple concurrent MGs), the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated. < Way forward >: - FFS whether Pre-MG priority can be further decided by the associated Mos being measured or other signalling parameters - Issue 2-15: Required changes for NCSG on collision < Agreement >: - When determining if a collision occurs between a NCSG instance and another gap instance, the baseline requirement considers the total NCSG duration, including both the VILs and the ML. < Way forward >: - FFS other potential enhancements regarding spare RF chains, interruption due to collision on RRT, revised proximity condition, … etc. - Issue 2-16: general gap collision handling < Agreement >: - On gap collision handling, take priority rule and overlapping rules from Rel 17 concurrent gaps as the baseline < Way forward >: - FFS whether to consider gap sharing rule

As discussed in some of our previous proposals, the principles used for concurrent gaps in Rel17 should be kept unchanged, and the scope of the work should be on enabling Pre-MG and NCSG to be used in the concurrent gaps framework. As a result, we can reuse the gap priority that is already specified for the concurrent gaps in Rel-17, for Pre-MG and NCSG, such that the RAN4 requirements are specified considering the priorities. Pre-MG and NCSG can already be configured with a priority considering Rel-17 ASN.1.

Reuse gapPriority-r17 from GapConfig-r17 for priority definition of Pre-MG and NCSG.

Reuse collision handling rules from Rel-17 concurrent gaps for activated Pre-MG and NCSG.

Conclusion

This paper has presented Nokia’s views on Rel-18 measurement gaps enhancements. As part of this discussion, we have reached the following observations and make following proposals:

- RAN4 to focus on requirements for the combination of Pre-MG, concurrent gaps and NCSG on SA scenarios and if time allows develop requirements for MR-DC.

Rel-17 Pre-MG principles for configuration and activation/deactivation should be maintained for network-controlled mechanism.

Further consideration is needed for UE autonomous-Pre-MG activation when a Pre-MG instance collides with a network-controlled concurrent MG instance.

Rel-17 concurrent gaps principles should not be changed for Rel-18 MG_enh2.

Keep the maximum number of gaps as in Table 9.1.8-1 of 38.133, for concurrent operation of concurrent gaps with Pre-MG/NCSG, considering that only activated Pre-MG gaps are counted.

- RAN #97 plenary discussion made clear that focus is on Case 1 and Case 2 initially.

Follow scope of new WID and focus on Case 1 and Case 2 initially.

Pre-MG and NCSG can already be configured with a priority considering Rel-17 ASN.1.

Reuse gapPriority-r17 from GapConfig-r17 for priority definition of Pre-MG and NCSG.

Reuse collision handling rules from Rel-17 concurrent gaps for activated Pre-MG and NCSG.

- References

- R4-2214346, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.