3GPP TSG-RAN WG4 Meeting # 104-bis-e                	R4-2216707

Electronic Meeting, 10-19 Oct, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	CSI simulation results for ext71GHz

Agenda item: 	4.3.7.2.4

Document for: 	Discussion

Introduction

This paper presents CSI simulation results for the definition of demodulation requirements related to the operation between 52.6 GHz and 71 GHz.

Simulation Assumptions

Table 1 Results for CQI PN Set 1

| Assumptions | Value |
| --- | --- |
| CBW/SCS | 100MHz / 120kHz |
| UE PN Model | TR38.808 PN model Set 1 and Set 2 |
| PN Compensation | CPE only |
| NoF PRBs | 66 |

CQI Simulation Results

Table 2 Results for CQI No PN

| SNR | Median CQI | Prob (Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100 | 0 | 0 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -2 | 2 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 2 | 4 | 100 | 0 | 0 | 100 |
| 4 | 5 | 100 | 0 | 0 | 100 |
| 6 | 6 | 99.6 | 0 | 0 | 100 |
| 8 | 7 | 100 | 0 | 0 | 100 |
| 10 | 8 | 100 | 0 | 0 | 100 |
| 12 | 9 | 100 | 0 | 0.1 | 100 |
| 14 | 10 | 100 | 0 | 0.1 | 100 |

Table 3 Results for CQI PN Set 1

| SNR | Median CQI | Prob (Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100 | 0 | 0 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -2 | 2 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 2 | 4 | 100 | 0 | 0 | 100 |
| 4 | 5 | 100 | 0 | 0 | 100 |
| 6 | 6 | 100 | 0 | 0 | 100 |
| 8 | 7 | 100 | 0 | 0 | 100 |
| 10 | 8 | 95.1 | 0 | 0 | 100 |
| 12 | 9 | 67.3 | 0 | 0 | 100 |
| 14 | 9 | 85.6 | 0 | 0 | 0.7 |

Table 4 Results for CQI PN Set 2

| SNR | Median CQI | Prob (Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100 | 0 | 0 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -2 | 2 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 2 | 4 | 100 | 0 | 0 | 100 |
| 4 | 5 | 100 | 0 | 0 | 100 |
| 6 | 6 | 98.4 | 0 | 0 | 100 |
| 8 | 7 | 93.2 | 0 | 0 | 100 |
| 10 | 8 | 58.8 | 0 | 0 | 100 |
| 12 | 8 | 72.1 | 0 | 0 | 0 |
| 14 | 9 | 73.6 | 0 | 0 | 0 |

References

- R4-2214390 - WF on FR2-2 UE demodulation requirements