3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216769

Electronic meeting, October 10th – 19th 2022

Agenda Item:	6.13.5

Source:	Ericsson

Title:	Discussions on ATG RRM requirements

Document for:	Discussion, Decision

# 1	Introduction

During RAN4#104-e, RAN4 had initial discussions to identify the RRM impact of introducing requirements for ATG. Some high-level agreements related to scenario, specification impact, assistance information were reached. In addition, technical proposals related to various type of RRM requirements in RRC IDLE,  INACTIVE and CONNECTEDs mode were discussed and outcome of the discussions were summarized in a way forward documents in [1]. In this contribution, we discuss and provide our view on those proposals.

# 2	Discussion

## 2.1	General

Issue 1-2-1: Impaction on TS38.133 Section 3: Definitions, symbols and abbreviations - ATG terminologies need to be introduced. - FFS on ATG bands table based on input from RF session.

It is clear that the new ATG bands introduced by RF group needs to be included in the frequency band grouping section of TS 38.133. RAN4 can simply postpone the discussions until sufficient progress has been reached in RF group.

RAN4 to include the ATG bands (agreed by RF group) into frequency band grouping section 3.5 in TS 38.133.

Different types of assistance information were discussed in the last meeting and agreements were captured as follows [1]:

Issue 1-2-3: UE assistance information - RAN4 further study ATG UE assistance information - such as altitude, location, propagation delay difference, flight path, speed, velocity etc., or change in any of these parameters.

It shall be clarified that this assistance information can be two ways:

- detected by the UE and informed to the ground base station

- detected by the ground base station and informed to the UE

In 1) our intention with UE detecting changes in its operation mode (such as flight path, speed, altitude) is to allow the UE to not perform the neighbour cell measurements or to allow the UE to meet the relaxed neighbour cell measurement requirements under stable flight operating conditions. It is also noted that the typical cell size considered in ATG scenario is very large compared to the cell sizes of the typical terrestrial MBB network and thus not expected to perform frequent cell changes, hence the UE can be allowed to measure only on the serving cell until any unpredictable change in its flight operation mode is detected in the UE. We also believe informing the ground base station (NW) about unexpected change detected by the UE can be very useful as NW can use it for adapting its procedures or parameters, or even informing other nodes (e.g., third party nodes) as it concerns flight safety.

In 2) our intention is that critical information related to the flight operation mode can also be provided to the UE by the ground base station which it may have obtained from other nodes. We believe it would be useful to have mechanism for the NW to signal such information upon which UE may revert to normal operation which in this case would mean that UE reverting to normal measurement mode, measuring on all detected neighbour cells on all carriers etc.

About how to determine or define unpredictable change in flight operation mode, we understand that this might be challenging question as change in multiple parameters can be considered as critical or unpredictable. Therefore, we are open to not consider only one type of change to define unpredictable change, but we are open to consider unpredictable change based on at least: changes in altitude, flight path, and speed. Also one needs to agree on how much it should change in order to consider it as unpredictable or critical change in flight operation mode. Our view is that the work can be done stepwise, firstly RAN4 needs to identify what types of parameters to consider and second step is to identify the amount of change (which can even be discussed as part of performance part). In this meeting, our view is that RAN4 shall discuss and agree on what type of parameters shall be considered in detecting changes in unpredictable or critical changes in flight operation mode.

Assistance information related to unpredictable changes in flight operation mode can be two ways: 1) UE detecting and informing the NW, 2) NW informing the UE

Assistance information comprises at least changes in the UE altitude, flight path and speed, remaining service time for serving cell and side conditions can be FFS.

- Whether all the assistance information listed in proposal 3 is needed when the assistance information is detected and signalled by the NW to UE can be further discussed. Our view is that, the assistance information from NW to UE can contain less information compared to the assistance information provided by the UE to NW.

- As we discussed in last meeting, one of the key differences for ATG network with traditional network is larger coverage of serving cell for ATG BS which implies UE will stay in the serving cell for a long time (e.g. 10-20 minutes). Some normal mobility procedures are unnecessary when UE is staying in cell center. RAN4 can further discuss the technical impact to RRM requirement due to the large serving cell radius, such as RRM mobility procedure, measurement procedure etc.

RAN4 to discuss the technical impact to RRM requirement due to the large serving cell radius in ATG system, such as IDLE mode mobility, CONNECTED mode mobility, and measurement procedure etc.

## 2.2	IDLE mode Mobility

Measurement capability

The scenario is under discussion in RAN4 RF group and important characteristics of the scenario such as ISD, cell radius needs to be agreed before measurement capability can be agreed for the RRM requirements.

Discussions on measurement capability shall be postponed until the scenario is agreed in RF group.

Neighbour cell measurements

Based on the motivations provided in section 2.1, it may not be necessary for the ATG UE to perform periodic neighbour cell measurements in RRC IDLE and INACTIVE mode. Under stable operating conditions, which can be defined by UE not detecting any abnormal changes in flight operation based on the assistance information discussed in section 2.1 and based on the coverage information of the serving cell (e.g. serving cell RSRP level), it may be sufficient for the UE to measure on the serving cell since the typical cell size considered for ATG scenario is very large compared to the cell sizes of the typical terrestrial MBB network. This reduces the unnecessary UE processing and complexity introduced by frequent neighbour cell measurements. For example, it may take several hundreds of seconds to minutes until a cell change is needed depending on the cell radius, square km per aircraft etc [xx] and the UE is therefore not expected to perform frequent cell changes. However, the UE should resume measurements on the neighbour cells upon detecting any unpredictable changes in flight operation which can be based on assistance information detected by the UE or received from the NW.

The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above a certain threshold which is FFS.

For cell reselection, the A2G UE should resume the neighbor cell measurement in normal manner without any relaxation based on assistance information related to unpredictable changes in flight operation mode.

Small data transmission (SDT)

As discussed in section 2.1, the UE can detect unpredictable changes in flight operation mode (such as flight path, sudden drop in height, etc. ) and inform the NW. This type of information concerns flight safety and is therefore critical for the UE to communicate the NW quickly. Small data transmissions (SDT), which was introduced in Rel-17 NR, allows the UE to transmit small uplink data to the NW while in RRC INACTIVE state without switching to RRC CONNECTED mode, can be used for this purpose.  Therefore, two types of SDT: RA-SDT and CG-SDT and both can be considered for ATG. Since the ATG UE is capable of applying pre-compensation for the timing difference, TA validation based on RSRP changes may not be needed for ATG. Our view is that most of the previous Rel-17 SDT requirements can be reused, and impact is minimal while it may have significant benefit for the ATG.

SDT is supported for A2G in Rel-18, and SDT requirements for ATG is FSS.

## 2.3	CONNECTED mode Mobility

CHO HO

It remains open whether conditional handover shall be supported for ATG as follows [1]:

- FFS which kind of CHO will be introduced - FFS whether ATG specific CHO requirements are needed.

Our view is that conditional handover can be useful for ATG scenario, where the UE can be configured to trigger a handover based on certain conditions configured by the NW. Especially given that the ATG cells are large, operated at high altitude and high speed and on a dedicate network, it may be beneficial to discuss whether the existing handover procedures are suitable for ATG. For example, since propagation channels models when operating at higher altitude can be different compared to the channel models assumed in the legacy TN network, it maybe beneficial for the NW to identify and configure the conditions for handover. Details of the requirements can be further discussed after RAN4 reaches a conclusion on whether to support CHO.

CHO is supported for A2G in Rel-18, and CHO requirements for ATG is FSS.

RRC Re-establishment

At last meeting it was agreed to reuse the principle of defining RRC re-establishment delay requirements from the legacy NR and to identify potential ATG specific impact [1].

The RRC re-establishment requirements are defined in terms of maximum time within which the UE shall be capable of sending RRCReestablishmentRequest message upon UE detecting a loss in RRC connection. The maximum time is defined as follows [2]:

Tre-establish_delay=TUE_re-establish_delay+TUL_grant

TUL_grant is the time required to acquire and process uplink grant from the target PCell and TUE_re-establish_delay comprises multiple delay components as shown below [2]:

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

where

- Tidentify_intra_NR: the  time required to identify the target intra-frequency NR cell

- Tidentify_inter_NR,i: the time required to identify the target inter-frequency NR cell

- TSI-NR: It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target NR cell.

- TPRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms.

- Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment;

In the formula above, it is expected that Tidentify_intra_NR, Tidentify_inter_NR,I and TSI-NR are impacted due to due to A2G operation. The delay components related to Tidentify parts (intra- and inter-frequency) will certainly depend on whether the cell search is going to be impacted due to significantly larger cell coverage and higher speed of an aircraft. Therefore, evaluations are needed to study the impact on the cell search performance under ATG scenario.

The delays are defined differently for known and unknown cell and the current definitions of known and unknown cell can be reused for ATG. Regarding the impact on TSI-NR, the SI acquisition time is not explicitly defined in the core requirements, but they are defined in the test cases only. Therefore, no impact is foreseen.

The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify_intra_NR and Tidentify_inter_NR.

Current definitions of known/unknown cells and side conditions in the RRC re-establishment requirements can be reused for ATG.

RAN4 to study the larger cell impact to RRC re-establishment, such as the target cell list and total delay requirement.

Random access

Following agreements was reached at last meeting RA discussions [1]:

- Reuse the principle from the legacy random access requirements as baseline for ATG - Further discuss the ATG specific impaction and details - Further discuss whether to define requirements for 2-step RA for A2G, and whether ATG specific impaction should be involved.

The physical design and procedures of random access remain unchanged for ATG. The random access is specified in TS 38.213 and the control of the RACH transmission is specified in TS 38.321 which are both unaffected. Therefore, no ATG specific impact on the RA requirements are expected and the existing requirements can e reused.

A legacy NR UE can select between performing 2-step RA and 4-step RA based on measured RSRP level of the cell. Typically, the 2-step RA is used for small cells and may therefore not be suitable for ATG scenario where the cells are significantly larger.

RAN4 to define RA requirements only for 4-step RA for ATG scenario.

Existing 4-step RA requirements are reused for ATG.

RRC Connection Release with Redirection

Following agreements was reached at last meeting RA discussions [1]:

- Reuse the principle from the legacy RRC Connection Release with Redirection requirements as baseline for ATG - Further discuss whether the ATG specific RRC Connection Release with Redirection requirement is needed

Current specification contains requirements for RRC connection release with redirection to NR and E-UTRAN cells. At last meeting it was agreed that inter-RAT measurements are not applicable for Rel-18 ATG. Thus, RAN4 shall focus on RRC connection release with redirection to NR cell only.

After receiving the last slot containing the RRCRelease message, the UE shall transmit the random access on the target cell within Tconnection_release_redirect_NR for NR cell where [2]:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

TRRC_procedure_delay should not impacted since no ATG specific changes were introduced for RRC procedure delay by RAN2. Similar to the earlier discussions on RRC re-establishment, Tidentify-NR and Tidentify-E-UTRA might be impacted due to significanly larger cell coverage and higher UE speed for ATG scenario. RAN4 needs to evaluate the impact on the cell cell detection performance for under UE speed corresponding to ATG scenario and the outcome of that study can be used to decide whether the Tidentify-NR should be shortened/extended with respect to the legacy requirements. TRACH  is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell and shall not be impacted since the physical design of RACH is same as in legacy NR.

The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify-NR.

Current definitions/requirements on TRACH are reused for ATG.

RAN4 to study the larger cell impact to RRC connection release with redirection, such as the target candidate cell and total delay requirement.

## 2.4	Signalling characteristics

Radio link monitoring

Rel-15 radio link monitoring requirements assume 10 RLM-RS samples for SSB based out-of-sync evaluation period and 5 RLM-RS samples for SSB based in-sync evaluation period. Similarly, 20 RLM-RS samples for CSI-RS based out-of-sync evaluation period and 10 RLM samples for CSI-RS based in-synch are assumed in the evaluation period, where it is assumed CSI-RS is configured with density 3 and 24 RBs. The ATG UE is expected to have at least same capability as a legacy NR UE (e.g. Rel-15 NR UE) in terms of number of receive antenna, UE processing etc. Therefore, it is reasonable to assume that the ATG UE can meet the channel quality accuracy defined for Rel-15 RLM.

However, it is noteworthy that the Rel-15 RLM evaluations were performed under channel models which may be different than the channel models assumed for ATG. More specifically, the Rel-15 RLM performance were studied for TDLA30, TDLB100 and TDLC300 channel models. If RAN4 agrees to use different channel models for evaluating the performance then the RLM requirements need to be revisited. Therefore whether the existing NR RLM requirements can be reused for ATG depends on the type of channel model agreed for evaluating the ATG performance which is under RF discussion.

Regarding the evaluation period, it is noteworthy that the RLM evaluation period for HST is same as Rel-15 RLM evaluation period for FR1. This means the number of samples assumed in the RLM in-sync and out-of-sync evaluations are same. Following same approach, our view is that the legacy (Rel-15) RLM in-sync and out-of-sync evaluations can be reused for ATG. However, we are also fine to evaluate the RLM performance under the UE speed corresponding to the ATG scenario.

Discussions on whether the legacy RLM requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

Beam failure detection evaluation period

Rel-15 beam failure detection requirements assume the same number of SSB/CSI-RS samples as RLM In-sync to define BFD evaluation period, that is, 5 samples for SSB based BFD and 10 samples for CSI-RS based BFD (density=3, 24RBs). Since the ATG UE is expected to have at least same capability as a legacy NR UE (e.g. Rel-15 NR UE) in terms of number of receive antenna, UE processing etc., it is reasonable to assume that the ATG UE can meet BFD based on the same number of samples.

Discussions on whether the legacy BFD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

Beam failure detection evaluation period

After UE detects a beam failure, the UE is required to find the candidate beam whose L1-RSRP exceeds the configured threshold. Rel-15 candidate beam detection and L1-RSRP measurements assume 3 samples for both SSB based and CSI-RS based L1-RSRP measurements. Note the evaluation period is set based on 1 sample when gNB configures the timeRestrictionForChannelMeasurement according to TS38.214. For the reasons explained under BFD section above, it is reasonable to assume same number of samples for the CBD.

Discussions on whether the legacy CBD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

Discussions on whether the legacy L1-RSRP measurement requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

The accuracy level of L1-RSRP belongs to the performance part of the WI and thus the discussions can be postponed.

Active BWP switching requirements

The active BWP switching requirements comprises of the following various types of requirements:

- Active BWP switching delay defined in section 8.6 in TS 38.133

- Scheduling restriction during the active BWP switching delay defined in section 8.6 in TS 38.133

- Interruption due to active BWP switching on other NR serving cells in section 8.2, TS 38.133 (for EN-DC, SA and NE-DC).

- Interruption due to active BWP switching on LTE serving cells in sections 7.32.2.7 and 7.36.2.6, TS 36.133 (for EN-DC and NE-DC respectively).

For ATG it was agreed at last meeting that CA/DC and inter-RAT related requirements do apply in Rel-18. Therefore the simultaneous and non-simultaneous BWP switching on multiple CCs requirements that were introduced in Rel-16 are not relevant for ATG. Also, the interruption part of the active BWP switching requirements are not relevant for ATG.

For the BWP switching delay and scheduling restriction parts, no ATG specific impact is expected due to the higher speed. It can also be noted that BWP switching delay requirements for HST is same as for legacy NR.

No need to define interruptions due to active BWP in section 8.2 of TS 38.133 for NR serving cells for EN-DC and NE-DC.

Existing active BWP switching requirements (excluding simultaneous and non-simultaneous BWP switching delay on multiple CCs) defined are reused for ATG.

Active TCI state switching delay requirements and active spatial relation switching delay requirements

The RRM impact on TCI state switching requirements are defined in section 8.10 in TS 38.133, and there are three types of TCI state switching delay: MAC-CE based, DCI-based and RRC based TCI state switching delays. Most of the delay components in the TCI state switching delay requirements in all three cases are not affected by the ATG except TL1-RSRP. TL1-RSRP is used in RRC-based and MAC-CE based requirements.

The MAC-CE based TCI state switching delay requirements for the case when target TCI state is unknown is defined as:

n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length

Similarly, the RRC-based TCI state switching delay requirements for the case when target TCI state is unknown is defined as:

n+ (TRRC_processing  +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length

where T L1-RSRP is 0 for FR1. Since only FR1 is considered in Rel-18 ATG, no impact in expected on the existing TCI state switching requirements. Therefore, we make following proposal:

Existing TCI state switching requirements are reused for ATG.

For the similar reasons, the current active spatial relation switching delay requirements can also be reused for ATG.

Existing uplink spatial relation switching delay requirements are reused for ATG.

UE-specific CBW change

The UE-specific CBW change is triggered by the UE upon receiving the RRC reconfiguration involving offsetToCarrier or carrierBandwidth change on the old CBW.

The UE-specific CBW change related requirements comprises of the following types of requirements:

- UE-specific CBW change delay defined in section 8.13.

- Scheduling restriction during the UE-specific CBW change delay defined in section 8.13.

The existing procedure is used for performing the UE-specific CBW change. The UE-specific CBW change delay requirements are fundamentally derived from the RRC based active BWP switching delay. Therefore, the existing UE-specific CBW change delay and the scheduling restriction requirements defined in section 8.13 can apply also for redcap UE.

Existing requirements on UE-specific CBW change are reused for ATG.

## 2.5 Measurements

Pre-MG

In Rel-17, Pre-MG is introduced for dynamically switching between intra-frequency measurement with gap and intra-frequency without gap. We should also notice that one of the KPIs for ATG network is the throughput. If the MG can be disabled, it will help to improve the total system throughput. Thus, from our understanding, the Pre-MG can have benefits than legacy MG and will be used for ATG network.

Pre-MG can be used in ATG network to improve the total system performance.

RAN4 to investigate conditions under which Pre-MG can be disabled/enabled.

Intra-frequency measurement

According to previous RAN4 agreements, FR2 related requirements are not going to be defined in Rel-18 ATG WI. Therefore only FR1 MG shall be discussed in Rel-18 ATG. The intra-frequency measurement requirement may need to be updated if only single carrier is supported in ATG system, such as no deactivated SCell measurement, no SCCs, PSCell measurement. CSSF may also need to be updated to support single serving cell. The RedCap measurement requirement can be a good start point.

Only FR1 MG will be used in Rel-18 ATG network.

The intra-frequency measurement requirement needs to be updated, such as no deactivated SCell measurement, no SCCs, PSCell measurement.

CSSF outside gap needs to be updated.

CSI-RS measurement is unnecessary in ATG network.

Inter-frequency measurement

Inter-frequency measurement is mainly used for handover procedure. However, UE may stay in serving cell for a long time (several minutes) in ATG network. Furthermore, the possible target cells are determined to network planning and dedicated flight path which can be known to UE in advance. Thus, inter-frequency measurement with MG may be unnecessary for a long time which means the MG can be disabled. As we mentioned before, one important KPI for ATG system is the system throughput. RAN4 can further study the trade-off between Inter-frequency measurement within MG and the throughput due to large cell coverage.

RAN4 to study the trade-off between Inter-frequency measurement within MG and the data throughput due to large cell coverage.

## 2.6 	TDD and NTN as base for ATG

In RAN4 #104 we have the following agreements from RRM WF [1]:

... 1. General Issue 1-2-2: TDD impaction - Option 1: FFS, RAN4 needs to study impact on TDD band operation due to longer propagation delay between ground gNB and ATG UE. … 3 Timing and frequency adjustment 3.1 General issues Issue 3-1-1: Whether ATG UE should be capable of GNSS measurement - ATG UE should be capable of GNSS measurement Issue 3-3-4: Cell phase synchronization accuracy - Cell phase synchronization accuracy will be defined for ATG, the legacy TN requirement can be the baseline - FFS whether to tighten the requirements or not ...

and in the main session we have [3]:

## ... ## Issue 3-1-2: GNSS assumption It is proposed that ATG UE in R18 is GNSS capable. ## Issue 3-1-2a: Doppler/timing pre-compensation assumption companies are encouraged to check whether NTN functionality can be re-used together with GNSS. Revisit if any issue is found. ...

In this section we analyze the agreements that RAN4 needs to study impact on TDD band operation, that an ATG UE shall be GNSS capable and that companies are encouraged to check whether NTN functionality can be re-used together with GNSS.

### 2.6.1	NTN TA mechanism and ATG compatibility

The RAN1 specified mechanism is specified below.

... TTA = (NTA + NTA,UE-specific + NTA,common + NTA, Offset) x Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signalling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e. ...

The NTA,UE-specific term requires GNSS support and broadcasted satellite orbit data. A Newtonian predictor in the UE use the UE GNSS derived position and the received satellite orbit data in order to calculate the NTA,UE-specific required to have the UE transmissions time aligned at the satellite. The NTA,common represent the feeder link between Gateway and satellite. NTA,common is calculated by the network and broadcasted to the UE as well if one wants time alignment at the NTN gNB/Gateway.

Figure 1: Non-terrestrial network typical scenario based on transparent payload (Figure 4.1-1: in TR 38.821)

Since we can assume that ATG UE has GNSS, then NTA,UE-specific can be calculated. The NTA,common term can be set to 0, since an ATG system does not have a feeder link.

Observation 1: If NTN is used for ATG then ATG UE GNSS support enables calculation of NTA,UE-specific if orbit data corresponding to path of BS relative to ATG UE is provided.

The issue of orbit data needs, to be discussed. One form of orbit data in NTN standard assumes broadcast of ephemeris information using standard orbit elements for a Newtonian orbit. This is not the case here. However there is a second form of orbit data that use position and speed: PositionVelocity-r17 in TS 38.331. This is shown below:

#### EphemerisInfo

The IE EphemerisInfo provides satellite ephemeris. Ephemeris may be expressed either in format of position and velocity state vector or in format of orbital parameters. FFS more detailed description.

EphemerisInfo information element

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

Observation 2: PositionVelocity-r17 in TS 38.331can be used to signal path of BS relative to ATG UE

### 2.6.2	NTN TDD specification status and ATG

The RAN4 TS 38.133 RRM specification is agnostic when it comes to FDD or TDD. There are no statements in TS 38.133 which are particular to NTN FDD only.

RAN1 specifications are not automatically compatible with NTN TDD, since NTN IoT specification parts are split in FDD and TDD parts, where only FDD sections are updated. For example, TS 36.213 v17.1.0 shows the variable “Koffset” as part of the TDD statement, whereas v17.2.0 shows that the variable “Koffset” has now been removed from the TDD statement.

| TS 36.213 v17.1.0 | TS 36.213 v17.2.0 |
| --- | --- |
| ### 16.4.2 UE procedure for reporting ACK/NACK The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of - DL subframe for FDD, - NB-IoT UL subframes following the end of n+12+Koffset subframe for TDD, | ### 16.4.2 UE procedure for reporting ACK/NACK The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of - DL subframe for FDD, - NB-IoT UL subframes following the end of n+12 subframe for TDD, |

However, it seems that RAN1 incompatibilities are restricted to LTE NTN IoT. A first survey of RAN2 and RAN3 specification has not find any incompatibilities, but should be studied further.

Observation 3: The RAN4 TS 38.133 RRM specification is agnostic when it comes to FDD or TDD. RAN1 incompatibilities are restricted to LTE NTN IoT. RAN2 and RAN3 specifications seems to be compatible with NTN TDD.

### 2.6.3	TDD Guard Period for Terrestrial Networks

The guard period of a TDD system must fit all the timing uncertainties like Cell Phase Sync (3 µs) and RF transients (10 or 5 µs), but the dimensioning factor in this case is that we need to fit the propagation time of the cell range as well. In [4] we derive that the guard period, TGuard, becomes:

TGuard ≥ 2* TSync + 2*Tprop_cell edge +max ((TBS on off), (TUE off on)) + max ((TBS off on), (TUE on off))   (1)

where TSync is cell phase sync, TBS and TUE are the RF transients and Tprop_cell edge equals the travel time of the signal across the cell range. The term Tprop_cell edge = 300 kmc = 1 ms. This is a full slot or several slots, even at SCS = 15 kHz depending on how far away the interference will reach (first ring of interferers, second ring etc.). However, the expression in (5) is only valid if there is overlapping coverage. The large ISD and beamforming might mitigate any issues with regards to TGuard.

Observation 4: An ATG system needs a full slot or even several slots of GP, however the large ISD and beamforming might mitigate any issues with regards to GP for TDD.

The formula (1) above is a simplification. To take the analysis one step further is to acknowledge that TGuard ≥ max(TDL_UL) + max(TAOffset). The DL to UL guard is dominating. The TDL_UL is dimensioned by two cases, BS to BS interference and UE to UE interference.

#### 2.6.3.1 	BS to BS interference

BS-B´s (aggressor) “late” DL transmission can cause interference to BS-A´s (victim) “early” UL reception.

Figure 2 BS-to-BS interference at DL to UL switch

As can be seen in Figure 2 the guard period for the DL to UL switch (TDL_UL) must include sufficient time for:

- Time synchronization error between base stations (TSync), in this case base station B is TSync late in relation to base station A.

  - TSync is the timing at the Antenna Reference Point (ARP) and hence in addition to synchronization source errors also include distribution and margins for holdover operation.

- Transition time for turning off the transmitter (TBS on off).

  - The time relates to a specified TX OFF level

- A propagation time (Tprop_BS2BS) between aggressor base station and victim base station.

- TDL_UL ≥ TSync +TBS on off + Tprop_BS2BS	(BS2BS-DL2UL)

The propagation time (Tprop_BS2BS) increases with the distance between aggressor and victim (interference arrives later at victim base station A which in this case is a disadvantage) however due to path loss, the interference level also decreases by distance.

For a certain acceptable interference level, a corresponding minimum time must be allocated for Tprop_BS2BS. This must be analysed in detail for different systems and deployments, and depends on path loss, transmitter output power and receiver properties.

If an acceptable interference level is reached already between neighbouring base stations, then Tprop_BS2BS would equal the propagation time between two nearby base stations (TISD) i.e. Tprop_BS2BS =TISD but if an acceptable interference level first is reached at the neighbour’s neighbour then Tprop_BS2BS =2* TISD and so on.

Observation 5: The guard period for BS to BS interference, for the DL to UL switch is mainly dimensioned by the Tprop_BS2BS, if and when there is unacceptable interference between BS.

#### 2.6.3.2	UE to UE interference

UE-A (aggressor) “early” UL transmission can cause interference to UE-B (victim) still in DL reception, see Figure 6.1.1.2.1-1.

Figure 3 UE-to-UE interference at DL to UL switch

In Figure 3 the guard period in the DL to UL switch must include sufficient time for:

- Time synchronization error (TSync) between UE´s connected to different base stations, in this case UE A is TSync early in relation to UE B. UE A waits at least TSync before ramping its transmitter.

- Transition time for turning on the transmitter (TUE off on).

  - The time relates to a specified TX OFF level

- Propagation time between base station and UE´s at cell edge (Tprop_cell_edge).

To simplify and to get a direct relation towards cell edge we define this as LOS propagation and multiply with a NLOS path compensation factor αNLOS (αNLOS >1).  

In Figure 3, a small distance and thereby small propagation time between the UEs (Tprop_UE2UE ~0) is a disadvantage both from a timing view (the interference from UE A would arrive “earlier” at UE B) and from an interference view (short distance also means higher level of interference). Also early UL transmission of aggressor UE A (large TA) and late DL reception of UE B is a disadvantage timing wise i.e. worst situation is when UEs at cell edge and close to each other (motivation why Tprop_UE2UE not subtracted in formula below). Note, again for worst case, that UEs are close to each other, but both UE are at Tprop_cell edge from their respective base station, at max TA = 2*Tprop_cell edge, to find largest DL to UL gap needed.

- TDL_UL ≥ TSync +TUE off on + 2*Tprop_cell edge	(UE2UE-DL2UL)

Observation 6: The guard period for UE to UE interference, for the DL to UL switch is mainly dimensioned by the 2*Tprop_cell edge, if and when there is unacceptable interference between nearby UE.        

Any deployment of ATG like the figures 4 and 5 below will have to be analyzed with regards to the two factors dimensioning TDL_UL, BS to BS interference and UE to UE interference.

An aircraft has coverage as it flies over the BS. Figure 1 shows the coverage area of the BS, but it is assumed that the beam to an individual aircraft will be a narrow beam that is much smaller than the coverage area.

Figure 4: ATG BS antenna pointing upwards

In the second approach, BS point their beams towards the horizon. In Figure 2, the coloured areas represent the coverage of individual BS. It is assumed that the beams to individual aircraft are much narrower than the coverage width such that there is no interference between beams.

Figure 5: ATG BS point their beams towards the horizon

With the second approach, aircraft are covered when they are distant to the BS and not when they are directly overhead.

# Conclusion

In this contribution we have discussed and provided our view on the RRM issues identified in the way forward document at last meeting. More specifically, we have discussed RRM requirements for the different types of mobility procedures in RRC IDLE and CONNECTED mode and also discussed the measurement impact on CONNECTED mode. Lastly, we have also analyzed the impact on TDD band operation for ATG. Based on the discussions, we have made following proposals and observations:

- RAN4 to include the ATG bands (agreed by RF group) into frequency band grouping section 3.5 in TS 38.133.

- Assistance information related to unpredictable changes in flight operation mode can be two ways: 1) UE detecting and informing the NW, 2) NW informing the UE

- Assistance information comprises at least changes in the UE altitude, flight path and speed, remaining service time for serving cell and side conditions can be FFS.

RAN4 to discuss the technical impact to RRM requirement due to the large serving cell radius in ATG system, such as IDLE mode mobility, CONNECTED mode mobility, and measurement procedure etc.

Discussions on measurement capability shall be postponed until the scenario is agreed in RF group.

The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above a certain threshold which is FFS.

For cell reselection, the A2G UE should resume the neighbor cell measurement in normal manner without any relaxation based on assistance information related to unpredictable changes in flight operation mode.

SDT is supported for A2G in Rel-18, and SDT requirements for ATG is FSS.

CHO is supported for A2G in Rel-18, and CHO requirements for ATG is FSS.

The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify_intra_NR and Tidentify_inter_NR.

Current definitions of known/unknown cells and side conditions in the RRC re-establishment requirements can be reused for ATG.

RAN4 to study the larger cell impact to RRC re-establishment, such as the target cell list and total delay requirement.

RAN4 to define RA requirements only for 4-step RA for ATG scenario.

Existing 4-step RA requirements are reused for ATG.

The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify-NR.

Current definitions/requirements on TRACH are reused for ATG.

RAN4 to study the larger cell impact to RRC connection release with redirection, such as the target candidate cell and total delay requirement.

Discussions on whether the legacy RLM requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

Discussions on whether the legacy BFD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

Discussions on whether the legacy CBD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

Discussions on whether the legacy L1-RSRP measurement requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.

No need to define interruptions due to active BWP in section 8.2 of TS 38.133 for NR serving cells for EN-DC and NE-DC.

Existing active BWP switching requirements (excluding simultaneous and non-simultaneous BWP switching delay on multiple CCs) defined are reused for ATG.

Existing TCI state switching requirements are reused for ATG.

Existing uplink spatial relation switching delay requirements are reused for ATG.

Existing requirements on UE-specific CBW change are reused for ATG.

Pre-MG can be used in ATG network to improve the total system performance.

RAN4 to investigate conditions under which Pre-MG can be disabled/enabled.

Only FR1 MG will be used in Rel-18 ATG network.

The intra-frequency measurement requirement needs to be updated, such as no deactivated SCell measurement, no SCCs, PSCell measurement.

CSSF outside gap needs to be updated.

CSI-RS measurement is unnecessary in ATG network.

RAN4 to study the trade-off between Inter-frequency measurement within MG and the data throughput due to large cell coverage.

Observation 1: If NTN is used for ATG then ATG UE GNSS support enables calculation of NTA,UE-specific if orbit data corresponding to path of BS relative to ATG UE is provided.

Observation 2: PositionVelocity-r17 in TS 38.331can be used to signal path of BS relative to ATG UE

Observation 3: The RAN4 TS 38.133 RRM specification is agnostic when it comes to FDD or TDD. RAN1 incompatibilities are restricted to LTE NTN IoT. RAN2 and RAN3 specifications seems to be compatible with NTN TDD.

Observation 4: An ATG system needs a full slot or even several slots of GP, however the large ISD and beamforming might mitigate any issues with regards to GP for TDD.

Observation 5: The guard period for BS to BS interference, for the DL to UL switch is mainly dimensioned by the Tprop_BS2BS, if and when there is unacceptable interference between BS.

Observation 6: The guard period for UE to UE interference, for the DL to UL switch is mainly dimensioned by the 2*Tprop_cell edge, if and when there is unacceptable interference between nearby UE.

# References

R4-2214347, "WF on NR ATG RRM core requirements", CMCC

TS 38.133 V 17.6.0

R4-2215147, WF on UE requirements for ATG, Apple

R4-1703013, TDD timing budget, Ericsson