3GPP TSG-RAN WG4 Meeting #104bis-e	R4- 2216774

Electronic Meeting, 10 – 19 October 2022

Source: 	Huawei, HiSilicon

Title:	Initial consideration on PC2 FDD UE RF requirements

Agenda Item:	5.22.2

Document for:	Agreement

# 1	Introduction

The objectives of the core part are as follows:

- Scope of this basket WI includes NR FDD bands

- Specify the band specific RF requirements for all listed power class 2 NR FDD bands including

  - UE maximum output power and Tx power tolerance

  - A-MPR requirements if needed

  - PC2 sensitivity degradation requirements if needed

Note 1: Ensure that the UE RF requirements of power class 2 UEs shall comply with those of power class 3 when the maximum transmit power is limited to 23dBm by gNB configuration.

Note 2: The ability to define 1Tx and 2Tx requirements is subject to data availability.

Note 3: Both 1Tx and 2Tx UE architecture are considered in this WI.

A new WID [1] on adding PC2 support to more FDD bands was established in RAN#97-e. The objectives on core RF requirements are duplicated below:

The NR FDD bands proposed in the WID are: n2, n5, n8, n13, n25, n26, n28, n66, n71 and n85. In the following, we share some initial analysis on the UE RF requirements.

# 2	Discussion

Based on the study of the Rel-17 WI, there’re two major issues to be solved in order to enable the support of PC2 on FDD bands, i.e., new A-MPR requirements and MSD requirements owing to the increased Tx power.

As per TS 38.101-1, the following A-MPR requirements shown in Table 1 may need to be updated to account for PC2.

Table 1: Mapping of network signalling label (extracted from Table 6.2.3.1-1A in TS 38.101-1)

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n85 | NS_01 | NS_06 |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. NOTE 2: Additional emission requirements and associated network signalling for Band n104 are not defined in this version of the specification but may be forthcoming in the future. |  |  |  |  |  |  |  |  |

In Rel-17, a set of RF assumptions were agreed for A-MPR evaluation in [2], which may be reused for the current WI.

Among the proposed FDD bands, many have relatively small duplexer gap. It’s likely that they will suffer from sensitivity degradation similar to band n3. The same methodology in Rel-17 can be reused to determine the MSD requirements.

Proposal 1: Reuse the Rel-17 methodology and RF assumptions [2] to evaluate A-MPR and MSD requirements for the requested FDD bands.

Since there’re already 10 bands being requested, they may not be studied simultaneously, e.g. at the same meeting. Companies are encouraged to share the evaluation results for interested band(s), which can be captured in the TR via TPs. After reasonable amount of data is accumulated, the RF requirements for the corresponding band may be determined.

Proposal 2: Companies are encouraged to share the evaluation results for interested band(s), which can be captured in the TR via TPs. The RF requirements for the given band(s) are determined after reasonable amount of data is accumulated.

# 3	Conclusion

In this paper, we have shared our initial analysis on the PC2 FDD bands. And two proposals are presented as follows.

Proposal 1: Reuse the Rel-17 methodology and RF assumptions [2] to evaluate A-MPR and MSD requirements for the requested FDD bands.

Proposal 2: Companies are encouraged to share the evaluation results for interested band(s), which can be captured in the TR via TPs. The RF requirements for the given band(s) are determined after reasonable amount of data is accumulated.

# References

- RP-222625 New WID on high power UE for FR1 for FDD single band(s) with power class 2, China Unicom, RAN#97-e

- R4-2119946 WF on NR FDD PC2 HPUE, China Unicom, RAN4#101-e

- 3GPP TR38.861 V17.0.0 Study on high power UE (power class 2) for one NR FDD band