3GPP TSG-RAN WG4 Meeting #104bis-e	R4- 2216776

Electronic Meeting, 10 – 19 October 2022

Source: 	Huawei, HiSilicon

Title:	Further discussion on the feasibility of improving MSD

Agenda Item:	6.6.4.1

Document for:	Agreement

# 1	Introduction

A WF [1] on study for lower MSD was agreed in RAN4#104-e for the WI [2].  Several band combinations were selected as examples for further study of the feasibility of MSD improvement for different MSD types:

- CA_n28-n40 (harmonic mixing)

- CA_n41-n77 (cross band isolation)

- CA_n1-n3-n78 and fallback combinations (IMD on the 3rd band, cross band isolation on CA_n1-n3 using 50MHz channel bandwidth, IMD2/4 and 2nd harmonic and harmonic mixing on CA_n3-n78)

Following the methodology of our previous contribution [3], we share our further analysis on the example band combinations, attempting to identify the key contributors to MSD and the potential range of improvement.

# 2	Discussion

Some common RF assumptions are listed in Table 1 below, which are the baseline for the following MSD analysis. Some parameters such as PCB/antenna isolation may be varied to evaluate their effect on the MSD performance. Other parameters related to certain types of MSD are described in the individual sub-sections.

Table 1: Common RF assumptions

| Diplexer/Triplexer Isolation | 15 dB |
| --- | --- |
| Antenna Isolation | 10 dB |
| PCB isolation | 70 dB |
| Front-end insertion loss | 4~5 dB |
| PA output power | 27~28 dBm |

## 2.1 MSD by Harmonic Mixing

The L-H band combinations such as CA_n20-n40, CA_n28-n40 and CA_n18-n41 are susceptible to harmonic mixing problems. More than 30dB MSD is specified for CA_n28-n40 when the mixing product of the UL in band n40 and the 3rd harmonic of the DL LO in band n28 falls within the receiver channel bandwidth. The MSD requirements reuse those for LTE CA_28-40, which were defined based on the study of several contributions, e.g. [4][5].

According to the analysis in [4], the conductive path from the aggressor band transmitter to the victim band receiver dominates the interference level. And the victim band receiver duplexer/filter rejection at the aggressor frequency is the key factor in addition to the victim band receiver mixer harmonic suppression. Figure 1 shows the MSD variation with the optimization of the two key factors.

Figure 1: MSD by harmonic mixing varies with the receiver filter rejection and the mixer harmonic rejection. The PCB isolation of 70 dB is assumed.

## 2.2 MSD by Cross-band Isolation

As seen from the REFSENS requirements in TS 38.101-1, the MSD is usually more severe when the source of cross-band isolation interference is “ACLR1” or “ACLR2” (e.g. CA_n18-n28, CA_n5-n28). If the aggressor channel BW is large (e.g. 50/100MHz), the MSD may increase to more than 10 or even above 20 dB (e.g. CA_n1-n3, CA_n1-n40), even if the source is classified as “>ACLR2”. It’s worth noting that the spec requirements target the worst cases, where the aggressor carrier frequency is set as close as possible to that of the victim.

As shown by the analysis in [9], the PA noise and cross-band isolation are the main factors affecting the MSD when the carrier frequencies and UL RB allocation are fixed.

## 2.3 MSD by Harmonic Interference

The band combination of CA_n3-n78 or DC_3_n78 may suffer from MSD caused by H2 of the UL in band 3/n3 falling into the DL of band n78. All the RF components in the front-end of band 3/n3 could contribute to the generation of H2, including: duplexer, antenna switch, diplexer/triplexer and etc. It was identified in [6] that the main factors are:

- PA linearity

- Total filter attenuation (including Harmonic Trap Filter and B3 duplexer)

- PCB isolation.

The filter survey in [6] shows that the B3 duplexer attenuation at B42 ranges from 10 to 27 dB, and the discussions recorded in [7] indicates that 30 dB is possible for future design. Additionally, the following assumptions were considered optimistic at that time: PA H2 = -35 dBc, HTF = 30dB and PCB isolation (@3.5GHz) = 70dB [7][8].

By tuning the total filter attenuation and the PCB isolation while fixing the PA H2=-35dBc, the MSD trend is obtained as shown in Figure 2.

Figure 2: MSD by Tx harmonic varies with the Tx filter attenuation and the PCB isolation. The PA H2 is assumed to be -35dBc.

Figure 3 illustrates the combination of carrier frequencies that may suffer from MSD caused by Tx harmonic.

Figure 3: Illustration of carrier frequency combinations that may be affected by MSD caused by H2.  CBW: 10MHz for n3, 20MHz for n78.

## 2.4 MSD by Dual-UL Inter-Modulation Distortion

When both uplinks are active, the DL band n3 of CA_n3-n78 may suffer from IMD2 and IMD4 of the dual-UL. It has been identified in [3] that PCB isolation and antenna isolation are the main factors. The MSD trend is reproduced in Figure 4 below.

Figure 4: MSD caused by IMD2 for CA_n3-n78 varies with different PCB isolation and antenna isolation. The 40dB antenna isolation is impractical for small form-factor UEs but serves as an performance indicator for conductive measurements.

In order for the self-interference to fall within the own receiver, the aggressors’ carrier frequencies and the victims’ carrier frequencies need to satisfy certain mathematical constraints, subject to the order of the IMD. Figure 5 indicates the occurrences of IM interferences for CA_n3-n78.

Figure 5: Illustration of the relationship between IMD and the carrier frequencies. The red color indicates that the IM product may fall into the victim Rx channel, assuming CBW: 10MHz for n3, 20MHz for n78.

It can be seen Figure 5 and 3 that for the majority of the carrier frequency configurations of CA_n3-n78, there’s no self-interference from IMD2 or IMD4 or H2. Furthermore, when MSD from IMD2 happens for a given deployment, MSD from IMD4 does not, or vice visa. Since n78 is a TDD band, the MSD caused by H2 of n3 will not happen at the same time as the MSD caused by IMD2 or IMD4 of the dual UL.

## 2.5 Summary of MSD

Based on the above discussions, the key contributors for different types of MSDs are summarized in Table 2 below.

Table 2: Key contributors for different MSD types

| MSD Types | Key Contributors |
| --- | --- |
| Harmonic Mixing (e.g. UL1/DL3, UL1/DL5, UL2/DL3, UL3/DL2) | Victim filter rejection and mixer harmonic suppression |
| Tx Harmonic (e.g. H2/3/4/5) | Aggressor PA linearity, harmonic filter and duplexer attenuation, PCB isolation |
| Cross-band Isolation (e.g. ALCR1/2, >ACLR2) | Aggressor PA noise and filter attenuation |
| Dual-UL IMD (e.g. IMD2/3/4/5/7) | PCB isolation, antenna isolation |

It has been shown that the MSD may be improved by optimizing the key contributors listed in Table 2. However, it can also be seen that for large MSD values (>20dB) it would be very challenging to reduce them to below 10dB. Furthermore, we can make the following observations:

Observation 1: Different types of MSDs may have different limiting factors. Which one can be improved for what band combinations are design choices of the UE implementation.

Observation 2: For large MSD values (>20dB) it’s very challenging to reduce them to below 10dB. The relatively small MSD values (<10 dB) may be further improved depending on UE implementation.

Observation 3: The antenna isolation plays an important role in MSD performance. However, the effect is not verified by the existing conductive tests.

Observation 4: For a given band combination, MSD from different sources are unlikely to happen simultaneously, depending on the configuration of carrier frequencies, duplex mode of the component bands, the order of IMD or harmonics and etc.

Based on the above observations, we make the following proposals:

Proposal 1: When the MSD is improved to below [5] dB, the concern about the self-interference level for such UEs may be alleviated.

Proposal 2: There is no need to report MSD > [15] dB. Adaptive scheduling by the network will be the main means to avoid the high self-interference level for such UEs. Whether/how to report MSD values between [5] and [15] dB is FFS.

Proposal 3: Given that the MSD of different types are unlikely to happen simultaneously, the network should be able to enquire and select the most relevant MSD capability to be reported by the UE.

# 3	Conclusion

By analyzing some of the example band combinations, we further discuss the feasibility of MSD improvement. The following observations and proposals have been presented:

Observation 1: Different types of MSDs may have different limiting factors. Which one can be improved for what band combinations are design choices of the UE implementation.

Observation 2: For large MSD values (>20dB) it’s very challenging to reduce them to below 10dB. The relatively small MSD values (<10 dB) may be further improved depending on UE implementation.

Observation 3: The antenna isolation plays an important role in MSD performance. However, the effect is not verified by the existing conductive tests.

Observation 4: For a given band combination, MSD from different sources are unlikely to happen simultaneously, depending on the configuration of carrier frequencies, duplex mode of the component bands, the order of IMD or harmonics and etc.

Proposal 1: When the MSD is improved to below [5] dB, the concern about the self-interference level for such UEs may be alleviated.

Proposal 2: There is no need to report MSD > [15] dB. Adaptive scheduling by the network will be the main means to avoid the high self-interference level for such UEs. Whether/how to report MSD values between [5] and [15] dB is FFS.

Proposal 3: Given that the MSD of different types are unlikely to happen simultaneously, the network should be able to enquire and select the most relevant MSD capability to be reported by the UE.

# References

- R4-2214452 WF on study for lower MSD, Huawei, HiSilicon, RAN4#104-e

- RP-220987 New WID Further RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN#95

- R4-2212452 Discussions on the feasibility of improving MSD, Huawei, HiSilicon, RAN4#104-e

- R4-156835 TP for TR 36.852-13: on CA_28A-40A UE RF requirements, Huawei, HiSilicon, RAN4#76bis

- R4-152188 MSD analysis for inter-band CA with lower band Rx harmonic mixing problem, MediaTek Inc, RAN4#74bis

- R4-147283 MSD analysis on CA_3A-42A and CA_3A-42C, NTT Docomo Inc, RAN4#73

- R4-148024 UE CA issues minutes, RAN4#73

- R4-73AH-0099 More on B3+B42 A2 CA combination, Qualcomm, RAN4#73-AH-UE-RF

- R4-2214004 Corrections to NR-CA cross-band isolation MSD test points, Skyworks, RAN4#104-e