3GPP TSG-RAN WG4 Meeting # 104-Bis-e 	R4-2216787

# Electronic Meeting, Oct. 2022

Source: Qualcomm Incorporated

Title: 	 On test setup for UE RF requirements for 2AoA FR2 DL MIMO

Agenda item:	6.8.2.2

Document for:	Discussion

# Introduction

The overall objective of the multi-Rx chain DL reception WI is to enable a UE to receive up to 4L ‘with simultaneous DL reception with two different QCL TypeD RSs on single component carrier’ [1]. In this contribution we propose that UE RF requirement definition work can continue despite lack of convergence on test set up.

# Discussion

## High level direction for UE RF requirements

Fig 2.1-1: UE RF requirement configuration

Each arrow represents a Rank1 signal

A previous agreement [2] established that UE RF requirements …. shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions.

This scheme is shown graphically in Figure 2.1-1. Note that UL is arbitrarily shown to be associated with one of the TCI states. For this feature, the UL is not intended to be also upgraded for multi-TCI states. The UL configuration is assumed to be single TCI state rank1.

This agreement is significant as a boundary condition for the construction of a UE RF requirement because it deviates from directly verifying the eventual goal of this WI, which is to enable 4L DL MIMO.

Put another way, the UE RF requirement is merely a FOM to score UE functionality to independently track 2 QCLD RSs, rather than staying true to some 4L deployment condition. This freedom allows closer co-development across the test setup and the UE requirement. In this case, it is generally understood that the UE requirement would be a strong function of the test set up capability.

Observation 1: RAN4 needs to determine test set feasibility before finalizing details of the FOM requirement for 2 TCI state spherical coverage.

We note however that many details of the UE RF requirement are agnostic of the test setup and can be pursued in parallel with the test set discussion. We consider some of these details in a companion contribution [3].

Proposal 1: RAN4 to continue work towards defining UE RF requirements on aspects that are not influenced by test-set design.

# Conclusions

Observation 1: RAN4 needs to determine test set feasibility before finalizing details of the FOM requirement for 2 TCI state spherical coverage.

Proposal 1: RAN4 to continue work towards defining UE RF requirements on aspects that are not influenced by test-set design.

# Reference

[1]	RP-221753, ‘Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception’, Qualcomm, RAN#96, Jun. 2022

[2]	R4-2214457, ‘WF on FR2 multi-Rx chain DL reception’, Sony, RAN4#104-e, Aug. 2022

[3]	R4-2216786, ‘On UE RF requirements for 2AoA FR2 DL MIMO’, Qualcomm, RAN4#104-Bis-e, Oct. 2022