3GPP TSG RAN WG4 Meeting #104-bis-e	R4-2216831

E-meeting, 10 Oct. 2022 – 19 Oct. 2022

Agenda Item:	6.20.3

Source: 	Ericsson

Title:	Discussion on L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the RAN4 aspects to support L1/L2 based inter-cell mobility for mobility latency reduction.

# Discussion

## Scenarios

Whether to consider simultaneous Rx/Tx with both source cell and target cell:

In last meeting, following WF was agreed.

- Option 1: Not consider simultaneous Rx/Tx with both source cell and target cell.

- Option 2: FFS, need further clarification

We think DAPS plus L1L2 mobility is not part of WID and we don’t think we need to discuss this.

- RAN4 not to discuss simultaneous Rx/Tx with both source cell and target cell in the context of L1L2 mobility.

Whether to consider simultaneous multi-panel in FR2:

In last meeting, following WF was agreed.

- Option 1: Not consider simultaneous multi-panel in FR2

- Option 2: Consider simultaneous multi-panel in FR2

In our understanding UE supports multiple panels from Rel-16 onwards for receiving PDSCH from different TRPs with different QCL type-D. However, in RAN4 due to workload, we did not defined requirements though it was being discussed In Rel-17. Since this is the start of Rel-18, we think RAN4 shall consider UE multi-panel capability while defining requirements.

When UE is capable of receiving from different TRP, following scenarios may need to be considered.

- Scenario 1: Receive data on TRP1 + data on TRP2

- Scenario 2: Receiving data on TRP1 + perform measurements on TRP2

- Scenario 3: Perform measurements on TRP1 + Perform measurement on TRP2

We think all the scenarios mentioned above are supported from Rel-16 from RAN1 perspective and we think RAN4 shall consider these scenarios while defining the requirements.

In the case of L1/L2 mobility, we think the TRP in above scenarios can be replaced with neighbour cell or candidate cells. That means UE can receive PDSCH from one TRP or receive measurement RS from other TRP of candidate cell for L1/lL2 mobility at least in some scenarios.

Further, in some scenarios L1/L2 mobility may be a solution configured instead of L3 mobility. Hence, It is important that UE supports or measures as many cells as L3 mobility so that the benefit of the feature can be fully utilised. Unless we consider the UE capability of multi-RX chain or simultaneous reception of measurement RS from two different TRP, the full potential of L1L2 mobility cannot be utilised.

- UE capability of supporting multi-RX chain and simultaneous RX from different QCL type-D should be considered in L1/L2 mobility.

In Rel-17, inter-cell BM, target TRPs are considered as only intra-frequency carriers with same SCS and same active BWP. However, the L1/L2 mobility supports both intra-frequency and inter-frequency measurements (please see highlighted text below) from the scope of the WID.

Note 3: The procedure of L1/L2 based inter-cell mobility are applicable to the following scenarios:

    - Standalone, CA and NR-DC case with serving cell change within one CG

    - Intra-DU case and intra-CU inter-DU case (applicable for Standalone and CA: no new RAN interfaces are expected)

    - Both intra-frequency and inter-frequency

    - Both FR1 and FR2

    - Source and target cells may be synchronized or non-synchronized

Since L1/L2 mobility is kind of alternative to L3 mobility (to achieve better interruption downtime) for some scenarios, we think HO to an inter-frequency cell is a highly possible scenario and shall be considered to define L1-RSRP measurement requirements for an inter-frequency candidate cell.

- RAN4 to consider both intra-frequency and inter-frequency candidate cells for defining the requirements for L1 measurements.

Definition of intra-frequency/inter-frequency in inter-cell operation:

- Option 1 (Apple, Ericsson, vivo, HW, Nokia): RAN4 to discuss the definition of intra-frequency/inter-frequency in inter-cell operation

  - Option 1a (MTK): For SSB L1-RSRP measurement, follow the definition of L3 measurement.

  - Option 1b: For SSB L1-RSRP measurement, intra-frequency is defined as long as the SSB-based L1 measurement is performed within active BWP(s) of the UE.

- Option 2 (QC): discuss later

We think definition of intra-frequency or inter-frequency measurement can follow the L3-measurement definition as baseline and further discuss if any change is required further.

- RAN4 to consider the definition intra and inter-freq. of L3 measurement as baseline.

Inter-frequency measurements:

Inter-frequency measurements for L3 mobility are defined with both with gap and without gap. Just like L3 mobility measurements, we think UE needs measurement gaps to measure an inter-frequency L1-RSRP layer or cell.

At the end of Rel-17, RAN4 spec consists of multiple flavours of MG, such as legacy MG, concurrent MG, NCSG. RAN4 shall discuss the design principles to follow for L1 measurements in inter-frequency with all possible gap types.

- RAN4 to agree that MG can be used for inter-frequency L1 measurements.

## RRM requirements

In the last meeting following requirements are agreed to be defined.

- L1/L2 inter-cell mobility delay requirements

- L1-RSRP measurement delay requirements

- Timing management requirements

- Timing requirements

L1/L2 inter-cell mobility delay requirements:

Unless RAN1/RAN2 has decent level of agreements on the framework of L1/L2 mobility, we think RAN4 cannot discuss the delay component details. In last meeting RAN2 sent a LS to RAN1 and RAN4 and the contents of the LS are copied below for reference.

Assumption: HO interruption time for L1/L2-based inter-cell mobility is the time from UE receives the cell switch command to UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. FFS if TRS tracking after HO and CSI RS measurement should also be included, i.e. the time to use a high-performance beam (can be clarified further). Assumption: To reduce HO interruption time, investigate e.g. solutions to reduce the time for UE reconfiguration (already in the WID), downlink and uplink synchronization after handover decision (other parts of dynamic switch not precluded). Confirm to Support L1/L2-based inter-cell mobility for inter-DU scenario (as well as intra-DU scenarios). The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery. ICBM is one scenario considered for L1L2 mobility, but is not the only one, and is not a prerequisite for using L1L2 mobility. RAN2 to consider preparation of target cell configurations capable of dynamic switching without need for full configuration. Measurement delay can/may be considered in this work Assume that we rely on L1 measurements to trigger L1L2 mobility (still measurement for preparation could be L3, FFS) R2 will initially focus on PCell mobility. R2 assumption: Rel-18 L1/L2 mobility includes both non-CA (PCell only) and CA scenarios (PCell and SCell). This includes the following cases - a) the target PCell/target SCell(s) is not a current serving cell (CA CA scenario with PCell change) - b) FFS the target PCell is a current SCell - c) FFS the target SCell is the current PCell. DC scenarios are FFS (e.g. PSCell mobility may be a low hanging fruit FFS). Current options on the table: to configure a L1/L2 inter-cell mobility candidate cell: - a. One RRCReconfiguration message for candidate target cell - b. One CellGroupConfig IE for each candidate target cell - c. One SpCellConfig IE for each candidate target cell Will send an LS to RAN1 and RAN3 on the progress of this meeting.

From the above highlighted text from current RAN2 assumption for HO interruption time, start point of HO delay is UE receives the cell switch command. Stop point of the HO is when UE performs the first DL/UL reception/transmission on the indicated beam of the target cell. Unless RAN1/RAN2 design matures, first DL/UL transmission to NW is not clear. Since RACH less HO or any other solution to skip RACH may be a possibility, RACH preamble transmission may or may not be stop point of the HO. Having said that, by taking L3 HO delay requirements equation as baseline, the L1/L2 Handover delay equation can be sum of processing time of cell switch command plus interruption time.

Processing time of cell switch command may a RAN1 or RAN2 decision based on the command used for the L1/L2Mobility.

Since RAN1 and RAN2 is discussing the details of the procedures it may be too early to discuss this in RAN4 now.

- RAN4 to agree HO delay as processing time of cell switch command plus the interruption time.

## Measurement requirements

In L1/L2 mobility, UE can be configured with multiple candidate cells/TRPs, and they can be in same frequency or different frequency. We think these TRPs can be received within the CP or outside of the CP. Since L3 mobility do not consider neighbouring cells or candidate cells to be within CP alone, we should consider also RTD between serving cell and neighbour cell more than CP. While defining measurement requirements we think RAN4 should consider MRTD>CP.

- RAN4 to consider RTD>CP for L1/L2 mobility candidate cell measurements.

Intra-frequency L1-RSRP measurement accuracy requirements:

In last meeting following options are discussed.

- Option 1: Discuss whether intra-frequency L1-RSRP measurement accuracy can be improved for L1/L2 mobility

- Option 2: reuse the legacy intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

- Option 3: Discuss this issue later, e.g., in performance part

We think existing L1-RSRP accuracy requirements are quite relaxed (6.5 dB). Whereas L3 measurement accuracy requirements are bit better (4.0dB) than L1-RSRP accuracy. Since current HO decisions are based on L3-RSRP accuracy, if we make HO decision based on current L1-RSRP accuracy, it may result in pin pongs or HO failures. Therefore, it is beneficial to consider improving L1-RSRP accuracy.

- RAN4 to discuss the tightening of intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility

Inter-frequency L1-RSRP measurement accuracy requirements

- Option 1: Define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell

- Option 2: Further discuss whether and how to define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell later, e.g., in performance part.

For this issue, we support option 1 as L1/L2 HO is based on neighbour cell or candidate cells measurements, it is important to specify accuracy requirements for candidate cells and neighbour cells can be on inter-frequency.

- RAN4 to define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell

In Rel-17, RAN4 agreed that L1-RSRP on cell with different PCI not to impact the L3 mobility. However, since L1-RSRP measurements on inter-cell L1-RSRP is for mobility purpose, L1-RSRP for L1/L2 mobility can be measured within SMTC.

- Candidate cell L1-RSRP measurements can be measured within SMTC.

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for L1/L2 based inter-cell mobility and made following proposals.

- RAN4 not to discuss simultaneous Rx/Tx with both source cell and target cell in the context of L1L2 mobility.

- UE capability of supporting multi-RX chain and simultaneous RX from different QCL type-D should be considered in L1/L2 mobility.

- RAN4 to consider both intra-frequency and inter-frequency candidate cells for defining the requirements for L1 measurements.

- RAN4 to consider the definition intra and inter-freq. of L3 measurement as baseline.

- RAN4 to agree that MG can be used for inter-frequency L1 measurements.

- RAN4 to agree HO delay as processing time of cell switch command plus the interruption time.

- RAN4 to consider RTD>CP for L1/L2 mobility candidate cell measurements.

- RAN4 to discuss the tightening of intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

- RAN4 to discuss the tightening of intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

- Candidate cell L1-RSRP measurements can be measured within SMTC.

# References

- RP-221799 “WID on Mobility enhancements”, MediaTek

- R2-2209257