3GPP TSG-RAN WG4 Meeting # 104-bis-e 													R4-2216905

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			4.4.3

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [104-bis-e][321] NR_cov_enh_Demod

Document for:	Information

Introduction

This email thread discusses the demodulation part of the Rel-17 NR coverage enhancement WI: in agenda 4.4.2.1 and 4.4.2.2.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round:

  - Invite companies to provide comments for the remaining open issues in section 1.3 and 2.3 before Oct 13th 0:00 UTC.

  - Invite companies to provide comments for draft CRs in section 3.2 during the first-round discussion.

  - Invite companies to update the ideal and impairment simulation results for PUSCH and PUCCH in the summary spreadsheets during the first-round discussion.

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| China Telecom (Moderator) | Wu Jingzhou | wujingzhou@chinatelecom.cn |
| Huawei | Zehan Zhao | zhaozehan@hisilicon.com |
| Ericsson | Nicholas Pu | Nicholas.pu@ericsson.com |
| Samsung | Yunchuan Yang | yc0301.yang@samsung.com |
| Nokia, Nokia Shanghai Bell | Khouloud ISSIALI | Khouloud.issiali@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: PUSCH Enhancements of Rel-17 NR Coverage Enhancement

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215325 | China Telecom | Summary of simulation results for PUSCH coverage enhancements |
| R4-2215326 | China Telecom | Proposal 1: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in [1] Observation 1: No big performance difference is observed under practical HARQ process number of 2/4/8, due to the negligible time diversity gain under the low speed channel model (TDLA30-10) we have agreed. Proposal 2: Not to specify the number of HARQ process which is aligned with all other existing BS demodulation requirements in the spec. |
| R4-2215638 | Nokia, Nokia Shanghai Bell | Applicability rule for PUSCU and PUCCH JCE Proposal 1:RAN4 to remove the square bracket around aTDW, i;e: use “JCE requirements can be applied for different TDD patterns with 2 or more physical UL consecutive slots and with the same aTDW length of 2 consecutive slots.” |
| R4-2215639 | Nokia, Nokia Shanghai Bell | PUSCH demodulation performance of Rel-17 NR coverage enhancements: simulation results |
| R4-2215688 | Ericsson | Simulation results for PUSCH with TBoMS and JCE |
| R4-2215995 | Huawei,HiSilicon | Observation 1: There is maximum 0.63dB difference that can be negligible between different number of HARQ process. Proposal 1: If finally the simulation results are still not aligned in this meeting, the rule defined in Rel-15 R4-1904713 should be reused, i.e. if the ideal span is larger than 2dB, the results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available. Proposal 2: Do not explicitly specify the number of HARQ process for PUSCH JCE performance requirements. |
| R4-2215996 | Huawei,HiSilicon | Simulation results on BS coverage enhancement demod PUSCH |
| R4-2216686 | Samsung | Simulation results for PUSCH demodulation requirement for Rel-17 coverage enhancement |

Open issues summary

Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements

- Proposals:

  - Option 1: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure in R4-1904713 as below (CTC, HW):

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

- Recommended WF

  - Encourage feedback on option 1.

Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE

- Status in the last meeting Chairman note:

  - Agreement: Assuming no such limitation needed as baseline and further check in next meeting based on the simulation results

- Companies’ observations based on simulation results:

  - China Telecom: No big performance difference is observed under practical HARQ process number of 2/4/8, due to the negligible time diversity gain under the low-speed channel model (TDLA30-10).

  - Huawei: There is maximum 0.63dB difference that can be negligible between different number of HARQ process.

- Proposals:

  - Option 1: Not to specify the number of HARQ process (CTC, HW, Samsung)

- Recommended WF

  - Confirm the baseline in the last meeting and not to specify the number of HARQ process.

Issue 1-3: Test requirement applicability rule for BS PUSCH demod requirements with JCE

- Status in the last meeting Chairman note:

  - Agreement: The same TDD requirements are applicable to different UL-DL patterns with more than one consecutive UL slots when both pusch-TimeDomainWindowLength and PUSCH aggregation factor are configured as 2 slots. The UL (re)transmission of PUSCH is only scheduled for the actual TDW including 2 consecutive UL slots.

- Proposals:

  - Nokia: RAN4 to remove the square bracket around aTDW, i;e: use “JCE requirements can be applied for different TDD patterns with 2 or more physical UL consecutive slots and with the same aTDW length of 2 consecutive slots.”

- Recommended WF

  - Nokia’s proposal is aligned with last meeting agreement, which has also been captured in the big CRs endorsed in the last meeting in R4-2214391.

  - Considering the above, it is the moderator’s recommendation no more discussion on this issue.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE |
| China Telecom | Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements We support option 1. Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE Support the recommended WF. |
| Huawei | Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements OK with the recommended WF. Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE OK with the recommended WF. Issue 1-3: Test requirement applicability rule for BS PUSCH demod requirements with JCE OK with the recommended WF. |
| Ericsson | Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements We agree with Option 1. Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE We agree with Option 1. |
| Samsung | Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements OK with option 1, further alignment is needed Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE OK with option 1, if there is no much different with different HARQ process |
| Nokia, Nokia Shanghai Bell | Issue 1-1: SNR requirement value deriving rule for PUSCH BS demod requirements Support option 1. Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE Option 1. Support the recommended WF. |

Summary for 1st round

Issue 1-1: SNR requirement value deriving rule for PUSCH and PUCCH BS demod requirements

- Proposals:

  - Option 1: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure in R4-1904713 as below (CTC, HW):

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

- GTW Agreement: Option 1 agreed

Issue 1-2: Number of HARQ process for BS PUSCH demod requirements with JCE

- Proposals:

  - Option 1: Not to specify the number of HARQ process (CTC, HW, Samsung)

- GTW Agreement: Option 1 agreed

Discussion on 2nd round

No open issues.

Topic #2: PUCCH Enhancements of Rel-17 NR Coverage Enhancement

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215643 | Nokia, Nokia Shanghai Bell | Simulation results collection for coverage enhancement for PUCCH |
| R4-2215689 | Ericsson | Simulation results for PUCCH with JCE |
| R4-2216000 | Huawei,HiSilicon | Proposal 1: If finally the simulation results are still not aligned in this meeting, the rule defined in Rel-15 R4-1904713 should be reused, i.e. if the ideal span is larger than 2dB, the results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available. |
| R4-2216001 | Huawei,HiSilicon | Simulation results on BS coverage enhancement demod PUCCH |
| R4-2216507 | Nokia, Nokia Shanghai Bell | PUCCH demodulation performance of Rel-17 NR coverage enhancements: simulation results |
| R4-2216687 | Samsung | Simulation results for PUCCH demodulation requirement for Rel-17 coverage enhancement |

Open issues summary

Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements

- Proposals:

  - Option 1: Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure in R4-1904713 as below (CTC, HW):

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

- Recommended WF

  - Considering that there will be similar discussion for PUSCH, it is recommended to reuse the same discussion outcome in Issue 1-1 to avoid possible misalignment.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements |
| China Telecom | Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements Support to reuse the same discussion outcome for PUSCH requirements. |
| Huawei | Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements OK with the recommended WF. |
| Ericsson | Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements We are OK with recommended WF. |
| Samsung | Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements We are OK with recommended WF. |
| Nokia, Nokia Shanghai Bell | Issue 2-1: SNR requirement value deriving rule for both PUCCH BS demod requirements Support the recommended WF. |

Summary for 1st round

Discussion on 2nd round

No open issues.

Topic #3: Draft CRs Review

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215327 | China Telecom | Draft CR on PUSCH with DMRS bundling BS performance test for FR1 |
| R4-2215328 | China Telecom | Draft CR on PUSCH with DMRS bundling BS performance test for FR2 |
| R4-2215640 | Nokia, Nokia Shanghai Bell | draftCR for 38.104: FRC for TBoMS and PUSCH JCE |
| R4-2215641 | Nokia, Nokia Shanghai Bell | draftCR for 38.141-1: Perf requirements for PUCCH JCE |
| R4-2215642 | Nokia, Nokia Shanghai Bell | draftCR for 38.141-2: Perf requirements for PUCCH JCE |
| R4-2215685 | Ericsson | draftCR for TS38.104 PUSCH with JCE for FR1 and FR2 |
| R4-2215686 | Ericsson | draftCR for TS38.141-1 manufacture declaration, applicability rule, MU and TT |
| R4-2215687 | Ericsson | draftCR for TS38.141-2 manufacture declaration, applicability rule, MU and TT |
| R4-2215997 | Huawei,HiSilicon | Draft CR on requirements for PUSCH TBoMS (TS38.104, Rel-17) |
| R4-2215998 | Huawei,HiSilicon | Draft CR on requirements for PUSCH TBoMS (TS38.141-1, Rel-17) |
| R4-2215999 | Huawei,HiSilicon | Draft CR on requirements for PUSCH TBoMS (TS38.141-2, Rel-17) |
| R4-2216688 | Samsung | Draft CR on PUCCH JCE requirements for TS 38.104 |
| R4-2216689 | Samsung | Draft CR on FRC for TBoMS and PUSCH JCE for TS 38.141-1 |
| R4-2216690 | Samsung | Draft CR on FRC for TBoMS and PUSCH JCE for TS 38.141-2 |

Comments collection

Draft CR for TS 38.104:

| Draft CR | Comments |
| --- | --- |
| R4-2215997, requirements for PUSCH TBoMS, Huawei | Company A: |
|  | CTC: Update the SNR value based on the updated results. |
|  | Nokia: - Update the SNR values - In Table: 8.2.12.1-1 Test parameters for testing PUSCH TBoMS. We don’t have agreement to have 5 Mhz in 15 kHz and 30 kHz? Please update this by removing it as it doesn’t exist in legacy. Same comment for Table 11.2.2.9.1-1: Test parameters for testing PUSCH TBoMS, please remove the 50 Mhz from the table. In legacy, we have 5Mhz with 15 kHz and 10 Mhz with 30 kHz. |
| R4-2215685, PUSCH JCE for FR1 and FR2, Ericsson | Company A: |
|  | Huawei: There is change on change. Ericsson: We found some parts in previous draft CRs might need modification. As moderator suggestion, different change tracks could help to check the modifications. Please help to double check if new changes are necessary, thanks a lot. |
|  | CTC: 1) Note 2 in Table: 8.2.13.1-1 should be applicable for both FDD and TDD. 2) Update the SNR value based on the updated results. 3) Update the FRC numbering according to R4-2215640. |
|  | Nokia: G-FR1-Ax-y and SNR values to be updated |
| R4-2216688, PUCCH JCE requirements, Samsung | Company A: |
|  | Huawei: For Test1, Example TDD pattern should not be specified. For Test2, it should be 7D1S2U for 15kHz SCS rather than 7D1S1U. There is typo that intra-slot frequency hopping should be disabled rather than diable. |
|  | CTC: The channel model should be TDLA30-10 in Table 8.3.13.2-1 and Table 8.3.13.2-2. The SNR values need to be updated in the second round. |
|  | Nokia: - Typo in Table 8.3.12.1.1-1: : intra-slot frequency hopping: disabled - For clarity, we propose to add in Table 8.3.12.1.1-1: Test Parameters [TDD] and [FDD] to Test1 and Test2 respectively as follows: - In Table 8.3.13.1-1: Test Parameters, First PRB after frequency hopping is NA in case of frequency hopping disabled. We propose to remove it from the table. |
| R4-2215640, FRC for TBoMS and PUSCH JCE, Nokia | Company A: |

Draft CR for TS 38.141-1:

| Draft CR | Comments |
| --- | --- |
| R4-2215686, manufacture declaration, applicability rules and TT, Ericsson | Company A: |
|  | Huawei: For coversheet, TS 38.141-1 cannot affect TS 38.141-2. Ericsson: Generally, TS 38.141-2 1-O requirements would refer to TS38.141-1, so we think 38.141-1 could affect 38.141-2. |
|  | Nokia: Do we have agreement on the applicability rule for different receiver antenna in case of DM-RS bundling to have section 8.1.2.9.3 Applicability of requirements for different receiver antenna connectors? |
|  | CTC3: For Nokia’s information, we have agreed in the last meeting that an applicability rule will be defined for PUSCH JCE for 2/4/8 Rx tests as below. Issue 1-2-2: Antenna configuration for TBoMS - Follow the agreement in the last meeting: - Only cover 1T2R for TBoMS - PUSCH JCE: - 2/4/8 Rx - Introducing test applicable rules to only verify highest number of Rx antennas BS supported. |
| R4-2215998, requirements for PUSCH TBoMS, Huawei | Company A: |
|  | CTC: Update the SNR value based on the updated results. |
|  | Nokia: SNR values to be updated |
| R4-2215327, JCE BS performance test for FR1, CTC | Company A: |
|  | Huawei: 1. It should be RAN instead of ME in the cover sheet 2. There is change on change. 3. It is better to align the order of the requirements tables with current spec, i.e. Type A 5MHz, Type A 10MHz, Type B 5MHz, Type B 10MHz. |
|  | CTC: Update the SNR value based on the updated results. Update the FRC. |
|  | Nokia: FRC and SNR values to be updated. |
| R4-2215641, Perf requirements for PUCCH JCE, Nokia | Company A: |
|  | Huawei: For PUCCH JCE, Example TDD pattern and corresponding notes should be specified. |
|  | Ericsson: We think it would be better to align with previous sub-section methodology for PUCCH formats, i.e., 8.3.11 for PUCCH format 1 with DM-RS bundling, 8.3.12 for PUCCH format 3 with DM-RS bundling. |
|  | CTC: Update the SNR value based on the updated results. |
| R4-2216689, FRC for TBoMS and PUSCH JCE, Samsung | Company A: |
|  | Huawei: For coversheet, TS 38.141-1 cannot affect TS 38.141-2. |
|  | CTC: FRC for PUSCH JCE is missing in A.3B. Should be aligned with R4-2215640. |
|  | Nokia: - the number of section “A.3” is missing before “Fixed Reference Channels for performance requirements” - |

Draft CR for TS 38.141-2:

| Draft CR | Comments |
| --- | --- |
| R4-2215687, manufacture declaration, applicability rules and TT, Ericsson | Company A: |
|  | Nokia: Same comment as R4-2215686 about the applicability rule for different Rx antennas. |
| R4-2215999, requirements for PUSCH TBoMS, Huawei | Company A: |
|  | CTC: Update the SNR value based on the updated results. |
|  | Nokia: Same comment as R4-2215997 about the bandwidth of 5 Mhz for 30 kHz in Table: 8.2.12.4.2-1 We commonly use 10 Mhz for 30 kHz and not 5 MHz. |
| R4-2215328, JCE BS performance test for FR2, CTC | Company A: |
|  | Huawei: Same as comments to R4-2215327. 1. It should be RAN instead of ME in the cover sheet 2. There is change on change. 3. It is better to align the order of the requirements tables with current spec, i.e. Type A 5MHz, Type A 10MHz, Type B 5MHz, Type B 10MHz. |
|  | CTC: Update the SNR value based on the updated results. Update the FRC. |
| R4-2215642, Perf requirements for PUCCH JCE, Nokia | Company A: |
|  | Huawei: For PUCCH JCE, Example TDD pattern and corresponding notes should be specified. |
|  | Ericsson: We think it would be better to align with previous sub-section methodology for PUCCH formats, i.e., 8.3.11 for PUCCH format 1 with DM-RS bundling, 8.3.12 for PUCCH format 3 with DM-RS bundling. |
|  | CTC: Update the SNR value based on the updated results. |
| R4-2216690, FRC for TBoMS and PUSCH JCE, Samsung | Company A: |
|  | CTC: 1) Title of Table A.3B-2, should be for FR2 and DMRS pos1. 2) Please double check the payload size, CRC length abd bits per CB of G-FR2-A3B-1 and G-FR2-A3B-1, which is different from the calculation in R4-2215640. Based on our calculation, R4-2215640 is correct. 3) Table A.3-X1 is not needed in TS38.141-2. |
|  | Nokia: Typos in Table A.3B-2 - FR2 instead of FR1 - l=8 instead of l=10 for PUSCH mapping type B |

Summary for 1st round

Draft CR for TS 38.104:

| Draft CR | Recommendation |
| --- | --- |
| R4-2215997, requirements for PUSCH TBoMS, Huawei | To be revised. |
| R4-2215685, PUSCH JCE for FR1 and FR2, Ericsson | To be revised. |
| R4-2216688, PUCCH JCE requirements, Samsung | To be revised. |
| R4-2215640, FRC for TBoMS and PUSCH JCE, Nokia | Can be endorsed. |

Draft CR for TS 38.141-1:

| Draft CR | Recommendation |
| --- | --- |
| R4-2215686, manufacture declaration, applicability rules and TT, Ericsson | Can be endorsed based on E///’s and CTC’s response. |
| R4-2215998, requirements for PUSCH TBoMS, Huawei | To be revised. |
| R4-2215327, JCE BS performance test for FR1, CTC | To be revised. |
| R4-2215641, Perf requirements for PUCCH JCE, Nokia | To be revised. |
| R4-2216689, FRC for TBoMS and PUSCH JCE, Samsung | To be revised. |

Draft CR for TS 38.141-2:

| Draft CR | Recommendation |
| --- | --- |
| R4-2215687, manufacture declaration, applicability rules and TT, Ericsson | Can be endorsed based on E///’s and CTC’s response. |
| R4-2215999, requirements for PUSCH TBoMS, Huawei | To be revised. |
| R4-2215328, JCE BS performance test for FR2, CTC | To be revised. |
| R4-2215642, Perf requirements for PUCCH JCE, Nokia | To be revised. |
| R4-2216690, FRC for TBoMS and PUSCH JCE, Samsung | To be revised. |

Discussion on 2nd round

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | Big CR for coverage enhancement performance requirements for TS38.104 | China Telecom | Capture all endorsed agreements in the draft CRs for TS38.104. For post meeting agreements. |
|  | Big CR for coverage enhancement performance requirements for TS38.141-1 | Ericsson | Capture all endorsed agreements in the draft CRs for TS38.141-1. For post meeting agreements. |
|  | Big CR for coverage enhancement performance requirements for TS38.141-2 | Nokia | Capture all endorsed agreements in the draft CRs for TS38.141-2. For post meeting agreements. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2215325 |  | Summary of simulation results for PUSCH coverage enhancements | China Telecom | Return to |  |
| R4-2215326 |  | On BS PUSCH demodulation requirements for NR coverage enhancements | China Telecom | Noted |  |
| R4-2215327 |  | Draft CR on PUSCH with DMRS bundling BS performance test for FR1 | China Telecom | Revised |  |
| R4-2215328 |  | Draft CR on PUSCH with DMRS bundling BS performance test for FR2 | China Telecom | Revised |  |
| R4-2215638 |  | PUSCH demodulation performance of Rel-17 NR coverage enhancements | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215639 |  | PUSCH demodulation performance of Rel-17 NR coverage enhancements: simulation results | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215640 |  | draftCR for 38.104: FRC for TBoMS and PUSCH JCE | Nokia, Nokia Shanghai Bell | Endorsed |  |
| R4-2215641 |  | draftCR for 38.141-1: Perf requirements for PUCCH JCE | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2215642 |  | draftCR for 38.141-2: Perf requirements for PUCCH JCE | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2215643 |  | Simulation results collection for coverage enhancement for PUCCH | Nokia, Nokia Shanghai Bell | Return to |  |
| R4-2215685 |  | draftCR for TS38.104 PUSCH with JCE for FR1 and FR2 | Ericsson | Revised |  |
| R4-2215686 |  | draftCR for TS38.141-1 manufacture declaration, applicability rule, MU and TT | Ericsson | Endorsed |  |
| R4-2215687 |  | draftCR for TS38.141-2 manufacture declaration, applicability rule, MU and TT | Ericsson | Endorsed |  |
| R4-2215688 |  | Simulation results for PUSCH with TBoMS and JCE | Ericsson | Noted |  |
| R4-2215689 |  | Simulation results for PUCCH with JCE | Ericsson | Noted |  |
| R4-2215995 |  | Discussion on BS coverage enhancement demod PUSCH | Huawei,HiSilicon | Noted |  |
| R4-2215996 |  | Simulation results on BS coverage enhancement demod PUSCH | Huawei,HiSilicon | Noted |  |
| R4-2215997 |  | Draft CR on requirements for PUSCH TBoMS (TS38.104, Rel-17) | Huawei,HiSilicon | Revised |  |
| R4-2215998 |  | Draft CR on requirements for PUSCH TBoMS (TS38.141-1, Rel-17) | Huawei,HiSilicon | Revised |  |
| R4-2215999 |  | Draft CR on requirements for PUSCH TBoMS (TS38.141-2, Rel-17) | Huawei,HiSilicon | Revised |  |
| R4-2216000 |  | Discussion on BS coverage enhancement demod PUCCH | Huawei,HiSilicon | Noted |  |
| R4-2216001 |  | Simulation results on BS coverage enhancement demod PUCCH | Huawei,HiSilicon | Noted |  |
| R4-2216507 |  | PUCCH demodulation performance of Rel-17 NR coverage enhancements: simulation results | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216686 |  | Simulation results for PUSCH demodulation requirement for Rel-17 coverage enhancement | Samsung | Revised | Samsung would like to update the simulation results in the contribution. |
| R4-2216687 |  | Simulation results for PUCCH demodulation requirement for Rel-17 coverage enhancement | Samsung | Noted |  |
| R4-2216688 |  | Draft CR on PUCCH JCE requirements for TS 38.104 | Samsung | Revised |  |
| R4-2216689 |  | Draft CR on FRC for TBoMS and PUSCH JCE for TS 38.141-1 | Samsung | Revised |  |
| R4-2216690 |  | Draft CR on FRC for TBoMS and PUSCH JCE for TS 38.141-2 | Samsung | Revised |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents