3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2216920

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.7.4

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [104-bis-e][209] NR_IIOT_URLLC_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

Following Issues were left for discussion in the agreed WF (R4-2214338) in RAN4#104 meeting:

- PDC measurement period if PRS measurements occasionally/continuously collide with Type 1A/1B PPW/Type 2 PPW:

- Option 1: If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements if the PRS has higher priority than CSI-RS.

- Option 2: Other.

- how to account for overlap between PDC RS and Measurement Gaps

- Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps.

- Option 2: Allow for additional delay if there is any overlap between PDC RSs and Measurement gaps

- Option 3: Do not define requirements if there is any overlap between PDC RSs and Measurement gaps

- Companies to bring discussion and proposal for next meeting addressing the open issue: How to develop requirements when DRX is in use

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: All 3 Core requirement related Issues will be discussed during 1st round. Additionally, remaining aspects for performance requirements will be discussed.

  - Sub-Topic 1-1: PDC measurement period if PRS measurements occasionally/continuously collide with Type 1A/1B PPW/Type 2 PPW

  - Sub-topic 1-2: How to account for overlap between PDC RS and Measurement Gaps

  - Sub-topic 1-3: How to develop requirements when DRX is in use

- 2nd round: Any open issues not resolved in 1st Round

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Li Zhang | zhangli164@huawei.com |
| Nokia | Lars Dalsgaard | lars.dalsgaard@nokia.com |
| vivo | Qian Yang | qian9.yang@vivo.com |
| Qualcomm | Carlos Cabrera-Mercader | ccmercad@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: RRM core requirement maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215873 | vivo | Proposal 1: If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements. Proposal 2: Additional delay is allowed if there is any overlap between PDC RSs and Measurement gaps. Proposal 3: For PRS/TRS based UE Rx-Tx time difference measurement for TT-based PDC, DRX related requirements are specified with the assumption that UE measures once per DRX cycle. Proposal 4: The requirements when DRX is configured are specified as follows. For TRS based PDC measurements, TUERxTx,TRS=Nsample *max⁡(TTRS, DRX cycle) For PRS based PDC measurements, TUERxTx, PRS=Nsample*max⁡(NPRSslotN'*Teffect, DRX cycle) |
| R4-2216326 | Huawei, HiSilicon | Proposal 1: Allow for additional delay if there is any overlap between PDC RSs and MGs. Proposal 2: If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements. Proposal 3: Define DRX requirements based on assumption that UE is take one sample per DRX cycle. |
| R4-2216422 | Ericsson | Observation 1: As pointed out above, accurate TSN clocks are delivered periodically. Will there be time for DRX sleep? Observation 2: Waking up after DRX is an inaccurate process the Te value alone is much bigger than the whole control-to-control budget of +/- 145-275 ns. Observation 3: Existing positioning requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. Proposal 1: Existing PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. |
| R4-2216508 | Nokia, Nokia Shanghai Bell | - Define requirements including possible collision/overlap between one or more measurement gaps and DL PRS for PDC. - Introduce a scaling factor Kgap, to account for overlap between PDC RS and MG - RAN4 does not define PDC measurement period if PRS measurements occasionally/continuously collide with Type 1A/1B PPW/Type 2 PPW - Define requirements including when DRX is defined and in use. |
| R4-2216721 | Qualcomm Incorporated | Proposal 1: Introduce a scaling factor, like the scaling factor P for L1-RSRP, RLM and BFD measurements, to account for collisions between PDC RS resources and measurement gaps, including concurrent measurement gaps. Proposal 2: The measurement period for PDC can be longer if some of the PDC RS resources collide with Type 1A/1B/2 PPW instances. Proposal 3: PDC measurement requirements do not apply if all the PDC RS resources collide with Type 1A/1B/2 PPW instances. Proposal 4: Define PDC requirements so that one sample per DRX cycle is assumed when DRX is configured. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: PDC measurement period if PRS measurements occasionally/continuously collide with Type 1A/1B PPW/Type 2 PPW

- Proposals

  - Option 1: If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements.

  - Option 2: RAN4 does not define PDC measurement period if PRS measurements occasionally/continuously collide with Type 1A/1B PPW/Type 2 PPW

  - Option 3: PDC measurement requirements do not apply if all the PDC RS resources collide with Type 1A/1B/2 PPW instances.

- Summary:

  - 3 companies support option 1.

  - 1 company support option 2.

  - 1 company support option 3

  - 1 company is neutral.

- Recommended WF

  - More discussion is needed. See if companies can compromise to option 1?

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: how to account for overlap between PDC RS and Measurement Gaps

- Proposals

  - Option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps.

  - Option 2: Allow for additional delay if there is any overlap between PDC RSs and Measurement gaps

  - Option 3: Do not define requirements if there is any overlap between PDC RSs and Measurement gaps.

- Summary:

  - 2 companies support Option 1.

  - 2 companies support Option 2.

  - 1 company is neutral

  - No company support option 3

- Recommended WF

  - Agreement 1: Option 3 is no longer considered.

  - Continue the discussion related to define UE requirements according to Option 1 or Option 2.

### Sub-topic 1-3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3: How to develop requirements when DRX is in use

- Proposals

  - Option 1: Assume 1 sample per DRX cycle.

- Summary:

  - 3 companies support Option 1. No companies have concern.

  - 1 company raise the issue that PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331.

- Recommended WF

  - Agreement 1: Option 1: Define UE requirements assuming 1 sample per DRX cycle.

  - Continue the discussion related to the UE accuracy requirements when DRX is in use.

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Sub topic 1-1

Issue 1-1: PDC measurement period if PRS measurements occasionally/continuously collide with Type 1A/1B PPW/Type 2 PPW

| Company | Comments |
| --- | --- |
| XXX | Moderator summary after 1st round Moderator propose following WF with the additional explanation (which will not be captured in the specification): RAN4 agree on option 1: If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements Comments: This relaxation is listed to clarify that if UE is configured such that PDC RS resources overlap with Type 1A/1B/2 PPW the PDC measurement period may increase and hence measurement results delayed. If configuration is such that there is no overlap between PDC RS resources and Type 1A/1B/2 PPW, no relaxation in PDC measurement period is allowed. Option 3 is regarded as a very special case of overlap between PDC RS resources overlap with Type 1A/1B/2 PPW and is understood to be covered by option 1. |
| Huawei | Support option 1. In our view there is no fundamental difference between the options. Between “longer measurement period” and “no requirements apply”, we think the former is more generic and inclusive. |
| Ericsson | This is assumed to be avoidable by configuration: RAN1#108e Agreement For a UE configured with DL PRS for RTT-based PDC, the UE doesn't expect to be configured/scheduled to receive any other DL channel/signal in the PRBs that overlap those of the DL PRS for PDC in the OFDM symbols occupied by the DL PRS for PDC. - Spec change(s) if any is up to the editor. |
| Nokia | Although we propose option 2 and agree with Ericsson, we can also compromise to option 1. It may not harm much to add such line. Network can then configure the UE such that it is avoided while it is clarified that if overlap occur there may be impact on the measurement period. We agree that option 1 includes option 3 and option 1 should be rather generic. |
| vivo | Option 1. Similar requirements are specified for many cases, e.g., for positioning measurement requirements. Option 2/3 would be the same which mean no requirements to be specified. It would be clearer to have a sentence as option 1 in the spec. |
| Qualcomm | We support option 1 and option 3. The RAN1 agreement above does not fully address the issue. E.g. a UE that configured with PPW type 1A/1B would not be expected to process other lower-priority DL signals that collide with the PPW, even if they are in symbols where PRS is not allocated. Since RAN1 has not explicitly defined the priority of RS for PDC vs. PRS for positioning, RAN4 would prioritize positioning vs. PDC inside PPWs (which is for positioning) by agreeing to option 1 and option 3. |

Sub topic 1-2

Issue 1-2: how to account for overlap between PDC RS and Measurement Gaps

| Company | Comments |
| --- | --- |
| XXX | Moderator summary after 1st round Moderator propose following WF with the additional explanation: RAN4 agree on option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps. Comments: 4 companies can support option 1 while 2 companies have a preference not to select option 2. 1 company support option 2. Moderator agrees that network could likely avoid configuring measurement gaps and PDC RS such that they overlap. However, moderator also think that RAN4 need to consider that in practical deployment such avoidance of overlap may not always be possible. Moderator would like to ask vivo if they can compromise to option 1? |
| Huawei | We are fine with either option 1 or option 2. |
| Ericsson | We prefer option 1 (over option 2) sonce option 1 gives a deterministic measurement period formula. The delay in option 2 “additional delay” has to be specified somhow to be useful. We can not just specify “additional delay”. |
| Nokia | We agree with Ericsson prefer option 1. |
| vivo | Same approach would be applied for PDC RS overlapping/colliding with PPW and with measurement gaps. If scaling factor is introduced to account for overlap between PDC RS and measurement gaps, it also needs to consider defining scaling factor to account for overlap between PDC-RS and SMTC which is used for intra-frequency measurement. The intra-frequency measurement requirements are also needed to be revised to account for the overlapping/collision. In addition, collision between SMTC and measurement gap would also need to be further considered. So, option 2 is preferable and it is left to UE implementation how to handle the overlap/collision if it happens. Preference is that NW avoid such collision configuration because PDC RS measurement is only serving cell measurement. In our understanding, even with scaling factor, the measurement delay is not deterministic unless comprehensive requirements are specified by taking all possible cases into account. |
| Qualcomm | Prefer option 1. |

Sub topic 1-3

Issue 1-3: How to develop requirements when DRX is in use

| Company | Comments |
| --- | --- |
| XXX | Moderator summary after 1st round Moderator propose following WF with the additional explanation: RAN4 agree on option 1: RAN4 will develop PDC measurement delay requirements for UE Rx-Tx time difference based on the assumption that UE perform 1 sample per DRX cycle. Comments: Additionally, moderator would like to hear if anyone in the group have concern with following agreement: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. |
| Huawei | We support option 1. We are also fine with the proposal that the same accuracy requirements apply for both non-DRX and DRX case. |
| Ericsson | Option 1. |
| Nokia | Option 1 would be aligned with what RAN4 usually assume. We are also supporting that the same accuracy requirements applies for DRX and no-DRX case. |
| vivo | Agree with recommended WF. |
| Qualcomm | Option 1 |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216327, CR on requirements for UE Rx-Tx measurement for PDC, Huawei, HiSilicon | Ericsson. Needs to be merged with Nokia CR R4-2216509. We also need to agree on PDC/PPW and PDC/MG overlap. |
|  | vivo: Not clear why LCM is used. |
|  | Qualcomm: Pending issues 1-1, 1-2 and 1-3. Overlaps with R4-2216509. - LCM(TTRS,TDRX) -> this would be applicable only when DRX is configured |
| R4-2216423, Requirements for DRX case, Ericsson | Qualcomm: Pending issue 1-3. |
|  | Company B |
| R4-2216509, CR on requirements for NR_IIOT_URLLC, Nokia, Nokia Shanghai Bell | Huawei: For DRX requirements, would it be more straightforward to follow the same way as in existing RRM requirements, i.e. by using MAX or LCM between RS period (TPRS or TTRS) and DRX cycle (TDRX)? |
|  | Ericsson: Needs to be merged with Huawei CR R4-2216509. We also need to agree on PDC/PPW and PDC/MG overlap. |
|  | vivo: Too complicated requirements. Prefer to use vivo or Huawei’s format. |
|  | Qualcomm: Pending issues 1-1, 1-2 and 1-3. Overlaps with R4-2216327 |
| R4-2216672, CR to TS 38.133 Correction to measurements core requirements for PDC, vivo | Huawei: For PRS measurement requirements, the MAX should apply to Teffect and DRX cycle. |
|  | Ericsson: Needs to be merged with Huawei CR R4-2216509, and Noia R4-2216509. We also need to agree on PDC/PPW and PDC/MG overlap. |
|  | vivo: @Huawei, agree. It should be max (Teffect, DRX cycle) |
|  | Qualcomm: Pending issues 1-1, 1-2 and 1-3. Overlaps with R4-2216327. For PRS based PDC measurements, TUERxTx, PRS=Nsample*max⁡(NPRSslotN'*Teffect, DRX cycle) Same comment as Huawei, the term NPRSslotN' should be outside the max().Also, should it be max() or LCM()? We think it should be LCM since the measurement should be done during DRX ON period (and SRS is not expected to be transmitted elsewhere). |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 | Moderator summary after 1st round Moderator propose following WF with the additional explanation (which will not be captured in the specification): RAN4 agree on option 1: If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurements Comments: This relaxation is listed to clarify that if UE is configured such that PDC RS resources overlap with Type 1A/1B/2 PPW the PDC measurement period may increase and hence measurement results delayed. If configuration is such that there is no overlap between PDC RS resources and Type 1A/1B/2 PPW, no relaxation in PDC measurement period is allowed. Option 3 is regarded as a very special case of overlap between PDC RS resources overlap with Type 1A/1B/2 PPW and is understood to be covered by option 1. Agreements: Yes If PDC RS resources overlap with Type 1A/1B/2 PPW the UE is allowed longer measurement period for PDC measurementsCandidate options: No Recommendations for 2nd round: Sub-topic is closed. |
| Sub-topic #1-2 | Moderator summary after 1st round Moderator propose following WF with the additional explanation: RAN4 agree on option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps. Comments: 4 companies can support option 1 while 2 companies have a preference not to select option 2. 1 company support option 2. Moderator agrees that network could likely avoid configuring measurement gaps and PDC RS such that they overlap. However, moderator also think that RAN4 need to consider that in practical deployment such avoidance of overlap may not always be possible. Moderator would like to ask vivo if they can compromise to option 1? Tentative agreements: Yes Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps. Candidate options: Recommendations for 2nd round: Companies which have a technical concern may raise the concern. Moderator suggest to include the tentative agreement into the CR drafting. |
| Sub-topic #1-3 | RAN4 agree on option 1: RAN4 will develop PDC measurement delay requirements for UE Rx-Tx time difference based on the assumption that UE perform 1 sample per DRX cycle. Comments: Additionally, moderator would like to hear if anyone in the group have concern with following agreement: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. Agreement: yes RAN4 will develop PDC measurement delay requirements for UE Rx-Tx time difference based on the assumption that UE perform 1 sample per DRX cycle. Candidate options: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. Recommendations for 2nd round: Companies to raise concern if they cannot agree to following: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2216327, CR on requirements for UE Rx-Tx measurement for PDC, Huawei, HiSilicon | merged (into R4-2216672) |
| R4-2216423, Requirements for DRX case, Ericsson | merged (into R4-2216672) |
| R4-2216509, CR on requirements for NR_IIOT_URLLC, Nokia, Nokia Shanghai Bell | merged (into R4-2216672) |
| R4-2216672, CR to TS 38.133 Correction to measurements core requirements for PDC, vivo | Revised |

## Discussion on 2nd round (if applicable)

| Sub-topic #1-2 | Moderator summary after 1st round Moderator propose following WF with the additional explanation: RAN4 agree on option 1: Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps. Comments: 4 companies can support option 1 while 2 companies have a preference not to select option 2. 1 company support option 2. Moderator agrees that network could likely avoid configuring measurement gaps and PDC RS such that they overlap. However, moderator also think that RAN4 need to consider that in practical deployment such avoidance of overlap may not always be possible. Moderator would like to ask vivo if they can compromise to option 1? Tentative agreements: Yes Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps. Candidate options: Recommendations for 2nd round: Companies which have a technical concern may raise the concern. Moderator suggest to include the tentative agreement into the CR drafting. |
| --- | --- |
| Sub-topic #1-3 | RAN4 agree on option 1: RAN4 will develop PDC measurement delay requirements for UE Rx-Tx time difference based on the assumption that UE perform 1 sample per DRX cycle. Comments: Additionally, moderator would like to hear if anyone in the group have concern with following agreement: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. Agreement: yes RAN4 will develop PDC measurement delay requirements for UE Rx-Tx time difference based on the assumption that UE perform 1 sample per DRX cycle. Candidate options: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. Recommendations for 2nd round: Companies to raise concern if they cannot agree to following: PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331. |

## Companies views’ collection for 2nd round

Sub topic 1-2

Issue 1-2: Companies which have a technical concern with the tentative agreement may raise the concern:

Introduce a scaling factor Kgap, to account for overlap between PDC RS and Measurement gaps

| Company | Comments |
| --- | --- |

Sub topic 1-3

Issue 1-3: Companies to raise concern if they cannot agree to following:

PDC accuracy requirements for UE Rx-Tx time difference apply without DRX as well as for any DRX configuration specified in TS 38.331

| Company | Comments |
| --- | --- |

# Topic #2: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216510 | Nokia, Nokia Shanghai Bell | Proposal 1: Apply averaging over companies’ TRS measurement accuracy simulation results. Proposal 2: Adopt the TRS measurement accuracy requirements in Table 2 and Table 3 addition with the group delay defined in TS 38.133 – 10.1.25.2. |
| R4-2216328 | Huawei, HiSilicon | Proposal 1: Use the average value in Table 1 for defining the accuracy number for TRS based PDC. Proposal 2: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC. |
| R4-2216722 | Qualcomm Incorporated | Proposal 1: Rel-16 UE Rx-Tx accuracy requirements that were derived assuming a sampling rate higher than 32∙Tc do not apply to RTT-based PDC using PRS as the DL reference signal. Proposal 2: Simulation results assuming sampling rates higher than 32∙Tc will not be used to define measurement accuracy requirements for RTT-based PDC using TRS as the DL reference signal. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: TRS measurement accuracy requirements

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Summary:

  - Two companies have provided averaged results based on the simulation results provided in earlier meetings by companies. For most scenarios the TUE-RX accuracy with TRS, 4 samples are similar except for 30 and 60KHz SCS in FR1. Additionally, some difference in the 60KHZ and 120KHz SCS results for FR2.

  - FR1:

    - Difference between the results seems to come from using different results from Nokia where R4-2216328 use [88, 68, 40, 64, 40, 32] and R4-2216510 use [32.0, 16.0, 8.0, 16.0, 8.0, 4.0] for Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

    - Other averaged results are similar

  - FR2:

    - Difference in the averaged results for 120KHz SCS with TRS BW of 64 and 128 RBs. Difference seems to from the averaging.

- Recommended WF

  - Agree on following tables with the FR2 120KHz SCS with TRS BW of 64 and 128 RBs in []:

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 15 | 24 | 103 |
|  | 52 | 53 |
|  | 104 | 26 |

    - Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 60 | 24 | 26 |
|  | 64 | 13 |
|  | 132 | 7 |
| 120 | 32 | 13 |
|  | 64 | [6, 7] |
|  | 128 | [3, 4] |

    - Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR2

  - Further discuss and agree on the TUE-RX accuracy for FR2 with 120KHz SCS with TRS BW of 64 and 128 RBs

  - Further clarify the differing results and agree on the averaged results marked FFS in the following table:

|  |  | TUE-RX accuracy with TRS, 4 samples, AWGN, TDD |
| --- | --- | --- |
|  |  | Accuracy (Tc) |
| SCS [KHz] | TRS bandwidth RB | Average |
| 30 | 24 | FFS |
|  | 48 | FFS |
|  | 132 | FFS |
| 60 | 24 | FFS |
|  | 64 | FFS |
|  | 132 | FFS |

    - Average TUE-RX accuracy with TRS, 4 samples, AWGN, TDD, FR1

### Sub-topic 2-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2: Adopt the TRS measurement accuracy requirements in Table 2 and Table 3 addition with the group delay defined in TS 38.133 – 10.1.25.2.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion

### Sub-topic 2-3

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-3: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - More discussion

### Sub-topic 2-4

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-4: Rel-16 UE Rx-Tx accuracy requirements that were derived assuming a sampling rate higher than 32∙Tc do not apply to RTT-based PDC using PRS as the DL reference signal.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Question from moderator: If this proposal is agreed, does this mean RAN4 need to a new round of simulations?

  - More discussion needed.

### Sub-topic 2-5

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-5: Simulation results assuming sampling rates higher than 32∙Tc will not be used to define measurement accuracy requirements for RTT-based PDC using TRS as the DL reference signal.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Question from moderator: If this proposal is agreed, does this mean RAN4 need to a new round of simulations?

  - More discussion

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

Issue 2-1: TRS measurement accuracy requirements

| Company | Comments |
| --- | --- |
| XXX | Moderator Summary: Based on comment from companies we will return to this during meeting or next meeting once results have been verified. |
| Huawei | For FR2 with 120KHz SCS with TRS BW of 64 and 128 RBs, suggest to use 7 and 4. For FR1 remaining cases, more discussion is needed. It is fine if companies would like to update their results and we should take the latest results into account. However, it seems the Nokia’s new results in 6510 for the FR1 remaining cases are almost half of the results from other companies. Since it is for AWGN channel, the accuracy would be somehow predictable based on the TRS BW, so we are wondering if Nokia’s new results are based on over-sampling. It is noted that in Rel-16 the accuracy requirements are derived based on nominal sampling rate and no over-sampling was assumed. |
| Ericsson | The WF is fine. The Nokia results in R4-2213775 refered to in 6510 are stricter by a factor of 2 almost compared to other companies. We see the same thing as Huawei |
| Nokai | We will take a look at our results and come back ether this meeting or next. |
| Qualcomm | Same comment as Huawei regarding the FR1 results from Nokia. The oversampling rate assumption needs to be confirmed. Also, before agreeing on the requirements, sub-topics 2-4 and 2-5 need to be addressed. |

Sub topic 2-2

Issue 2-2: Adopt the TRS measurement accuracy requirements in Table 2 and Table 3 addition with the group delay defined in TS 38.133 – 10.1.25.2.

| Company | Comments |
| --- | --- |
| XXX | Moderator summary: based on the discussion there is no need to discuss this further as it has already been agreed in RAN4#103 meeting |
| Huawei | Not sure if we need to have explicit agreement on this proposal. We agree that the accuracy will be based on sum of BB error and RF calibration error. The BB error is discussed in issue 2-1, and for RF calibration error it is straightforward to re-use the Rel-16 requirements. |
| Ericsson | Option 1, yes, is OK in principle, but we need settle table 2 and 3 fist, ie the TRX UE Rx-Tx delay accuracy. |
| Nokia | Option 1. This has been agreed in RAN4#103, no need for further discussion. |
| Qualcomm | Not sure if an agreement is needed here. We understand it was agreed earlier to reuse the group delay calibration margins from Rel-16 NR positioning. |

Sub topic 2-3

Issue 2-3: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC.

| Company | Comments |
| --- | --- |
| XXX | Moderator summary: All companies can support option 1. Moderator propose following agreement based on agreeing to option 1: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC |
| Huawei | This is our proposal and we support it. In Rel-16 spec, the BB and calibration error are already captured in separate tables, and we should follow the same approach for PDC Rx-Tx measurement. |
| Ericsson | Option 1, Yes. |
| Nokia | Option 1. |
| Qualcomm | Option 1. Follow the same approach as for NR positioning. |

Sub topic 2-4

Issue 2-4: Rel-16 UE Rx-Tx accuracy requirements that were derived assuming a sampling rate higher than 32∙Tc do not apply to RTT-based PDC using PRS as the DL reference signal.

| Company | Comments |
| --- | --- |
| XXX | Moderator summary: No agreement in first round. More discussion in 2nd round |
| Huawei | We do not see clear need for the proposed limit. In our view, the accuracy and reporting granularity are different. Using a granularity of 32Tc does not mean UE cannot achieve accuracy on the level of 4Tc, as we assume the sampling rate used by the UE for measurement does not depend on the granularity to be used for the reporting. |
| Ericsson | Same view as Huawei. The accuracy and reporting granularity are different. The sampling rate used by the UE for measurement does not depend on the granularity to be used for the reporting |
| Nokia | Same view as Huawei and Ericsson. |
| Qualcomm | After the discussion in GTW, it seems many companies agree that there is an issue. If the measurements are quantized with step size of 32*Tc when they are reported, then the achievable measurement accuracy will be limited by the quantization step size. Most of the potential improvement in baseband accuracy from using RS bandwidth higher than ~61.44 MHz (=1/(32*Tc)) will be lost. Our original proposal to address the issue is to cap the accuracy requirements by ignoring any results for PRS BWs that assumed a sampling rate higher than 61.44 MHz. e.g. in the table below the highlighted entries would be removed. Then for FR2 with SCS=120 kHz, all PRS BW ≥ 32 RB would have the same accuracy requirement. We’re open to discussing other ways to address this issue. FR; PRS BW (PRBs); SCS (kHz); sample rate(Tc); Num. samples; Accuracy [90%] (Tc). SINR = -3 dB. FR2; ≥32; 120; 32; 4; X1. ≥64; 16; 4; X2. ≥128; 8; 4; X3. ≥24; 60; 64; 4; X4. ≥64; 32; 4; X5. ≥132; 16; 4; X6. |

Sub topic 2-5

Issue 2-5: Simulation results assuming sampling rates higher than 32∙Tc will not be used to define measurement accuracy requirements for RTT-based PDC using TRS as the DL reference signal.

| Company | Comments |
| --- | --- |
| XXX | Moderator summary: No agreement in first round. More discussion in 2nd round |
| Huawei | Similar comment as for issue 2-4. |
| Ericsson | Similar comment as for issue 2-4. |
| Nokia | Similar comment as for issue 2-4. |
| Qualcomm | Similar response as for issue 2-4. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216511, CR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC, Nokia | Huawei: The BB error depends on issue 2-1, and whether to capture BB and RF error in separate tables is discussed in issue 2-3. |
|  | Qualcomm: Merge with R4-2216329. Separate baseband accuracy and GD margin in different tables |
| R4-2216792, Draft CR to verify measurements for UE Rx-Tx time difference measurement with TRS for RTT based PDC in FR2 SA, Nokia | Qualcomm: Overlaps with R4-2216330 |
|  | Company B |
| R4-2216329, CR on PDC measurement accuracy requirements, Huawei | Ericsson: Merge with Nokia R4-2216511, pending how we settle average of input simulation results for table of accuracy. |
|  | Qualcomm: Agree with separation of baseband accuracy and group delay margin into different tables. There is a typo in the reference to the table for GD margins for FR2. Baseband accuracy numbers subject to agreement. Some of Nokia’s results seem to be off (different from Nokia’s paper). Also, measurement reporting granularity (issues 2-4, 2-5) needs to be addressed. |
| R4-2216330, CR on TCs for PDC measurement, Huawei | Ericsson: Merge with Nokia R4-2216792 |
|  | Qualcomm: Overlaps with R4-2216792. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Moderator Summary: Based on comment from companies we will return to this during meeting or next meeting once results have been verified. Tentative agreements: No Candidate options: Recommendations for 2nd round: If new results are provided, continue the discussion. Otherwise, come back next meeting. |
| Sub-topic#2-2 | Moderator summary: based on the discussion there is no need to discuss this further as it has already been agreed in RAN4#103 meeting Tentative agreements: No Candidate options: No Recommendations for 2nd round: Topic is closed |
| Sub-topic#2-3 | Moderator summary: All companies can support option 1. Moderator propose following agreement based on agreeing to option 1: Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC Agreements: Yes Capture BB and RF error in the separate tables in accuracy requirements for UE Rx-Tx for PDC Candidate options: No Recommendations for 2nd round: topic is closed |
| Sub-topic#2-4 | Moderator summary: No agreement in first round. More discussion in 2nd round Tentative agreements: No Candidate options: Issue 2-4: Rel-16 UE Rx-Tx accuracy requirements that were derived assuming a sampling rate higher than 32∙Tc do not apply to RTT-based PDC using PRS as the DL reference signal Recommendations for 2nd round: Continue discussion in 2nd round |
| Sub-topic#2-5 | Moderator summary: No agreement in first round. More discussion in 2nd round Tentative agreements: No Candidate options: Issue 2-5: Simulation results assuming sampling rates higher than 32∙Tc will not be used to define measurement accuracy requirements for RTT-based PDC using TRS as the DL reference signal Recommendations for 2nd round: Continue discussion in 2nd round |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2216511, CR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC, Nokia | Merge |
| R4-2216792, Draft CR to verify measurements for UE Rx-Tx time difference measurement with TRS for RTT based PDC in FR2 SA, Nokia | to be revised. Merge 6330 and 6792 |
| R4-2216329, CR on PDC measurement accuracy requirements, Huawei | to be revised. Merge 6511 and 6329 |
| R4-2216330, CR on TCs for PDC measurement, Huawei | Merge |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

| Sub-topic#2-1 | Moderator Summary: Based on comment from companies we will return to this during meeting or next meeting once results have been verified. Tentative agreements: No Candidate options: Recommendations for 2nd round: If new results are provided, continue the discussion. Otherwise, come back next meeting. |
| --- | --- |
| Sub-topic#2-4 | Moderator summary: No agreement in first round. More discussion in 2nd round Tentative agreements: No Candidate options: Issue 2-4: Rel-16 UE Rx-Tx accuracy requirements that were derived assuming a sampling rate higher than 32∙Tc do not apply to RTT-based PDC using PRS as the DL reference signal Recommendations for 2nd round: Continue discussion in 2nd round |
| Sub-topic#2-5 | Moderator summary: No agreement in first round. More discussion in 2nd round Tentative agreements: No Candidate options: Issue 2-5: Simulation results assuming sampling rates higher than 32∙Tc will not be used to define measurement accuracy requirements for RTT-based PDC using TRS as the DL reference signal Recommendations for 2nd round: Continue discussion in 2nd round |

## Companies views’ collection for 2nd round

Sub topic 2-4

Issue 2-4: Rel-16 UE Rx-Tx accuracy requirements that were derived assuming a sampling rate higher than 32∙Tc do not apply to RTT-based PDC using PRS as the DL reference signal.

| Company | Comments |
| --- | --- |
| XXX |  |

Sub topic 2-5

Issue 2-5: Simulation results assuming sampling rates higher than 32∙Tc will not be used to define measurement accuracy requirements for RTT-based PDC using TRS as the DL reference signal.

| Company | Comments |
| --- | --- |
| XXX |  |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR_IIOT_URLLC_enh | Moderator (Nokia) | Capture agreements and open issues |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2216327 |  | CR on requirements for UE Rx-Tx measurement for PDC | Huawei, HiSilicon | Merged | merged into 6672 |
| R4-2216423 |  | Requirements for DRX case | Ericsson | Revised |  |
| R4-2216509 |  | CR on requirements for NR_IIOT_URLLC | Nokia, Nokia Shanghai Bell | Merged | merged into 6672 |
| R4-2216672 |  | CR to TS 38.133 Correction to measurements core requirements for PDC | vivo | Revised |  |
| R4-2216511 |  | CR on UE Rx-Tx time difference measurement accuracy requirements for RTT-based PDC | Nokia, Nokia Shanghai Bell | Merged | merged into 6329 |
| R4-2216792 |  | Draft CR to verify measurements for UE Rx-Tx time difference measurement with TRS for RTT based PDC in FR2 SA | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2216329 |  | CR on PDC measurement accuracy requirements | Huawei, HiSilicon | Revised |  |
| R4-2216330 |  | CR on TCs for PDC measurement | Huawei, HiSilicon | Merged | merged into 6792 |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | WF on NR_IIOT_URLLC_enh | Moderator (Nokia) | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | Requirements for DRX case | Ericsson | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | CR to TS 38.133 Correction to measurements core requirements for PDC | vivo | Agreeable, Revised, Noted |  |
|  |  | Draft CR to verify measurements for UE Rx-Tx time difference measurement with TRS for RTT based PDC in FR2 SA | Nokia, Nokia Shanghai Bell |  |  |
|  |  | CR on PDC measurement accuracy requirements | Huawei, HiSilicon |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents