3GPP TSG-RAN WG4 Meeting # 104bis-e												R4-2216928

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.10.4

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [104-bis-e][217] NR_MG_enh2_part2

Document for:	Information

# Introduction

This document is the email discussion summary for [104-bis-e][217] NR_MG_enh2_part2 with the following topics covered

- Topic 1:	 Measurement without gaps for UEs reporting NeedForGapsInfoNR (AI 6.10.3.1)

- Topic 2:	 Inter-RAT measurement without gap (AI 6.10.3.2)

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Collect views from companies. Make early decision on issues with clear consensus. Decide on the scope, priority, options and tentative agreement to be discussed in the 2nd round.

- 2nd round:

  - Conclude the issues identified in the 1st round.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Huawei | Li Zhang | zhangli164@huawei.com |
| Ericsson | Zhixun Tang | Zhixun.tang@ericsson.com |
| ZTE | Chenchen Zhang | zhang.chenchen@zte.com.cn |
| Qualcomm | Hyunwoo Cho | hyuncho@qti.qualcomm.com |
| Xiaomi | Ziquan Hu | huziquan@xiaomi.com |
| OPPO | Jinyu Zhang | zhangjinyu@oppo.com |
| Intel | Rui Huang | Rui.huang@intel.com |
| vivo | Xusheng wei | Xusheng.wei@vivo.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| Nokia | Rafael Paiva | Rafael.paiva@nokia.com |
| MediaTek | Waseem Ozan | Waseem.ozan@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Measurement without gaps for UEs reporting NeedForGapsInfoNR (AI 6.10.3.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215368 | Intel Corporation | Observation 1: When UE indicates NeedForNCSG-NR the exact interruption requirements can be conducted. Observation 2: When UE indicates “no-gap” in NeedForGapInfoNR , it is ambiguous that whether neither NCSGs nor legacy measurement gaps are be configured by NW. Observation 3: How to define the interruption requirements need RAN2 further clarification on the indication of “no-gap” in NeedForGapsNR message (e.g. whether it is consistent with that in “NeedForNCSG-NR”). Proposal 1: The interruption requirements when UE performing SSB measurements without gap by reporting ‘NeedForGapsInfoNR' can be defined as: NW config UE capability; Case a: No MG. Gap; No requirements. no-gap; Measurements out of gap, interruption allowed and interruption requirements defined in TS38.133 9.1.9[3]. nogap-noncsg; Measurements out of gap, No interruption allowed. Proposal 2: When UE performing SSB measurement with the different “NeedForGapsInforNR” indications, the different RRM requirements of CSSF shall be defined in Rel18. Proposal 3: Take requirements NCSG requirements as a starting point(9.3.10 in TS38.133[4]) to define the measurement reporting delay requirements for the measurement without gap as UE reporting “NeedForGapInfoNR. |
| R4-2215427 | CATT | Requirement for intra-freq measurement without gap Proposal 1: For both intra-frequency and inter-frequency measurement, reporting ‘no-gap’ through NeedForGapsInfoNR means measurement without gap and without interruption. Proposal 2: For intra-frequency measurement, the measurement requirements and scheduling restriction for the case when UE report ‘no-gap’ follows the same requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap), and no other requirements are needed. Requirement for inter-freq measurement without gap Proposal 3: For inter-frequency measurement, the case when UE report ‘no-gap’ through NeedForGapsInfoNR should also be defined as inter-frequency measurement without gap, and requirements in Section 9.3.9 of TS38.133 (inter-freq wo/ gap) is taken as a starting point (at least for cell identification and measurement period, scheduling restriction can FFS) for further discussions. Proposal 4: CSSF in section 9.1.5 need to be updated to include inter-frequency measurement without gap for the case when UE report ‘no-gap’ through NeedForGapsInfoNR. |
| R4-2215467 | Xiaomi | Proposal 1: RAN4 to consider following scenarios where UE is capable of reporting ‘no gap’ via NeedForGapsInfoNR: - Another spare RF chain is available for UE; - The target SSB to be measured is with UE’s active RF chain. Proposal 2: For the interruption requirement when UE support NeedForGaps: Option 1: Interruption is always expected when UE reports ‘no gap’; Option 2: RAN4 to introduce another UE indication for ‘no gap no interruption’ besides the existing ‘no gap’. Proposal 3: If the interruption is allowed, RAN4 to take NCSG as a starting point. Proposal 4: The current requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) could be reused for UE indicating ‘no-gap’ via intraFreq-needForGap. Proposal 5: For the requirement of inter-freq measurement without gap when UE supporting NeedForGaps capability and indicating ‘no-gap’ via interFreq-needForGap, the requirement of Rel-16 inter-frequency without gap could be used as baseline. |
| R4-2215611 | Apple | Observation 1: in existing NeedForGap procedure, when UE indicates “no-gap” on target band, UE may or may not cause interruption when measuring target band. Proposal 1: consider the following two options on interruption design for NeedForGap - Option 1: interruption is always allowed for “no-gap” - Option 2: introduce additional UE capability to differentiate whether UE needs interruption Proposal 2: interruption length in NeedForGap (if allowed) is same as that defined in NCSG, i.e. 1ms in FR1 and 0.75ms in FR2. Proposal 3: for measurement on band(s) with no interruption, existing requirements of intra-frequency measurement without gap and inter-frequency measurement without gap shall still apply. Proposal 4: for measurement on band(s) with interruption, when measurement gap is not configured, UE needs to align interruption location (by align the time location when to switch on/off the RF chain and baseband resource) on all the carriers even though the SMTC length is different on different carriers. Proposal 5: for measurement on band(s) with interruption, when measurement gap is configured: - If the interruption length and SMTC to be measured on all carriers from the bands on which UE indicates ‘no-gap’ can be fully covered by measurement gap, UE shall measure all the carriers within gap. No additional interruption is allowed outside measurement gap. - If the SMTC to be measured on all carriers from the bands on which UE indicates ‘no-gap’ can NOT be fully covered by measurement gap, introduce network signaling to indicate whether UE shall measure the band(s) with ‘no-gap’ with or without measurement gap (similar to interFrequencyConfig-NoGap-r16). |
| R4-2215715 | CMCC | Observation 1: according to TS 36.133, if UE is capable of interFreqNeedForGaps or interRATNeedForGaps, the measurement is conducted without gaps and without interruption. Observation 2: for intra-frequency measurement without gaps when UE indicates ‘no-gap’ via intraFreq-needForGap, the existinng cell identification requirements, measurement period requirements, and scheduling availability specified in TS 38.133 9.2.5 can be resued. Proposal 1: it is proposed that interruption is not allowed when UE reporting ‘NeedForGapsInfoNR'. Proposal 2: for intra-frequency measurement without gaps when UE indicates ‘no-gap’ via intraFreq-needForGap, existing requirements on intra-frequency measurement without gaps can be reused (e.g. 9.2.5 in TS 38.133) Proposal 3: it is proposed to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap. The detailed update is proposed as following: TS 38.133 A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) for UE capable of interFrequencyMeas-NoGap provided - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. A measurement is defined as inter-frequency measurement without gaps if the UE indicates ‘no-gap’ via interFreq-needForGap for inter-frequency measurement. For UE supporting ncsg-MeasGapNR-r17 and indicating NeedForNCSG-InfoNR for inter-frequency measurement, - An inter-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoNR for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE. Proposal 4: for inter-frequency SSB based measurements without measurement gaps when UE indicates ‘no-gap’ via interFreq-needForGap, it is proposed to take 9.3.10.3 as baseline to define scheduling availability。 Proposal 5: for inter-frequency measurement without gaps when UE indicates ‘no-gap’ via interFreq-needForGap, it is proposed to specify cell identification (including PSS/SSS detection and time indec detection) and measurement period requirements by updating exsiting requirements (taking 9.3.4 or 9.3.10 as baseline). |
| R4-2215822 | OPPO | Proposal-1: Support option 3: Introduce additional UE capability to differentiate whether UE needs interruption. Proposal-2: Take NCSG as a starting point to define the interruption: - For interruption length, reuse VIL - For interruption location, use the boundary of SMTC as the starting/ending pointing Proposal-3: To define requirements for measurement without gap: - Measurement delay requirements: - For intra-frequency: take intra-frequency without gap in section 9.2.5 as a starting point - For inter-frequency: take inter-frequency without gap in section 9.3.9 as a starting point - Scheduling availability requirements: take NCSG as a starting point |
| R4-2215967 | vivo | Proposal 1: Prefer use option 3 for issue 3-1 and the UE capabilities could be {gap, no-gap, no-gap-no-interruption} Proposal 2: For the interruption length, requirements defined at Rel-17 NCSG could be used as a base where interruption length is defined with the unit of number of interrupted slots of the serving cells. Proposal 3: CSSFoutside_gap should be updated, similar methodology of Rel-16 inter-frequency measurement without gap WI on how to update CSSFoutside_gap can be used as the baseline. Proposal 4: For the concrete RRM requirements, the framework of current specification on the inter-frequency measurement performance should be reused. |
| R4-2216337 | Huawei, HiSilicon | Proposal 1: Interruption is not allowed for UE reporting ‘no-gap’ for NeedForGapsInfoNR. Proposal 2: If interruption is allowed, define the interruption requirements to enable visible interruption. Take NCSG as a starting point. Proposal 3: If interruption is allowed when UE reports ‘no-gap’, the interruption should be allowed for each of intra- and inter-frequency measurements for which UE reports ‘no-gap’. - The interruption will impact all the serving cells if UE does not support per-FR gap, and all the serving cells in the same FR as the measurement if UE supports per-FR gap. Proposal 4: The requirements for the case when UE reports ‘no-gap’ are - For intra-frequency: take requirements in Section 9.2.5 of TS38.133 as starting point - For inter-frequency: take requirements in Section 9.3.9 of TS38.133 as starting point, and the sample number should be 8 |
| R4-2216461 | Ericsson | Observation 1: When UE reports ‘no gap’ in a band, it implies the UE uses a spare RF chain to perform the related measurements in this band without gap. Observation 2: The total interruption ratio can be controlled by VIRP and ML in NCSG. Observation 3: Deactivated SCell measurement requirement is defined without gap but with interruption ratio. Proposal 1: The UE’s implementation behaviours are the same for NeedForGaps capability and NCSG capability which is to use a spare RF chain to perform the measurements. Proposal 2: When UE reports ‘no gap’ in NeedForGaps, the additional interruption due to RF switching before and after the measurement occasions may be expected. Proposal 3: The gap status indication in NeedForGaps should have 1-to-1 mapping with the gap status in NCSG if UE supports both NeedForGaps and NCSG capabilities. - UE should report ‘no gap’ in the same band for NeedForGaps if reporting ‘no gap no interruption’ or ‘no gap no interruption’ in a band for NCSG - UE should report ‘gap’ in the same band for NeedForGaps if reporting ‘gap’ in a band for NCSG Proposal 4: Different as NCSG with dedicated pattern, only define interruption length cannot control the total interruption for NeedForGaps capability. Proposal 5: RAN4 to further discuss how to control the total interruption ratio for NeedForGaps. Proposal 6: RAN4 cannot follow NCSG to define NeedForGaps’ measurement requirement since no pattern design for NeedForGaps. Proposal 7: RAN4 cannot follow intra-frequency measurement without gap to define NeedForGaps’ measurement requirement since it will result in unacceptable interruption ratio in the system. Proposal 8: The frequency layers in the band for which UE reports ‘no gap’ should be counted in CSSF outside gap. Proposal 9: The deactivated SCell measurement requirement which has a good control in total interruption ratio can be the start point to define the NeedForGaps’ measurement requirement. Proposal 10: RAN4 to further discuss UE’s behaviour in the following mismatch scenarios - Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps - Rel-16 UE which supports NeedForGaps in a Rel-17 NW which supports NCSG - Both UE and NW support NCSG and NeedForGaps |
| R4-2216484 | ZTE Corporation | Observation 1: In Rel-16, RRM requirements for UEs supporting NeedForGaps feature are specified for intra-frequency SSB based measurements without gaps, while requirements for inter-frequency measurements without gaps are missing. Proposal 1: Similar as the case of intra-f measurement without gap, the condition of inter-f measurement without gap should be added into Section 9.3.1 in TS38.133. Proposal 2: The switching from ‘gap’ to ‘no-gap’ would be accompanied by the change of UE RF BW and/or MO, so interruption can be allowed when UE reports ‘no-gap’ through NeedForGap. Proposal 3: Regarding to the length of interruption, directly reusing the length of VIL in NCSG is fine. During Rel-17, we had discussed the requirements of NCSG wildly and deeply including the length of VIL. Proposal 4: Given that the new interruption mechanism for NeedForGap can be covered by NCSG requirements, the requirements identified in NCSG can be a baseline for this Rel-18 WI. |
| R4-2216583 | Nokia, Nokia Shanghai Bell | - Assume for measurements without gaps target architecture without idle receiver is available. Need for gaps include “gap and “no-gap”. Additional interruption might be needed even if gaps are not needed. - Need for NCSG gaps include “gap”, “ncsg”, “nogap-noncsg”. - A UE not capable of NCSG might not need gap(s) but some interruption. If deriveSSBIndexFromCell-inter and deriveSSBIndexFromCell are signaled by the network, then the effective measurement duration can be reduced. - RAN4 to discuss if new signaling would be needed for needForGaps. - RAN4 to study feasible interruption times for measurement requirements without gaps. - A gNB supporting Rel-17 and Rel-16 interprets UEs signalling ‘no-gap’ and ‘nogap-noncsg’ as having no additional interruption. - A UE that requires interruption for performing measurements is not expected to signal ‘no-gap’ and ‘nogap-noncsg’ in Re-17. - Any change on the meaning of ‘no-gap’ and ‘nogap-noncsg’ in Rel-18 UEs would not be supported by Rel-17 gNBs. - Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2. - If interruption is needed for a UE without gaps, it should be indicated using new indication as part of needForGaps, needForGapsNCSG or a new information element. - Intra-frequency measurements are typically performed without measurement gaps. - Intra-frequency measurements do not need retuning. - For intra-frequency measurement requirements use requirements in 38.133, clause 9.2.5 as a starting point. - RAN4 to limit scope of the requirements without gaps with additional interruption to inter-frequency scenarios. - For inter-frequency measurement requirements use requirements in 38.133, clause 9.3.9 as a starting point. - Consider smaller interruption length than VIL1+VIL2 from NCSG for a UE that requires additional interruptions for measurements without gaps. |
| R4-2216738 | MediaTek inc. | Observation 1: Existing Rel-16 NeedForGap requirements are not complete regarding the need for interruption and its details. Proposal 1: RAN4 shall allow interruption for the Rel-16 NeedForGap capability when a UE indicates ‘no-gap’. Proposal 2: RAN4 shall define requirements for the interruption length, occasions and ratio to complete Rel-16 NeedForGap requirements. Proposal 3: RAN4 shall leverage the existing Rel-17 NCSG requirements to define the new interruption requirements for NeedForGap. Proposal 4: RAN4 shall define the requirements for intra-frequency measurement without gap (delay of a single layer) x CSSFoutside_gap, where the delay requirements can be reused from either Section 9.2.5 or NCSG. Proposal 5: RAN4 shall define the requirements for inter-frequency measurement without gap (delay of a single layer) x CSSFoutside_gap, where the delay requirements can be reused from either Section 9.3.9 or NCSG. |
| R4-2216746 | Qualcomm Incorporated | Observation1: In current 38.133 spec, there is no requirement to report via needforGap for inter-frequency measurement. Proposal 1. RAN4 define the requirement for inter-frequency measurement when UE report via needforgap. Observation2: It is not clear about R16 UE behaviour from defining a new requirement. Some R16 UE may cause interruption and some UE may not cause interruption when UE report no-gap via needforGAP. It is not clear how the R16 UE meet the requirements. Proposal 2. The requirement shall apply only for R18 UE who report no-gap. No impact on other release UE. Proposal 3. For R18 UE, we can compromise to define interruption requirement when UE report no-gap. We are open to discuss having an optional UE capability to indicate the R18 UE whether interruption is needed. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: Interruption

[Background:

The first objective for the measurements withoug gaps becasue of UE supporting NeedForGapInfoNR is:

    - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed

]

#### Issue 1-1-1: Whether interruption is expected when UE reports ’no-gap’ in ‘NeedForGapsInfoNR'

- Proposals

  - Option 1: Intel, Apple, Xiaomi,  Ericsson,   MTK, Qualcomm, ZTE

    - Yes

  - Option 2: CATT, CMCC, Huawei

    - No

  - Option 3: Intel, Xiaomi, Apple, OPPO, vivo, Qualcomm

    - Introduce additional UE capability to differentiate whether UE needs interruption

  - Option 4:

    - If interruption is needed for a UE without gaps, it should be indicated using new indication as part of needForGaps, needForGapsNCSG or a new information element.

- .Recommended WF

  - Collect company views in the 1st round

| Company | Comments |
| --- | --- |
| CMCC | Option 2. Firstly, according to TS 36.133, if UE is capable of interFreqNeedForGaps or interRATNeedForGaps, it is clearly stated that the measurement is conducted without gaps and without interruption. Same appraoch can be used for NR. Secondly, NCSG for NR is introduced in Rel-17.VIL (visible interuption length) is specified, which means interruption is allowed for NCSG. If there is additional interruption when UE reporting ‘NeedForGapsInfoNR' , there is no difference between NCSG and NeedForGaps. TS 36.133, 8.1.2.1: Inter-frequency and inter-RAT measurement requirements within this clause rely on the UE being configured with one measurement gap pattern unless the UE has signaled that it is capable according to the capability interFreqNeedForGaps or interRATNeedForGaps of conducting such measurements without gaps and without interruption.. |
| Huawei | We support option 2 but we can compromise to option 1. We are open to option 3, but a question is whether it is a 1-bit capability, or is it reported for each target band for the current serving cell band combination. |
| Ericsson | Option 1. We fully understand CMCC’s concern. However, we need to tradeoff between the application scope of this feature and the interruption. If we don’t allow any interruption, that means less use scenario for this feature since UE can’t use a spare RF chain to perform measurement. The outcome is UE will announce less band with ‘no gap’ and report ‘gap’ directly. If we hope UE to use ‘no gap’ to replace the ‘gap’ to minimize the interruption, it’s better to allow some interruptions to UE. We’re also open to further discuss whether to introduce an additional indication from UE to differentiate whether UE needs interruption when UE reports ‘no gap’. |
| ZTE | Prefer Option 1. In fact the discussion here is similar as NCSG. In NCSG, even when the UE has an idle RF chain to perform intra-f/inter-f measurement, since of RF re-tuning, RF on/off, interruption should be allowed before and after the measurement. Here the situation is similar. When UE reports ‘no-gap’ through NeedForGap mechanism, for the measurement itself, not need any interruption due to an idle RF chain can be used for measurement. But the switching from ‘gap’ to ‘no-gap’ would be accompanied by the change of UE RF BW and/or MO, so the latency of RF re-tuning or RF on/off should be considered. |
| Apple | Support option 1 and 3. To CMCC, even without interruption, it can also be covered by NCSG. UE with such implementation would report ‘nogap-noncsg’ for target band. To HW on option 3, in our understanding the need of interruption is band combination dependent. Therefore, it would be good to allow UE report different capabilities for different bands. |
| Qualcomm | We agree that the requirement of no-gap for inter-frequency measurement is missing in current spec and it is not clear when UE report no-gap for inter-f measurement and SSB is outside of active BWP. This issue is related to how to define interruption requirements. If NCSG format is used as the interruption requirement for no-gap, UE shall use NCSG and no need to define requirement. We think invisible interruption requirement should be considered similar to the one in SCell measurement. Otherwise, we don’t see any benefit from reporting no-gap via Needforgap because using NCSG will be effectively same. Finally, we can compromise to option1 for inter-frequency measurement when SSB is outside of active BWP and interruption requirement is invisible way as described in R4-2214346. We can also further compromise to option3 only for R18 UE as interruption may be required or may not be required per target band. |
| Xiaomi | Support option 1 and option 3 For option 3, we prefer to introduce another UE indication for ‘no gap no interruption’ besides the existing ‘no gap’, as we think the current indication ‘no gap’ means interruption is always expected. |
| OPPO | Support option 3. Interruption is needed in some scenarios like NCSG, while interruption is not needed in the other scenarios. We think introducing UE capability is a good compromise. |
| Intel | We support Option 1 and 3. The main concern for Option 2 is that it is unclear NW/UE behavior when UE reports ‘no-gap’ in ‘NeedForGapInforNR’ which may not cover Rel17 aspects (e.g. NCSG). For an example, does ‘no-gap’ mean neither Rel16 legacy gap nor NCSG configured to UE? In our view, if ‘no-gap’ needs to be consisted in both ‘NeedForGapInfoNR’ and ‘NeedForNCSGNR’, the interruption shall be allowed. But in order to more clear UE behavior, we are fine with Option 3 in which the more specific UE capability can be introduced for such case. |
| vivo | Ok with option 3 and option 1. Regarding the interruption, our understanding is similar definition to that of NCSG can be reused. |
| CATT | Support option 2. Share the same view as CMCC. We think this is a different and separate feature with NCSG and no spare RF chain is required for this feature. Secondly, the case of no gap and with interruption has already been covered by NeedForGapNCSG and no need to have another signaling to indicate the same case. Thirdly, it is not reasonable to allow interruption for periodic measurement since it is frequent and the interruption will seriously decrease the throughput. |
| Nokia | We support Option 2, Option 4 and Option 5. As for Option 1, we think that allowing interruption for UE that signals ‘no-gap’ is equivalent to changing the legacy behavior. If we allow that we will introduce confusion on the interpretation of the signaling, since Rel 17 UEs that signal ‘no-gap’ would behave differently than UEs signaling ‘no-gap’. Additionally, there is a potential scenario where a UE is Rel 18 and the BS is Rel 17. If such UE signals ‘no-gap’ the behavior will be unpredictable. For Option 3, we agree with the intention but we are not sure if UE capability is the correct way of implementing it. For that reason we introduced Option 4, which is more general than Option 3. We included Option 5 here, since it was in out paper but was not captured in any issues. [Moderator: Option 4 can be FFS because how to indicate such capability is still open if Option 3 agreed. Option 5 seems true but is most like UE behavior impacts due to the new capability indication, which can be discussed under sub-topic 1-3. Thus we will remove them from this issue to 1-3-1 to avoid the duplicated discussion.] |
| MediaTek | We support Option 1. When the UE performs RF re-tune the UE doesn’t need a gap, however, the UE needs interruption time. Also, the UE may be configured with inter-frequency layers more than the IDLE RF chains and in this case the UE needs to do RF re-tune to finish all jobs. For Option 2: we would like to understand how would the UE be capable of performing RF retuning without interruption nor gap? To CMCC: RAN4 doesn’t need to follow everything from LTE specs for 5G NR, otherwise, what is the point of this WI? For Option 3: our preference is not to define additional capability because this will be very similar to that of NCSG capability. Yet, we can compromise to this if all companies are in support for it. |
| Moderator | According to companies more detailed analysis and observations, the main diverse among companies is - [Q1] whether is UE’s behaviour when reporting ‘no-gap’ in NeedForGapsNR same as that when reporting ‘nogap-noncsg’ in NeeForNCSG-NR in Rel17. Moderator supposed that - the answer for Q1 is NO for the proponents of Option 1 but Yes for the proponents of Option 2. If the understanding is correct, we can check whether companies can be converged to Option 3 by which the additional UE capability (similar as ‘nogap-noncsg’ in ‘NeeforNCSG-NR) will be introduced to remedy such ambiguity. Which IE shall be used to indicate this new feature can be FFS. The IEs related were self-contained below. |

#### Issue 1-1-2: Requirements on the interruption length , if allowed

[Moderator notes: According to the issue 1-1-1, the interruption requirements to be defined is for the case when UE performing measurements without gap via “no-gap or others[TBD]” in NeedForGapInfoNR]

- Proposals

  - Option 1:  Apple, xiaomi, OPPO, vivo, Huawei

    - As a starting point, the interruption length can be same as these defined for NCSG,e.g.

      - When UE reporting “no-gap” in NeedForGapInfoNR  the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

      - When UE reporting “others[TBD]” in NeedForGapInfoNR no interruption allowed

  - Option 1b:

    - As a starting point, the interruption length can be same as these defined for NCSG,e.g.

      - When UE signals that interruption is needed for gap-less measurements the interruption length can be VIL=1 ms in FR1 and VIL=0.75 ms in FR2.

  - Option 2: Nokia

    - Consider smaller interruption length than VIL1+VIL2 from NCSG for a UE that requires additional interruptions for measurements without gaps.

- Recommended WF

  - Collect company views in the 1st round

| Company | Comments |
| --- | --- |
| Huawei | Option 1. The second sub-bullet of option 1 depends on outcome of Issue 1-1-1. |
| Ericsson | We suggest option 1a. - As a starting point, when UE reporting “no-gap” in NeedForGapInfoNR, the interruption length can be the same as defined for NCSG for each interruption occasion. |
| ZTE | Option 1. The interruption length defined in NCSG can be a starting point. |
| Apple | Support option 1. |
| Qualcomm | We do not support VIL from NCSG. But we support 1ms as reference point when design interruption ratio. |
| Xiaomi | Support option 1. |
| OPPO | Support option 1. |
| Intel | We are fine both Option 1 and 1a. For Qualcomm’s comment, we supposed that the absolute RTT length in unit of ‘s’ can be taken as the start point to define the interruption length in slots. |
| vivo | Ok with option 1. |
| CATT | It is not clear why the interruption is still called “VIL” which is used for NCSG and is not clear why 1ms and 0.75ms are used to define interruption when the RF retuning time is 0.5ms and 0.25ms. |
| Nokia | Option 2 as a baseline We don’t agree with the wording of Option 1, since it already assumes that we have agreed with interruption when UE signals ‘no-gap’. We introduced Option 1b which we think decouples the discussion on the signaling and on the interruption length, and we think that wording should be used for the discussion instead of Option 1. We still support Option 2. [Moderator: For Option 1b, actually VIL is specified for NCSG only, we may not use them for NFG. ] |
| MediaTek | As a starting point, we can support option 1 as a baseline. The sub-bullet in option 1 can be updated later based on the outcome of the previous issue 1-1-1. |
| Moderator | Further check the harmonized option also. Option 1c: As a starting point, when UE reporting “no-gap” in NeedForGapInfoNR, if interruption allowed the interruption length can be specified based on the same RTT assumption as for NCSG interruption occasion. 0.5ms in FR1 and 0.25ms in FR2 |

#### Issue 1-1-3: Requirements on the interruption location , if allowed

- Proposals

  - Option 1:  OPPO, MTK

    - Interruption location needs to be specified.

  - Option 1a:  OPPO,

    - For interruption location, use the boundary of SMTC as the starting/ending pointing

- Recommended WF

  - Collect company views in the 1st round

| Company | Comments |
| --- | --- |
| Huawei | Support option 1. Option 1a can be used as the starting point to define interruption location. |
| Ericsson | It needs further discussion. From our understanding, one option is to define a dedicated interruption location as option 1, another option is to define a total interruption ratio as deactivated SCell requirement. From our understanding, deactivated SCell requirement is very similar as this NeedForGaps. It will be measured within NCSG if NW configures NCSG, otherwise, it will be measured outside MG with interruptions. |
| ZTE | Option 1. Need further discussion. |
| Apple | Option 1 is fine. Option 1a needs further study, e.g. when SMTC duration is different on different carriers. |
| Qualcomm | We do not support all options. We think location may not be defined the specific location but define interrupting ratio as similar as 8.2.2.12.3. We are okay to consider SMTC as one of factor to control interruption ratio. 8.2.2.2.12.3 Interruptions due to RRM measurements during SCell dormancy When one or more SCells are in dormancy, the UE is for the purpose of RRM measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s). The rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from RRM measurements on dormant SCells shall not exceed 1.0%.. |
| Xiaomi | Fine with option 1, we can further check option 1a. |
| OPPO | Support option 1 and 1a. The interruption location should be clearly aligned between UE and NW. The exact location in option 1a could be considered as the baseline. |
| Intel | Support Option 1. How to define the specific location can be FFS. |
| vivo | Ok with option 1 |
| CATT | Need further discussion after the previous issues are decided. |
| Nokia | We support Option 1 as a baseline. It is important that the position of the interruption is clear from the specification, otherwise the gNB will not be aware of when this interruption is happening. If this position is unclear the scheduler will not be able to make good decisions and might schedule the UE when it is interrupted wasting network resources. |
| MediaTek | Option 1 can be supported. |
| Moderator | Can be FFS |

#### Issue 1-1-4: Requirements on the interruption ratio , if allowed

- Proposals

  - Option 1:  Ericsson,  MTK

    - RAN4 to further discuss how to control the total interruption ratio for NeedForGaps capability

- Recommended WF

  - Collect company views in the 1st round

| Company | Comments |
| --- | --- |
| Huawei | FFS. If we define location and length of interruption, do we still need to define interruption ratio? |
| Ericsson | Option 1 From our understanding, defining dedicated location is one of options to control the total interruption. We can further discuss the solutions once RAN4 agrees to allow the interruption. |
| ZTE | Agree with the motivation of Option 1, which tries to restrict the impact of interruption. But we share similar concern as Huawei. |
| Apple | FFS. Similar view as HW that if interruption location is clearly defined. We may not need to define interruption ratio. |
| Qualcomm | Option1. We prefer similar to interruption requirement in SCell measurement with invisible interruption length. We can further discuss how to design. |
| Xiaomi | Share the view with Huawei. |
| OPPO | Share the view with Huawei. |
| Intel | We are fine with Option 1 by which the explicit requirements on total interruption is to be defined. |
| vivo | Depend on the outcome of issue 1-1-3. |
| Nokia | Not clear how that can be acchieve. That might be kept as FFS. |
| MediaTek | Keep FFS. We believe this issue depends on the outcome of the issues 1-1-2 and 1-1-3. To our understanding this could be an alternative solution to the previous two issues. |
| Moderator | There is some dependency with issue 1-1-3. Suggest to be FFS with issue1-1-3 together. |

#### Issue 1-1-5: Other aspect on whether to allow interruption

- Proposals

  - Proposal 1:  Huawei

    - If interruption is allowed when UE reports ‘no-gap’, the interruption should be allowed for each of intra- and inter-frequency measurements for which UE reports ‘no-gap’.

      - The interruption will impact all the serving cells if UE does not support per-FR gap, and all the serving cells in the same FR as the measurement if UE supports per-FR gap.

  - Proposal 2: ZTE

    - The switching from ‘gap’ to ‘no-gap’ would be accompanied by the change of UE RF BW and/or MO, so interruption can be allowed when UE reports ‘no-gap’ through NeedForGap

  - Proposal 3: Nokia

    - RAN4 to limit scope of the requirements without gaps with additional interruption to inter-frequency scenarios.

- Recommended WF

  - Collect company views in the 1st round

| Company | Comments |
| --- | --- |
| Huawei | Support P1 assuming option 1 is adopted for Issue 1-1-1. If option 3 for Issue 1-1-1 is adopted we can further discuss this issue. P2 is not very clear to us, does it address the interruption during reconfiguration of serving cell combination or MO (where the UE capability may switch between ‘gap’ and ‘no-gap’)? |
| Ericsson | For P1, We have the same view with Huawei. For P2, needs further clarification. Does it mean interruption is allowed when UE reports ‘no gap’? |
| ZTE | Support P1. P2 is related with Issue 1-1-1, we want to analysis the reason of switching between ‘gap’ and ‘no-gap’. |
| Apple | P1 is fine if interruption is always allowed. P2 is a bit unclear to us. It seems similar to option 1 under issue 1-1-1. |
| Qualcomm | We support P1 for SSB is outside of BWP for both intra-f and inter-f measurement. |
| Xiaomi | Support P1 |
| OPPO | Support proposal 1. |
| Intel | The first bullet of P1 is fine for us. But the sub-bullet needs some clarification. For P2, in our understanding, such capability to support ‘NeedForGap’ cant’ be changed dynamically when BWP switching. |
| vivo | Support P1 |
| CATT | Need further discussion. As we commented in issue 1-1-1, if we follow proposal 1, the throughput will be highly decreased. |
| Nokia | We support Proposal 3. We support Proposal 1 and 2. They cannot be approved without decision on the signaling aspect of no-gap and will be changing legacy behavior. For option 1, we don’t understand why intra-frequency measurements would require additional interruption. For intra-frequency requirements the typical use case is measurements without gaps, and we don’t understand the reasons behind allowing interruptions on a scenario that is already supported since Rel 15. What is the technical justification behind needing additional interruption in that case? |
| MediaTek | Fine with option 1. Option 2 is not very clear to us yet. |
| Moderator | Can be FFS @Nokia, both intra-f and inter-f measurements are possible to perform measurement within gap [Please refer to the condition defined for intra-f with gap in TS38.133 9.2.1]. Thus the interruption under intra-f is valid. With this explanation we may need to limited the interruption requirement for inter-f only as Proposal 3. |

### Sub-topic 1-2: Measurement reporting delay requirements

[Moderator notes: it is better to differentiate the measurement without gap into the two scenarios below when considering the measurement reportint delay requirements as for the interruption requirements:

- Case 1: without gap and no interruption (e.g. ’nogap’ or ’nogap-nointerruption[TBD]’ indicated in NeedForGapInfoNR)

- Case 2: without gap but interruption allowed (e.g. ’nogap’ indicated in NeedForGapInfoNR)

Some companies’ proposals on these issues below are based on the assumption of ’no-gap’ inidicated in NeedForGapInfoNR message. Hereby in order to simplify our discussion , the exact value (’no-gap’, ’nogap-nointerruption’ or others ) for the scenario (in which no gap will be configured and no interruption allowed) can be decoupled from the measurement delay requirements firstly.]

#### Issue 1-2-1: Requirement for intra-freq measurement without gap when no interruption (intra-f case 1)

- Proposals

  - Option 1: CATT, Apple, CMCC, Xiaomi, OPPO, Huawei, Nokia, MTK

    - Reuse requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap)

  - Option 1a: Intel, MTK

    - updated CSSFoutside_gap is needed.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Option 1. As commented in Issue 1-1-1, no interruption is allowed. And the cell identification requirements, measurement period requirements, and scheduling availability, as specified in 9.2.5 can be resued. |
| Huawei | Support option 1. On option 1a, could proponents please clarify what updates to CSSF is foreseen? |
| Ericsson | For case 1, we’re fine with option 1. |
| ZTE | Support Option 1. |
| Apple | Support option 1. As for option 1a, we think existing CSSF already counts the carriers for which UE reports ‘no-gap’: The UE can perform intra-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3]. |
| Qualcomm | Support Option 1. |
| Xiaomi | Support option 1. |
| OPPO | Support option 1. |
| Intel | We can support Option 1. |
| vivo | Support option 1 |
| CATT | Support option 1. |
| Nokia | We support Option 1. |
| MediaTek | Support option 1. |
| Moderator | Agree Option 1 |

#### Issue 1-2-2 Requirement for intra-freq measurement without gap when interruption allowed (intra-f case 2)

- Proposals

  - Option 1: Intel, Xiaomi, Apple,

    - Take requirements NCSG requirements as a starting point

  - Option 1a: Apple

    - The other aspects can be FFS. e.g.

      - The time slot alignment among the measurement objects and interruption location

  - Option 2: 	Ericsson

    - RAN4 cannot follow NCSG to define NeedForGaps’ measurement requirement since no pattern design for NeedForGaps.

    - The deactivated SCell measurement requirement can be the start point.

  - Option 2a: Ericsson

    - The frequency layers in the band for which UE reports ‘no gap’ should be counted in CSSF outside gap

  - Option 3: Nokia

    - No interruption allowed for intra-freq measurement requirements

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | We suggest to consider a new option 3. - Option 3: - Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point On option 1, we assume when UE reports ‘no-gap’ for NFG, NW will not configure MG or NCSG, so it is unclear how NCSG requirements could apply, as mentioned by the first bullet of option 2. On option 1a, we are fine to keep this issue as FFS. On option 2, we agree with the first bullet. For the second bullet, the deactivated SCell measurement requirement is based on scellMeasCycle which can be much larger than SMTC. Would such large delay be an issue for normal intra- and inter-frequency measurement? On option 2a, we are fine with it. |
| Ericsson | Option 2, 2a Our intention is to follow deactivated SCell to define an interruption ratio and interruption length. For the total delay, we can follow the solution to measure the MO outside gaps but we’re fine to change the scellMeasCycle with other periodicity less than it. |
| ZTE | Prefer Option 1. |
| Apple | This issue can be revisited after RAN4 has conclusion on issue 1-1-3 and 1-1-4. If interruption location is to be clearly defined, we may have a similar concept of ‘pattern’. But if interruption ratio is to be defined, we probably can use the framework of measurement on deactivated SCell as mentioned in option 2. As for option 2a, we think it has already been captured in current spec: The UE can perform intra-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3]. |
| Qualcomm | Support option 2. And details need to be updated. |
| Xiaomi | We share the view with Apple, the interruption requirement discussion in previous issue may impact the mesaurement requirements. We can wait for the conclusion on interruption requirements. |
| Intel | We share same view as Apple. The requirements framework is up to issue 1-1-1 and 1-1-3. If the occasion of interruption due to ‘no-gap’ was known by UE, the actual measurement interval (like VIRP of NCSG) can be the replacement of measurement requirements for NCSG in TS38.133. |
| vivo | Similar view as APple |
| CATT | Need further study. Whether the interruption is needed and how to define the interruption should be decided firstly. And if interruption is allowed, we share the same view as Huawei that the requirements in 9.2.5 can be the starting point. For option 1, we assume it means the requirements when UE report “nogap-noncsg” as baseline, and if this is the understanding, it seems same as the proposed option 3 by Huawei. |
| Nokia | We support Option3, which in included in the options now. We included that in our paper but it was not included in any issues, so we would like that to be considered for the discussion. We don’t see the justification for additional interruptions on intra frequency measurements since there is no RF retuning considered. [Moderator: same comments as issue 1-1-5.] |
| MediaTek | We support option 1 on top of supporting option 2a (i.e. CSSF shall follow CSSFoutside_gap). |
| Moderator | Can be FFS. Companies have quite diverse views on this.. |

#### Issue 1-2-3: Requirement for inter-freq measurement without gap when no interruption (Inter-f case 1)

- Proposals

  - Option 1: CATT, Xiaomi, Apple, OPPO, vivo, Huawei, Nokia, Qualcomm

    - Take requirements in Section 9.3.9 of TS38.133 (inter-freq w/o gap) as a starting point

  - Option 1a: CMCC

    - to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap

  - Option 1b: Huawei

    - the sample number should be 8

  - Option 1c: Intel, CATT, MTK

    - updated CSSFoutside_gap is needed.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Support 1a. For option 1, one consideration is about the number of samples. Section 9.3.9 of TS38.133 (inter-freq w/o gap) is for the scenario that SSB is completely contained in the active BWP.The sample number in the requirements of Section 9.3.9 is 5 since AGC is not needed. For the case with inter-freq measurement without gap when UEs reporting NeedForGapsInfoNR, the SSB will be outside active BWP, AGC may be needed, we are not sure whether sample number of 5 can be reused. Option 1a is not conflict with other options. According to existing spec, the definition of inter-frequency SSB based measurements without measurement gaps only consider the case that SSB is completely contained in the active BWP of the UE and the case that UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoNR for the inter-frequency measurement. Similar like the intra-frequency case, when UE indicates ‘no-gap’ via interFreq-needForGap, it can also be considered as inter-frequency measurement without gaps. The detailed update of spec is proposed as following (the update part is highlighted in yellow): TS 38.133 A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) for UE capable of interFrequencyMeas-NoGap provided - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. A measurement is defined as inter-frequency measurement without gaps if the UE indicates ‘no-gap’ via interFreq-needForGap for inter-frequency measurement. For UE supporting ncsg-MeasGapNR-r17 and indicating NeedForNCSG-InfoNR for inter-frequency measurement, - An inter-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoNR for the inter-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE. |
| Huawei | Support option 1, 1a and 1b. On option 1c, could proponents please clarify what updates to CSSF is foreseen? |
| Ericsson | For case 1, we’re fine with option 1. |
| ZTE | Support Option 1. |
| Apple | Support option 1, 1a and 1b. As for option 1c, is this because existing CSSF doesn’t include case when UE indicates ‘no-gap’ via interFreq-needForG? If so, option 1a can address it. |
| Qualcomm | Support option1. when UE does not require interruption, the current capability already support it. Thus no need to change. |
| Xiaomi | Support option 1 and option 1a. The certain sample number can be further discussed, which we think is related to option 3 in issue 1-1-1. |
| OPPO | Support option 1, and we are open to option 1b. |
| Intel | Option 1 is fine for us for the case without interruptions. |
| vivo | Ok with option 1. |
| CATT | Support option 1, 1a. The sample number can be further studied. For option 1c, our initial thinking is to include the case when reporting “no-gap” for inter-frequently measurement. But as commented by Apple in previous issue, the case has already been included in CSSFoutside_gap, we are fine to further study whether the CSSF updates are needed. |
| Nokia | We agree with Option 1 For Option 1a it will depend on the outcome regarding the signaling. Option 1b and 1c can be FFS. |
| MediaTek | We support option 1 |
| Moderator | Option 1 can be agreed firstly, which is not conflict with Option 1a. |

#### Issue 1-2-4: Requirement for inter-freq measurement without gap when interruption allowed(Inter-f case 2)

- Proposals

  - Option 1: Intel,  Apple,

    - Take requirements NCSG requirements as a starting point

  - Option 1a: Apple

    - The other aspects can be FFS. e.g.

      - Alignment among the interruption location on all carriers

  - Option 2: 	Ericsson

    - RAN4 cannot follow NCSG to define NeedForGaps’ measurement requirement since no pattern design for NeedForGaps.

    - The deactivated SCell measurement requirement can be the start point.

  - Option 2a: Ericsson

    - The frequency layers in the band for which UE reports ‘no gap’ should be counted in CSSF outside gap

  - Option 3:

    - For inter-frequency measurement requirements use requirements in 38.133, clause 9.3.9 as a starting point

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | Same comment as for Issue 1-2-2. |
| Ericsson | Same as Issue 1-2-2. |
| ZTE | Prefer Option 1. |
| Apple | Same comments as for issue 1-2-2. |
| Qualcomm | Option2 and same as Issue 1-2-2. |
| Xiaomi | Same as issue 1-2-2 |
| OPPO | Support option 2. |
| Intel | Same as Issue 1-2-2. |
| Vivo | Same as Issue 1-2-2. |
| Nokia | We support Option 3 Option 3 was in our document but was not included here for some reason. We understand that measurements without gaps are not the same as NCSG, otherwise the UE would signal need for NCSG. Therefore the requirements for intra-frequency without gaps can be used as baseline for those requirements. |
| MediaTek | We are fine with option 1 with option 2a. |
| Moderator | Suggest to be combined with issue 1-2-2 |

### Sub-topic 1-3: UE behavior

#### Issue 1-3-1: UE behaviors when UE supports both NeedForGap and NCSG capabilities

- Proposals

  - Proposal 1: Ericsson

    - The gap status indication in NeedForGaps should have 1-to-1 mapping with the gap status in NCSG if UE supports both NeedForGaps and NCSG capabilities.

      - UE should report ‘no gap’ in the same band for NeedForGaps if reporting ‘no gap no interruption’ or ‘no gap no interruption’ in a band for NCSG

      - UE should report ‘gap’ in the same band for NeedForGaps if reporting ‘gap’ in a band for NCSG

  - Proposal 2:

    - The gap status indication in NeedForGaps should have 1-to-1 mapping with the gap status in NCSG if UE supports both NeedForGaps and NCSG capabilities.

      - The exact mapping of the reports in NeedForGaps, NeedForGapNCSG and/or other new signaling options is FFS.

  - Proposal 3: Nokia

    - Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | We suggest to postpone this issue after we have agreement for Issue 1-1-1. |
| Ericsson | Proposal 1 From our understanding, UE may support both NeedForGaps and NCSG. In this case, UE should guarantee the reporting have 1-to-1 mapping with the gap status in each band. We’re open to further discuss the detail. |
| ZTE | Generally fine with Proposal 1. |
| Apple | Outcome of issue 1-1-1 has impact on this issue. |
| Xiaomi | Wait for the conclusion on issue 1-1-1. |
| OPPO | Fine with Proposal 1. But we are not clear why UE will report both need for gap and need for ncsg information for the same band. |
| Intel | Can be FFS upon 1-1-1 |
| vivo | FFS |
| CATT | FFS. There is no reporting of “no gap no interruption”, does it mean “nogap-noncsg”? |
| Nokia | Proposal 2 We support the general idea of the proposal 1, but since it depends on the outcome of signaling we drafted Proposal 2, which we believe captures the idea in a more general way. |
| MediaTek | We can discuss this issue after consensus is reached in issue 1-1-1. |
| Moderator | Can be FFS upon on the conclusion of issue1-1-1 |

#### Issue 1-3-2: UE behaviors mismatch between UE and NW

- Proposals

  - Proposal 1: Ericsson

    - FFS:  UE’s behaviour in the following mismatch scenarios

      - Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps

      - Rel-16 UE which supports NeedForGaps in a Rel-17 NW which supports NCSG

      - Both UE and NW support NCSG and NeedForGaps

  - Proposal 2: Nokia

    - Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | Fine with P1 to further study. |
| Ericsson | Fine with P1. It may further include some Rel-18 UEs which supports NeedForGaps if we agree to differentiate the Rel-16 and Rel-18 UE for NeedForGaps. |
| ZTE | Need further study. |
| Apple | Fine for further study. We can understand the need to study the last sub-bullet if both UE and NW support both features. Regarding the first two sub-bullets, we are not sure what to study. |
| Qualcomm | We are okay with P1. |
| Xiaomi | Fine to further study, we are not sure if its OK to link the two UE capabilities. |
| OPPO | Fine to further study. |
| Intel | P1 is fine. This issue can be FFS. |
| vivo | Need further study before any conclusion |
| Nokia | We think that legacy behavior should not be changed. Therefore we propose P2. As for Proposal 1, it is good to reflect on the mismatch scenarios, but we cant really change product that is already deployed. So we need to be extra careful on the requirements that could lead to misleading interpretations when combining UEs and gNBs from different releases. |
| MediaTek | Fine to consider this for further study. |
| Moderator | Can be FFS |

### Sub-topic 1-4: Scheduling availability

#### Issue 1-4-1: Scheduling availability

- Proposals

  - Option 1: CMCC, OPPO

    - take the similar requirements for NCSG (TS38.133 v17.6.0 9.3.10.3) as baseline to define scheduling availability。

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Option 1. |
| Huawei | Fine with option 1. |
| Ericsson | Option 1 |
| ZTE | Fine with Option 1. |
| Apple | Option 1. |
| Qualcomm | We support Option1. |
| Xiaomi | Fine with Option 1. |
| OPPO | Support option 1. |
| Intel | Option 1 is fine for us. The framework of NCSG can be taken as the start point to define scheduling availability |
| vivo | Ok with option 1 |
| CATT | Fine to take it as starting point. But one question for clarification, is the signaling deriveSSB-IndexFromCell-inter still applicable? |
| Nokia | Issue is not clear Does that scheduling availability apply for UEs indicating additional interruption? We have issues also for gap-less without additional interruption, does that proposal applies for them as well? |
| MediaTek | Fine with option 1. |

### Sub-topic 1-5: Requirements applicalbilty

#### Issue 1-5-1: General requirements applicalbilty

- Proposals

  - Proposal 1: Qualcomm

    - The requirement shall apply only for R18 UE who report no-gap. No impact on other release UE

  - Proposal 2:

    - The requirement shall apply only for R18 UE who report no-gap with additional interruption. No impact on other release UE

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | Fine with P1. |
| Ericsson | We need further study this issue and the following impact in issue 1-3-2. |
| Apple | Fine with P1 in principle. |
| Qualcomm | To make clear, our proposal is related to Option 3 in issue 1-1-1. New capability is only applicable for R18 UE. |
| OPPO | Support proposal 1. |
| Intel | With the clarification of QC, P1 is fine for us. |
| vivo | Ok with P1 |
| MediaTek | We support proposal 1. |
| Nokia | We support Proposal 2. We think that the wording of proposal one is leading to a conclusion on the signaling, which is covered on other issues. Therefore we proposal P2, which is more generic and we believe captures the intention of proposal 1 [Moderator: these requirement to be defined need to cover two cases below: - Case 1: without gap and no interruption (e.g. ’nogap’ or ’nogap-nointerruption[TBD]’ indicated in NeedForGapInfoNR) - Case 2: without gap but interruption allowed (e.g. ’nogap’ indicated in NeedForGapInfoNR) ] |
| Moderator | Option 1 can be agreed as it is generic principle. But the more precise wording is needed. |

## CRs/TPs comments collection

Moderator: No CR/TP in this AI

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Status summary

#### Issue 1-1-1: Whether interruption is expected when UE reports ’no-gap’ in ‘NeedForGapsInfoNR' Status: 9 companies support Option 1 or 3, 3 companies Option 2, and 1 company Option 4 and Option 5. Tentative agreements: N.A. Recommendations for 2nd round: To be further discussed in GTW and 2nd round email discussion. We can check whether the following harmonized proposal in WF can be accepted by companies. Proposal : Introduce additional UE capability to differentiate whether UE allows interruption when reporting them via ‘NeedForGapsInfoNR’. The exact indication for such capability can be FFS.

#### Issue 1-1-2: Requirements on the interruption length , if allowed Status: 3 companies had some technical concerns on Option 1. Tentative agreements: N.A. Recommendations for 2nd round: Companies can further discuss the following candidates options in WF in 2nd round. - Option 1: - As a starting point, the interruption length can be same as these defined for NCSG,e.g. - When UE reporting “no-gap” in NeedForGapInfoNR the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2. - When UE reporting “others[TBD]” in NeedForGapInfoNR no interruption allowed - Option 1a: - As a starting point, the interruption length can be same as these defined for NCSG,e.g. - When UE reporting “no-gap” in NeedForGapInfoNR the interruption length can be specified based on the same RTT assumption(1ms in FR1 and 0.75ms in FR2) as for NCSG interruption occasion. - When UE reporting “others[TBD]” in NeedForGapInfoNR no interruption allowed - Option 1b: - As a starting point, the interruption length can be same as these defined for NCSG,e.g. - When UE signals that interruption is needed for gap-less measurements the interruption length can be VIL=1 ms in FR1 and VIL=0.75 ms in FR2. - Option 2: - Consider smaller interruption length than VIL1+VIL2 from NCSG for a UE that requires additional interruptions for measurements without gaps. - Option 3(Harmonization): - As a starting point, when UE reporting “no-gap” in NeedForGapInfoNR, the interruption length can be specified based on the same RTT assumption as for NCSG (0.5ms in FR1 and 0.25ms in FR2) interruption occasion.

#### Issue 1-1-3: Requirements on the interruption location , if allowed Status: 9 companies supported Option 1. But 3 companies suggested FFS. Tentative agreements: N.A Recommendations for 2nd round: Further check and continue discussion in the 2nd round on the candidate options below in WF. - Option 1: - Interruption location needs to be specified. - FFS on the interruption location - Option 2: - No need to define the specific interruption location but the total interruption ratio

#### Issue 1-1-4: Requirements on the interruption ratio , if allowed Status: There are some diverse views among companies on this issue. Tentative agreements: N.A Recommendations for 2nd round: Further discussion are needed. This issue may depend on the conclusion of 1-1-1 and 1-1-3.The following candidates options will captured in WF for 2nd round discussion. - Option 1: - RAN4 to further discuss how to control the total interruption ratio for NeedForGaps capability - Option 1a: - RAN4 needs to define the total interruption ratio if no specific interruption location was required. - Option 2: - RAN4 needs not to define total interruption ratio but the interruption

#### Issue 1-1-5: Other aspect on whether to allow interruption Status: Companies still have quite diverse views on P1.For P2, it needs more technical clarifications. For P3, from the moderator perspective, we suggested to not considerations on it. Tentative agreements: N.A Recommendations for 2nd round: Regarding to some dependency with issue 1-1-1, the following candidates options will captured in WF for 2nd round discussion - Proposal 1: - If interruption is allowed when UE reports ‘no-gap or others [TBD]’, the interruption should be allowed for each of intra- and inter-frequency measurements for which UE reports ‘no-gap’. - The interruption will impact all the serving cells if UE does not support per-FR gap, and all the serving cells in the same FR as the measurement if UE supports per-FR gap. - Proposal 2: - The switching from ‘gap’ to ‘no-gap’ would be accompanied by the change of UE RF BW and/or MO, so interruption can be allowed when UE reports ‘no-gap’ through NeedForGap

#### Issue 1-2-1: Requirement for intra-freq measurement without gap when no interruption (intra-f case 1) Status: All companies can support Option 1. Tentative agreements: Reuse requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) for the reporting delay requirements for intra-frequency measurement without gap and no interruption allowed. #### Recommendations for 2nd round: No need further discussion. This agreement will be included in WF.

#### Issue 1-2-2&Issue1-2-4 Requirement for intra-freq/inter-freq measurement without gap when interruption allowed (case 2) Status: Companies have quite diverse views on this. Tentative agreements: N.A #### Recommendations for 2nd round: No further discussion needed. The open issues can be postponed to next meeting. The candidate options will be captured in WF. - Option 1: - Take requirements NCSG requirements as a starting point - The other aspects can be FFS. e.g. - The time slot alignment among the measurement objects and interruption location - Option 2: - The deactivated SCell measurement requirement can be the start point. - The other aspects can be FFS, e.g. - The frequency layers in the band for which UE reports ‘no gap’ should be counted in CSSF outside gap - Option 3: - Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point #### .

#### Issue 1-2-3: Requirement for inter-freq measurement without gap when no interruption (Inter-f case 1) Status: Majority companies can support Option 1 .For option 1a, 1b, 1c can be FFS upon the scenario issues are converged. Tentative agreements: - Take requirements in Section 9.3.9 of TS38.133 (inter-freq w/o gap) as a starting point - FFS on: - to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap - the sample number should be 8 - updated CSSFoutside_gap is needed. Recommendations for 2nd round: No further discussion needed. The open issues can be postponed to next meeting. The candidate options will be captured in WF.

#### Issue 1-2-4: Requirement for inter-freq measurement without gap when interruption allowed(Inter-f case 2) Status: All companies thought this issue is same as issue 1-2-2. Tentative agreements: Combine with issue 1-2-2 #### Recommendations for 2nd round: No further discussion needed.

#### Issue 1-3-1: UE behaviors when UE supports both NeedForGap and NCSG capabilities Status: Majority companies prefer to wait for the conclusions of issue 1-1-1. Tentative agreements: N.A. #### Recommendations for 2nd round: No further discussion needed. The open issues can be postponed to next meeting. The candidate options below will be captured in WF. - FFS on: - Proposal 1: The gap status indication in NeedForGaps should have 1-to-1 mapping with the gap status in NCSG if UE supports both NeedForGaps and NCSG(or other[TBD] ) capabilities. - Proposal 2: Legacy behavior of existing indication in needForGaps and needForGapsNCSG shall not be changed in Rel 18 NR_MG_enh2

#### Issue 1-3-2: UE behaviors mismatch between UE and NW Status: Majority companies prefer to keep it as FFS. Tentative agreements: N.A. #### Recommendations for 2nd round: No further discussion needed. The open issues can be postponed to next meeting. The candidate options will be captured in WF. - FFS on: UE’s behaviour in the following mismatch scenarios - Rel-17 UE which supports NCSG in a Rel-16 NW which only supports NeedForGaps - Rel-16 UE which supports NeedForGaps in a Rel-17 NW which supports NCSG - Both UE and NW support NCSG and NeedForGaps

#### Issue 1-4-1: Scheduling availability Status: All companies except 1 company can support Option 1. Tentative agreements: - Take the similar requirements for NCSG (TS38.133 v17.6.0 9.3.10.3) as baseline to define scheduling availability #### Recommendations for 2nd round: Companies can check this tentative agreement in WF in 2nd round. Companies can bring more detailed proposal on this requirement when the issues about using scenario are settled down.

#### Issue 1-5-1: General requirements applicalbilty Status: All companies except 1 company can support Proposal 1. Tentative agreements: - The requirement (e.g. the new UE capability to definite the interruption needed when UE report ‘nogap’ in Rel18) shall apply only for R18 UE who report no-gap. No impact on other release UE #### Recommendations for 2nd round: No further discussion needed for the technical perspective but some more exact wording is expected.

### CRs/TPs

Moderator: No CR/TP in this AI

## Discussion on 2nd round (if applicable)

The discussions in the WF are captured below

# Topic #2: inter-RAT measurement without gap(AI 6.10.3.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215369 | Intel Corporation | Proposal 1: In Rel18, we can focus on LTE/NR SA case only when discussing inter-RAT NR/LTE measurements without gap. Proposal 2: For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, the mixed numerology shall be considered. Observation 1: In Rel18, RAN4 need NOT to duplicate the discussion on the scenario of inter-RAT LTE measurement without gap when UE supporting NCSG capability. Proposal 3: The inter-RAT EUTRAN measurement only considers the case when LTE CRS to be measured is contained in UE’s active BWP. Proposal 4: Inter-RAT measurements without gaps shall be supported by separate basic UE capability. The other necessary UE capability to support the inter-RAT measurement wo gap can be FFS upon the different UE aspects. Proposal 5: If CRS is completely contained in the active BWP, RAN4 can take the capability to support Rel-16 inter-frequency measurement without MG as a baseline. Proposal 6: A new requirements on the cell identification and measurement reporting for inter-RAT measurement without MG should be specified in TS38.133. Proposal 7: The updates of CSSF requirements (e.g. CSSF_outside_gap) is needed given the impacts from inter-RAT measurements without MG. Proposal 8a: For the inter-RAT measurements without gap, the restrictions on the scheduling availability need to be considered. Proposal 8b: The existing scheduling availability specified for intra-frequency measurements in TS 38.133 section 9.2.5.3 can also be applied to the inter-RAT measurement without measurement gaps as a start point. |
| R4-2215428 | CATT | Inter-RAT NR measurements Proposal 1: No need to extend the capability of NeedForNCSG to inter-RAT NR measurement. UE reporting ‘FALSE’ for interRAT-NeedForGapsNR can be defined as supporting the inter-RAT NR measurement without gaps. Proposal 2: It is confirmed that the previous issue 3-9 in the agreed WF is about which requirement framework is used as a baseline or starting point for RAN4 to develop the Inter-RAT NR measurements. Proposal 3: For inter-RAT NR measurements, new measurement requirements can be defined based on NR inter-frequency measurement without gap (when ‘no-gap’ reported via NeedForGapsInfoNR). Inter-RAT LTE measurements Proposal 4: There is no need to extend the capability of NeedForGaps to inter-RAT LTE measurement. Proposal 5: The case when UE reporting ‘nogap-noncsg’ for NeedForGapNCSG-InfoEUTRA is defined as supporting the inter-RAT LTE measurement without gaps for the corresponding LTE target band. Rel-17 measurement requirements will be specified for this case, so no need to define new requirements in Rel-18. Proposal 6: For the case when LTE CRS to be measured is contained in UE’s active BWP, new measurement requirements for inter-RAT LTE measurement without gaps need to be defined and the measurement requirements for LTE inter-frequency measurement can be taken as a starting point. |
| R4-2215468 | Xiaomi | Proposal 1: RAN4 to consider following scenarios where UE could perform inter-RAT LTE and inter-RAT NR measurement without MG: - Another spare RF chain is available for UE; - The target RS to be measured is with UE’s active RF chain. Proposal 2: RAN4 to define inter-RAT NR measurement without gap requirements in TS 36.133 for UE supporting NeedForGaps capability and indicating InterRAT-BandInfoNR-r16 for inter-RAT measurement. Proposal 3: RAN4 to define inter-RAT LTE measurement without gap requirements in TS 38.133 for UE supporting NeedForGapNCSG capability and indicating NeedForGapNCSG-InfoEUTRA-r17 for inter-RAT measurement. Proposal 4: RAN4 to extend the capability of NeedForGaps for allowing UE indicating gap information for inter-RAT LTE measurement. Proposal 5: RAN4 to extend the capability of NeedForGapNCSG for allowing UE indicating gap information for inter-RAT NR measurement. |
| R4-2215612 | Apple | Proposal 1: MR-DC scenarios are deprioritized. Observation 1: inter-RAT LTE measurement has already been supported by eutra-NeedForGapNCSG-reporting-r17. Proposal 2: no need to extend the capability of NeedForGaps to cover inter-RAT LTE measurement. Observation 2: ‘extend the capability of NeedForNCSG to inter-RAT NR measurement’ is unclear since there is no NeedForNCSG in LTE. Observation 3: inter-RAT NR measurement has already been supported by interRAT-NeedForGapsNR-r16. Observation 4:no need to further discuss inter-RAT LTE measurement without gap, since it has already been supported in R17. |
| R4-2215716 | CMCC | Observation 1: according TS 36.331 and TS 36.133, both NeedForGap (interRAT-NeedForGapsNR) and NCSG can be used for inter-RAT NR measurement. Observation 2: neither RRM requirements for inter-RAT NR measurement with NeedForGap nor RRM requirements for inter-RAT NR measurement with NCSG are specified. Proposal 1: for inter-RAT NR measurement without measurement gap when UE operating in LTE SA, it is proposed to specify requirements for following two cases: - NeedForGap (interRAT-NeedForGapsNR) - NCSG Proposal 2: for inter-RAT LTE measurement, it is proposed to consider following two cases: - Case 1: when LTE CRS to be measured is contained in UE’s active BWP - Case 2: when LTE CRS to be measured is not completely contained in UE’s active BWP, but there is spare RF chain Proposal 3: for inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, it is proposed to support mixed numerology. |
| R4-2215823 | OPPO | Proposal 1: Inter-RAT NR measurements can be applicable to EN-DC and inter-RAT LTE measurement can be applicable to NE-DC. Proposal 2: UE capability could be discussed after the target scenarios are clear. Proposal 3: Consider the scenario when LTE CRS to be measured is contained in UE’s active BWP. Proposal 4: Further discuss whether interruption is required for inter-RAT measurement without gap. Proposal 5: The frequency layers or carrier specific sharing factor for measurement without gaps should be updated due to inter-RAT NR or LTE measurements, e.g., Nfreq or CSSFoutsidegap. |
| R4-2215968 | vivo | Proposal 1: For issue 3-9, option 2 is preferred. However the IE interRAT-NeedForGapsNR could be updated to enable three status like {gap, ncsg, nogap-noncsg} to be provided. Proposal 2: The DSS scenario, i.e., the LTE CRS to be measured are fully contained within a UE’s active BWP, should be considered in the Inter-RAT LTE measurement. Further clarification is needed on whether the scenario where a UE have a spare RF chain used for LTE measurement should be considered in this WI or not. |
| R4-2216338 | Huawei, HiSilicon | Proposal 1: De-prioritize MR-DC for inter-RAT NR (or LTE) measurements without MG. Proposal 2: Support mix numerology scenario for inter-RAT measurement without MG. - Define UE capability for the scenario where LTE within active BWP of NR serving cell. Proposal 3: Do not extend any existing UE capabilities for MG-less inter-RAT measurement. Proposal 4: RAN4 to define requirements for inter-RAT NR measurement without MG based on interRAT-NeedForGapsNR. - FFS whether interruption is allowed when UE reports interRAT-NeedForGapsNR. Proposal 5: RAN4 to define requirements for inter-RAT LTE measurement without MG for the scenario where LTE carrier is within active BWP of NR serving cell. - A new UE capability should be defined to indicate whether UE supports MG-less measurement in this scenario. |
| R4-2216462 | Ericsson | Observation 1: When RAN4 introduced the CRS-IM technology for enhancement DSS in Rel-17, additional performance degradation or network deployment restriction is expected due to inter-RAT LTE measurement within gap. Observation 2: RAN4 had agreed to define requirement for LTE inter-RAT measurement without gap in Rel-17 NCSG. Observation 3: RAN4 introduced searcher limitation for NR measurement, but there is no searcher limitation for LTE measurement. Proposal 1: RAN4 to introduce a separate inter-RAT E-UTRAN measurement without gap for DSS enhancement as follow. - The UE can perform inter-RAT LTE measurements without measurement gaps if the LTE cell’s target to be measured bandwidth are fully within the NR active BWP. Proposal 2: RAN4 to further discuss defining inter-RAT E-UTRAN measurement without gap in which release, Rel-17 NCSG maintenance or Rel-18 inter-RAT measurement without gap. Proposal 3: RAN4 to define inter-RAT E-UTRAN measurement without gap for NCSG in SA only, but for DSS in both SA and MR-DC. Proposal 4: Inter-RAT measurement without gap can be performed in parallel with NR measurement without searcher limitation. Proposal 5: When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, no scheduling restriction is expected. Proposal 6: When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, scheduling restriction is expected, such as UE performing measurements in TDD bands or with different SCS. Proposal 7: RAN4 to discuss how to apply the scheduling restriction based on LTE measurement RSs. Proposal 8: RAN4 needs to further discuss where to perform the inter-RAT E-UTRAN measurement without gaps to reduce the performance degradation to legacy NR intra-frequency measurement without gap and L1-RSRP measurement. Proposal 9: Both NW and UE shall have the same understanding on the measurement occasions for Inter-RAT E-UTRAN measurement without gap. Proposal 10: RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. The effective measurement window can be defined based on measurement duration, measurement periodicity and offset. Proposal 11: The scaling factor for inter-RAT E-UTRAN measurement without gap equals to the total number of frequency layers for E-UTRAN measurement without gap. Proposal 12: RAN4 to discuss whether to introduce the inter-RAT LTE measurements without gap for DSS as release independent from Rel-17. |
| R4-2216483 | ZTE Corporation | Observation 1: During Rel-17 NCSG, the NeedForGapNCSG has been decoupled with NCSG itself. Proposal 1: the MR-DC case can be deprioritized for both inter-RAT NR and inter-RAT LTE. Proposal 2: Since of the mixed numerology scenario for inter-RAT is very possible and perhaps inevitable, we believe the same numerology and mixed numerology should both be supported for inter-RAT measurement without gap. Proposal 3: Re-using the UE reports defined in NCSG for E-UTRA bands to identify inter-RAT measurement with gap or without gap. |
| R4-2216739 | MediaTek inc. | Proposal 1: RAN4 can define requirements for mix numerology inter-RAT measurements without gap. Proposal 2: RAN4 shall not extend the capability of NeedForGaps to inter-RAT LTE measurement and shall not extend the capability of NeedForNCSG to inter-RAT NR measurement. Proposal 3: RAN4 shall define the NeedForGap requirements on top of the NeedForGapNR-r16 capability. Proposal 4: RAN4 shall define the NeedForNCSG requirements on top of the NeedForNCSG-InfoEUTRA capability. |
| R4-2216746 | Qualcomm Incorporated | Proposal 4: No other requirements are needed in Rel-18 for inter-RAT LTE measurement as RAN4 agreed to define the requirements for inter-RAT measurement without gap when UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN in Rel-17. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: Using scenarios

[Moderator notes:

Up to this meeting, all proposed possible using scenarios for inter-RAT NR/LTE measurements without gap can summarized as:

- the inter-RAT NR measurements without gap in Rel18 includes the two scenarios below.

  - Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

  - Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

- the inter-RAT LTE measurements without gap in Rel18 includes the two scenarios below.

  - Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

  - Case b-2: LTE CRS are fully contained within UE’s active BWP

We can use these index below to aviod any confustion. -> optional capablity

]

#### Issue 2-1-1: extend to MR-DC cases

- Proposals

  - Option 1:  [Xiaomi], OPPO

    - inter-RAT NR measurements without gap can be applicable to EN-DC and

    - inter-RAT LTE measurements without gap can be applicable to NE-DC

  - Option 2:  Intel,  Apple, Huawei, ZTE

    - Deprioritize the inter-RAT measurements without gap in MR-DC

  - Option 3:  Ericsson

    - Define inter-RAT E-UTRAN measurement without gap for NCSG(case b-1) in SA only,

    - Define inter-RAT E-UTRAN measurement without gap for DSS(case b-2) in both SA and MR-DC.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | For inter-RAT NR measurements for EN-DC for case a-2, if SSB is completely contained in the active BWP of the UE, it is similar like Rel-16 inter-frequency measurement without MG, and the requirements in 9.3.9 can be used as baseline. This scenario can be considerred. Similar consideration for inter-RAT E-UTRAN measurement without gap for case b-2. While for case a-1 and case b-1, with spare RF chain, NCSG and/or NeedForGap can be considered. But the signaling of NCSG and NeedForGap are not supported for MR-DC. More work is expected. We are open for discussion. In summary, we support to consider at least following cases: - Inter-RAT NR measurements without gap for case a-2 (NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth) can be applicable to EN-DC - inter-RAT E-UTRAN measurement without gap for case b-2 (LTE CRS are fully contained within UE’s active BWP) can be applicable to EN-DC For case a-1 and case b-1, we prefer to consider them and are open for discussion. |
| Huawei | Support option 2. |
| Ericsson | Option 3. We can agree with the 1st bullet for case b-1 and continue the discuss for case b-2. |
| ZTE | Prefer Option 2 to simplify the discussion. Further more, for case a-2 and b-2, since of uncertain active BWP information exchange between MN and SN, it is hard to identify whether inter-RAT without gap is applicable by MN. |
| Apple | Support option 2. The BW can be changed via DCI (BWP switching) or MAC (SCell (de)activation) while the availability of spare RF chain can be changed via MAC (SCell (de)activation) by the other CG. MN and SN may fail to exchange such information timely. |
| Qualcomm | We support option2 |
| Xiaomi | Consdiering the current NCSG and NeedForGap capability dont support MR-DC, we can support option 2. |
| OPPO | Support option 1. Share the same view as CMCC. For case a-2 and b-2, MR-DC scenario could be considered since the target inter-RAT measurement can be covered within UE bandwidth or active BWP. If no consensus is achieved, we can compromise to option 2. |
| Intel | Support Option 2. |
| vivo | Support option 2 |
| CATT | Support option 2. |
| Nokia | We support option 2. |
| MediaTek | We support option 2. |
| Moderator | 10 companies support Option 2. 2 companies support Option 1, 1 company Option 3. But for case a-1 and b-1 companies can be considered them in SA only. |

#### Issue 2-1-2: Numerology

[Moderator notes:

in the last RAN4 meeting, the agreements below was achieved. In this meeting, we can focus on whether the mixed numberology shall be supported or not.

### Issue 3-7: [inter-RAT] Numerology < Agreement >: - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, same numerology is to be supported < Way forward >: FFS mx-numerology case and potential UE capability

]

- Proposals

  - Option 1:  Intel, CMCC, Huawei, ZTE, MTK

    - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, the mixed numerology need to be supported.

      - FFS on whether the additional UE capability is needed

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Support Option 1. Mixed numerology is a common and important cenario. For example, a cell with TDD 30KHz SCS is impacted by LTE CRS with 15KHz SCS, which is a typical deployment for CRS-IM from our point of view. It is proposed to consider mixed numerology. As for the UE capability, we are open for discussion. |
| Huawei | Support option 1. |
| Ericsson | Support option 1. |
| ZTE | Support Option 1. Since LTE only support single 15 kHz. So the mixed numerology scenario for inter-RAT is very possible and perhaps inevitable. |
| Apple | Fine with option 1. |
| Qualcomm | Support option 1. |
| Xiaomi | Fine with option 1. |
| OPPO | Support option 1 |
| Intel | Option 1. And share same view as CMCC this scenario is very typical and important. |
| vivo | OK with option 1 |
| CATT | Fine with option 1. |
| Nokia | Fine with Option 1 |
| MediaTek | Fine with option 1. |
| Moderator | Option 1 can be agreed. |

#### Issue 2-1-3: inter-RAT NR target scenarios

- Proposals

  - Option 1:  Xiaomi, CMCC

    - Case a-1: Another spare RF chain is available for UE and

    - Case a-2: The target RS to be measured is with UE’s active RF chain

[Moderator notes: whether both of scenarios can be indicated by the same or different capability can be FFS in issue2-2-2 and 2-2-3]

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Support Option 1. For case a-1, with spare RF chain, there are two approaches: NCSG and “NeedForGap”, we propose to consider both. From signaling point of view, both NeedForGap (interRAT-NeedForGapsNR) and NCSG can be used for inter-RAT NR measurement. For LTE SA, from orrespond point of view, “NeedForGap” is supported for inter-RAT measurement. interRAT-NeedForGapsNR is used to indicate need for measurement gaps when operating on the E-UTRA band and measuring on the NR band. According to existing TS 36.133, NCSG can be used for inter-RAT measurement. Since from orrespond point of view, both NeedForGap (interRAT-NeedForGapsNR) and NCSG can be used for inter-RAT NR measurement, and orrespondi that NeedForGap or NCSG will result in different requirements, in order to support inter-RAT NR measurement from RAN4 perspective, it is necessary to specify RRM requirements for both cases. |
| Huawei | We suggest to focus on Case a-1. We do not see clear use case for Case a-2. It is noted that Case b-2 is valid use case because we have CRS-IM in NR side in Rel-17, but we do not have such demod enhancement in LTE side. |
| Ericsson | We have the same view as Huawei. Suggest to deprioritize the case a-2. |
| Apple | Case a-1 has already been covered by NeedForGaps in signaling design. If the first sub-bullet means RAN4 shall define inter-RAT requirements for UE capable of interRAT-NeedForGapsNR-r16, we are fine. Case a-2 is also possible in our understanding. If case b-2 is considered valid, from deployment point of view there are both E-UTRAN and NR on the same frequency domain. |
| Qualcomm | We are okay case a-1.. |
| Xiaomi | We support option 1 to consider both case a-1 and case a-2. |
| OPPO | Support option 1 and we are also fine to prioritize case a-1. |
| Intel | In our views, Case a-1 can be covered by current signaling in 36.331 (‘NeedForGapInfoNR’ instead of ‘NeedForNCSG’). But from the requirements perspective, the requirements for case a-1 was absent in TS36.133 so far. That is Case a-1 can be considered. For Case a-2, the new requirements in TS36.133 is still needed. |
| CATT | Fine with option 1 to consider both cases. But for case a-1, should it be the case when UE report FALSE by interRAT-NeedForGapsNR? It seems no RF spare chain is required as pre-condition. |
| Nokia | We can agree to prioritize a-1. Still the the details and conditions should be detailed for a1, and what is the difference in comparison to NCSG that we have already? What are the benefits in comparison to NCSG in LTE. |
| MediaTek | Support Case a-1. We don’t understand which scenario is covered by case a-2. |

#### Issue 2-1-4: inter-RAT LTE target scenario

- Proposals

  - Option 1:  CMCC, Xiaomi

    - Case b-1: when LTE CRS to be measured is not completely contained in UE’s active BWP, but there is spare RF chain and

    - Case b-2: when LTE CRS to be measured is contained in UE’s active BWP

  - Option 1a:  vivo

    - Case b-2: when LTE CRS to be measured is contained in UE’s active BWP

    - FFS on case b-1:when LTE CRS to be measured is not completely contained in UE’s active BWP, but there is spare RF chain

  - Option 2:  Intel, Apple, CATT,OPPO, Ericsson, Huawei, Qualcomm

    - Case b-2 ONLY: Only when LTE CRS to be measured is contained in UE’s active BWP

      - For the inter-RAT LTE gap-less  measurement when UE has the vancant RF chain, the corresponding requirements was defined in Rel17. Thus no need to consider this scenario in Rel18 scope.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | For option 2, we agree that for the inter-RAT LTE gap-less measurement when UE has the vancant RF chain, the corresponding requirements was defined in Rel17, but the requirements are only defined for NCSG for SA operation. If MR-DC is considered (pending on Issue 2-1-1), the orresponding requirements need to be specified. Another consideration is that with spare RF chain, there are two approaches: NCSG and “NeedForGap”. If E-UTRANA interRATNeedForGaps is considered, the requirements need to be specified. |
| Huawei | Support option 2. We understand Case b-1 is agreed to be supported in Rel-17 (Issue 1.3 in R4-2214336). |
| Ericsson | For case b-1, we suggest companies to further check how to handle it. From our understanding, many issues in case b-1 are the same as case b-2, such as scheduling restriction, measurement occasion, CSSF. It’s better not to split the discussion in two places. For case b-2, we’re fine with option 2. |
| ZTE | Prefer Option 2. |
| Apple | Support option 2. Case b-1 has been agreed to be supported in R17 in the last RAN4 meeting. |
| Qualcomm | We support option2. |
| Xiaomi | We support option 1. Share the view with CMCC. For case b-1, the current requirement is defined based on NCSG, we prefer to consider the requirement based on NeedForGap capability. |
| OPPO | Support option 2. |
| Intel | Option 2. For case b-1, from both signaling and requirements perspective, they were already defined beyond Rel17. |
| vivo | Ok with option 2 |
| CATT | Support option 2. Case b-1 is covered by NCSG and is agreed to be defined in R17. |
| Nokia | We support Option 2. Case b-1 is already covered by NCSG. |
| MediaTek | Fine with Option 2. |

### Sub-topic 2-2: UE capabilities

[Moderator notes:

It shall be noted that the main tasks related to RAN4 (listed in the table below) are to investigate how UE to support these features and define the necessary measurement requirements. Based on the general principles, we can also organize the discussion on UE capabilities in the several sub issues below, which are coupled with the using scenarios.

| Using scenarios (sub-topic 2-1) | Capability (sub-topic 2-2) |
| --- | --- |
| Case a-1: Inter-RAT NR wo gap because of the vacant RF chain available | FFS: issue 2-2-1 |
| Case a-2: Inter-RAT NR wo gap because the measurement reference signal can be contained within UE’s LTE bandwidth | FFS: issue 2-2-2 |
| Case b-1: Inter-RAT LTE wo gap because of the vacant RF chain available | FFS: issue 2-2-3 |
| Case b-2: Inter-RAT LTE wo gap because the measurement reference signal can be contained within UE’s active BWP | FFS issue 2-2-4 |

]

#### Issue 2-2-1: On top of which UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain avaiable (Case a-1)

- Proposals

  - Option 1:  MTK

    - NeedforNCSGgap-infoNR

  - Option 2:  CATT,   Xiaomi, Apple, Huawei

    - interRAT-NeedForGapsNR-r16

  - Option 2a:  CATT,

    - interRAT-NeedForGapsNR-r16, e.g., when UE report ‘FALSE’ through interRAT-NeedForGapsNR-r16 can be defined as inter-RAT NR measurement without gaps

  - Option 3: OPPO

    - FFS upon the target scenarios converged.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | We support both Option 1 and option 2. We propose to consider both NCSG and interRAT-NeedForGapsNR-r16. From signaling point of view, both NeedForGap (interRAT-NeedForGapsNR) and NCSG can be used for inter-RAT NR measurement, and consdiering that NeedForGap or NCSG will result in different requirements, in order to support inter-RAT NR measurement from RAN4 perspective, it is necessary to specify RRM requirements for both cases. |
| Huawei | Support option 2 and 2a. On option 1, so far NeedforNCSGgap-infoNR can only be used for intra-NR measurement. |
| Ericsson | We have the same understanding as Huawei. |
| ZTE | Prefer Option 2 and 2a. For Option 1, NeedforNCSGgap-infoNR is used to report the support of NCSG inside NR, not referring to any inter-RAT. |
| Apple | Support option 2 and 2a. As for option 1, shall it be ‘NeedForGapNCSG-InfoNR’ rather than ‘NeedforNCSGgap-infoNR’? if so, current NeedForGapNCSG-InfoNR is an NR IE used for intra-NR measurement. Does option 1 mean we need to introduce a new structure in LTE spec? |
| Qualcomm | We support option2 and 2a. |
| Xiaomi | We support option 1 and option 2. For option 1, we suggest to extend the capability of NeedForGapNCSG for allowing UE indicating gap information for inter-RAT NR measurement. Based on our knowledge, E-UTRAN supports NCSG gap in Rel-14, we think it is applicable to consider to extend the capability of NCSG to inter-RAT NR measurement. |
| OPPO | Prefer option 2, and also open to option 1 with some signaling extension. |
| Intel | Option 2. For Option 1, in LTE, there is not such message to support NCSG for NR. So does it mean the complete new message shall be introduced in LTE? |
| vivo | Ok with option 2 and 2a |
| CATT | Support option 2 and 2a. Share the similar view as Apple and Intel that NeedForGapNCSG is a NR IE and not applied in LTE. |
| Nokia | Option 2 |
| MediaTek | Need further time to check. |

#### Issue 2-2-2: On top of which UE capability to support the inter-RAT NR measurement without gap when the measurement reference signal can be contained within UE’s LTE channel bandwidth (Case a-2)

- Proposals

  - Option 1:  MTK

    - Same capability indication as for case a-1 (NeedforNCSGgap-infoNR)

  - Option 2:  CATT, Xiaomi, Apple, Huawei

    - Same capability indication as for case a-1 ( interRAT-NeedForGapsNR-r16, e.g., when UE report ‘FALSE’ through interRAT-NeedForGapsNR-r16 can be defined as inter-RAT NR measurement without gaps)

  - Option 3: OPPO

    - FFS

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | We are not sure whether the UE capability need to be coupled with NCSG or NeedForGap. The scanrio (a-2) is that measurement reference signal is contained within UE’s LTE channel bandwidth, in our view, no spare RF chain is needed. |
| Huawei | Support option 2. With option 2, we mean no separate requirement or UE capability would be defined for Case a-2. |
| Ericsson | We have the same understanding as Huawei. |
| ZTE | Prefer Option 2. |
| Apple | Option 2 can work. We are open to further discussion on whether to introduce another UE capability if the UE doesn’t support NeedForGap. |
| Qualcomm | Option2 |
| Xiaomi | For case a-2, we prefer to define requirement based on NeedForGap capability,as the NCSG always assumes UE has another spare RF chain. |
| OPPO | Support option 2. |
| Intel | Option 2. |
| CATT | It is a little confused whether the capability is still needed in this case. We understand it seems not needed. We think case a-1 and a-2 are independent cases to define inter-RAT measurement without gap, one is based on the signaling report and the other one is based on bandwidth limit. |
| Nokia | Case a-2 can be deprioritized. |
| MediaTek | Need further time to check. |

#### Issue 2-2-3: On top of which UE capability to define the inter-RAT LTE measurement requirements when UE has vacant RF chain avaiable(Case b-1)

- Proposals

  - Option 1:  Intel, Xiaomi, CATT, ZTE,

    - Reuse the existing capability. UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Except NeedForNCSG, we propose to also consider NeedForGap. We would like to hear companies’ view to support case b-1 with NeedForGap. |
| Huawei | Support option 1. |
| Ericsson | Support option 1. |
| ZTE | Support Option 1. |
| Apple | Option 1 has already been agreed in RAN4#104e in NCSG discussion. |
| Qualcomm | Option1. Same as Apple |
| Xiaomi | We support to consider both NeedForGap and NCSG capability. In our view, NeedForGap capability also support the scenario where UE has another spare RF chain. |
| OPPO | If case b-1 is supported, we can support option 1. |
| Intel | Option 1 |
| vivo | Option 1 |
| CATT | Support option 1. |
| Nokia | We support CMCC’s view. We propose to also consider NeedForGap. |
| MediaTek | Need further time to check. |

#### Issue 2-2-4: On top of which UE capability to define the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

- Proposals

  - Option 1:  Intel, Apple

    - the capability of NeedForGaps for inter-frequency measurement wo gap in Rel16 can be taken as the baseline

  - Option 1a:  Xiaomi, MTK

    - extend the capability of NeedForGaps

  - Option 2: Huawei, Ericsson

    - A new UE capability should be defined to indicate whether UE supports MG-less measurement in this scenario.

  - Option 3:  OPPO

    - FFS

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | Same comment as Issue 2-2-2. We are not sure whether the UE capability need to be coupled with NCSG or NeedForGap. The scanrio (a-2) is that measurement reference signal is contained within UE’s LTE channel bandwidth, in our view, no spare RF chain is needed. |
| Huawei | Support option 2. In our view, Case b-2 is similar to Rel-16 inter-frequency without gap, so it is reasonable to define a separate capability for it. |
| Ericsson | Option 2. Same view as Huawei. |
| ZTE | We are open to all options. |
| Apple | Support option 2. |
| Qualcomm | We are okay to have an optional capability for option2. |
| Xiaomi | We support option 1a, the current NeedForGap capability could also support the scenario b-2. We support to extend the capability of NeedForGaps to allowing UE indicating gap information for inter-RAT LTE measurement. |
| OPPO | Support option 2. |
| Intel | Option 1. Let’s clarify the our understanding on capability indications defined by the option 1 ,1a and 2 more. - Option 1 and 1a: ‘NeedForGapInfoNR’ { interRAT-needForGap-r18[ext] }. - Option 2: ‘NeedForGapInfoNR’ { NEW capability[TBD] }. If the approaches above is correct, I supposed that these two options are same. |
| CATT | Same comments as issue 2-2-2. Is the capability still needed when the bandwidth is limited within the active BWP? |
| Nokia | FFS |
| MediaTek | Need further time to check. |

### Sub-topic 2-3: Measurement requirements

#### Issue 2-3-1: General principle to define the requirements

- Proposals

  - Proposal 1:  CATT,

    - For the inter-RAT NR measurements without gap, the  requirements frameworks can be based on NR inter-frequency measurement without gap

  - Proposal 2: CATT, Xiaomi

    - For the inter-RAT LTE gap-less measurement when LTE CRS to be measured is contained in UE’s active BWP, the requirements can be based on the measurement requirements for LTE inter-frequency measurement.

  - Proposal 3: Ericsson

    - RAN4 needs to further discuss where to perform the inter-RAT E-UTRAN measurement without gaps to reduce the performance degradation to legacy NR intra-frequency measurement without gap and L1-RSRP measurement.

    - Both NW and UE shall have the same understanding on the measurement occasions for Inter-RAT E-UTRAN measurement without gap.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | How to define the requirements are related with scenarios. For the scenario that RS is within UE bandwith, the Rel-16 inter-f measurement without gap can be used as baseline. For the scenario that there is spare RF chain, requirements for NCSG or NeedForGap are needed. |
| Huawei | On P1, in general fine to take NR inter-frequency without gap as baseline, but one difference is that the number of samples should be 8. On P2, in general fine to take LTE inter-frequency as baseline, and we understand it refers to LTE inter-frequency without gap. On P3, the first bullet needs more discussion. In our view, as baseline the LTE MOs to be measured without MG should be counted CSSFoutside_MG together with other NR Mos that can be measured without MG, no matter when UE takes measurement on the LTE Mos (there is no SMTC for LTE measurement and UE can take any 5ms duration for LTE measurement). The second bullet of P3 may be meaningful when the LTE measurement causes scheduling restriction, so we are open to further study it. |
| Ericsson | P1. Same view as Huawei. P3. From our understanding, inter-RAT LTE measurement is different with NR measurement. It can be performed in any places. Furthermore, there is no searcher limitation for LTE measurement. Thus, we suggest to measure inter-RAT LTE measurement outside any SMTC with a separate scaling factor as LTE requirement. |
| ZTE | Share similar view as CMCC. |
| Apple | Can be revisited after RAN4 concludes on support of scenarios. |
| Xiaomi | Share the view with Apple to wait for the onclusion of scenarios. |
| OPPO | Agree with CMCC that the requirements depends on scenarios. |
| Intel | Share same view as CMCC. So it is better to postponed after sub-topic 2-1 2-2 converged at least |
| vivo | Similar view that wait for conclusion on the supporting scenarios |
| CATT | Support proposal 1 and 2. But for proposal 2, we would like to remind that it seems there is no inter-frequency measurement without gap in LTE. So actually the measurement requirements need further study. |
| Nokia | FFS |
| MediaTek | Share same view as Apple. |

#### Issue 2-3-2: Scheduling restriction

- Proposals

  - Proposal 1:  Intel

    - The existing scheduling availability specified for intra-frequency measurements in TS 38.133 section 9.2.5.3 can also be applied to the inter-RAT measurement without measurement gaps as a start point.

  - Proposal 2: Ericsson

    - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, no scheduling restriction is expected.

    - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, scheduling restriction is expected, such as UE performing measurements in TDD bands or with different SCS.

    - RAN4 to discuss how to apply the scheduling restriction based on LTE measurement RSs.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | On P1, suggest FFS, we understand scheduling restriction for measurement with NCSG in cl. 9.3.10.3 may be more relevant for inter-RAT measurement. On P2, the first bullet may not be always true, e.g. UE may not support simultaneous Tx and Rx for the band pair of NR serving cell and LTE target carrier. The last two bullets are fine. |
| Ericsson | We’re fine with P2 plus Hauwei’s complement for simultaneous Tx and Rx capability. |
| ZTE | Need futher study. |
| Intel | In our view, the similar high level principle to considered scheduling restriction can be used for this objective. E.g. the cause leads the scheduling for NCSG. But we are fine to FFS. |
| Noia | FFS |
| MediaTek | Need further study too. |

#### Issue 2-3-3: Searcher limitation

- Proposals

  - Proposal 1:  Ericsson

    - Inter-RAT measurement without gap can be performed in parallel with NR measurement without searcher limitation.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | We understand P1 is same as the first bullet of P3 of Issue 2-3-1. It would be good if we can merge the discussion to avoid duplication. As we commented for Issue 2-3-1: In our view, as baseline the LTE MOs to be measured without MG should be counted CSSFoutside_MG together with other NR Mos that can be measured without MG, no matter when UE takes measurement on the LTE Mos (there is no SMTC for LTE measurement and UE can take any 5ms duration for LTE measurement). |
| Ericsson | Support P1. From our understanding, this is a general issue. For example, even if the UE performs the measurement within SMTC, it mean no searcher sharing issue between LTE MO and NO MO. Issue 2-3-1 is one of the possible results following this proposal. |
| ZTE | Proposal 1 is closely related with the assumption of case. For case -2, maybe the UE can not perform both inter-RAT measurement without gap and NR measurement in parallel. |
| Apple | Take inter-RAT E-UTRAN measurement within NCSG for example. LTE Mos are counted in CSSFwithin_ncsg,i together with intra/inter-frequency measurement within NCSG. UE is not expected to conduct inter-RAT measurement and intra-NR measurement simultaneously. |
| Qualcomm | We do not support this. For NR-SA, UE will tune to measure neighbor LTE when there is no spare chain. |
| Intel | Need to be clarified. E.g. whether such condition(P1) is valid for inter-RAT LTE measurement. Can be FFS when RAN4 to discuss CCSF issue. |
| CATT | Can be FFS, should differentiate inter-RAT NR measurement and inter-RAT LTE measurement. |
| Nokia | FFS |
| MediaTek | Similar view to QC. |

#### Issue 2-3-4: CCSF

- Proposals

  - Proposal 1:  Intel, OPPO

    - The updates of CSSF requirements when these inter-RAT measurements without gap introduced (e.g. CSSF_outside_gap) is needed

  - Proposal 2: Ericsson

    - The scaling factor for inter-RAT E-UTRAN measurement without gap equals to the total number of frequency layers for E-UTRAN measurement without gap.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | P1 is fine. We understand P2 is same as the first bullet of P3 of Issue 2-3-1. It would be good if we can merge the discussion to avoid duplication. As we commented for Issue 2-3-1: In our view, as baseline the LTE MOs to be measured without MG should be counted CSSFoutside_MG together with other NR MOs that can be measured without MG, no matter when UE takes measurement on the LTE MOs (there is no SMTC for LTE measurement and UE can take any 5ms duration for LTE measurement). |
| Ericsson | P2. It’s better to solve some general issues such as 2-3-1 and 2-3-3 firstly. |
| Apple | P1 is fine. Regarding P2, we have similar comments as under issue 2-3-3. |
| Qualcomm | We support P1. |
| OPPO | Support proposal 1. |
| Intel | Support the P1 in which the general factor to impact the measurement requirements was identified firstly. But we are fine FFS it. |
| vivo | Ok with P1 |
| CATT | Fine with proposal 1. |
| Nokia | FFS |
| MediaTek | Fine with P1 |

#### Issue 2-3-5: Interruption requirements for inter-RAT measurement without gap.

- Proposals

  - Option 1:  OPPO

    - FFS

  - Option 1a: Huawei

    - FFS on whether interruption is allowed for inter-RAT NR measurements without MG when UE reports interRAT-NeedForGapsNR.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | Support option 1a. Maybe conclusion from Issue 1-1-1 can be re-used for inter-RAT NR measurement, but we are open to further discussion. |
| Ericsson | FFS |
| Apple | Open for further study. For case a-1 maybe we can reuse outcome of issue 1-1-1. |
| Qualcomm | We are open to discuss. FFS |
| Intel | Can be FFS. |
| MediaTek | FFS. |

#### Issue 2-3-6: Effiective measurement window

- Proposals

  - Proposal 1:  Ericsson

    - RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. The effective measurement window can be defined based on measurement duration, measurement periodicity and offset.

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| Huawei | In general we are fine with P1 but details can be FFS. We can see the point to have such effective measurement window for defining clear scheduling restriction requirements for inter-RAT LTE measurement. |
| Ericsson | Support P1. We can discuss the detail once we have some agreements in general pricinples. |
| ZTE | We are not sure why we need to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. So as to specify the duration of scheduling restriction ? |
| Apple | We expect requirements case b-1 can be finalized in R17. Regarding case b-2, we not so sure if P1 is necessary. |
| Qualcomm | It is not clear about P1. We are open to discuss. |
| OPPO | Open to discuss. |
| Intel | We thought more clarification needed. |
| CATT | More details are needed. |
| Nokia | More details needed |
| MediaTek | We are not if we understand the proposal, more details needed. |

#### Issue 2-3-7: Release independent requirements

- Proposals

  - Proposal 1:  Ericsson,

    - RAN4 to discuss whether to introduce the inter-RAT LTE measurements without gap for DSS (case b-2) as release independent from Rel-17

- Recommended WF

  - Collect company views in the 1st round.

| Company | Comments |
| --- | --- |
| CMCC | We support proposal 1. CRS-IM is supported from Rel-17. From our point of view, inter-RAT LTE measurement without MG is benificail for CRS-IM, which could reduce the nterruption due to measurement for requiring parameters for CRS-IM. |
| Huawei | We suggest to postpone the issue after we have stable requirements for Case b-2. |
| Ericsson | We support P1. We’re also fine to postpone the issue until we have a stable requirement for case b-2. |
| Apple | Same view as HW that this can be revisited after case b-2 is stable. |
| Qualcomm | We are open to discuss |
| Intel | Can be FFS. |
| vivo | FFS |
| Nokia | FFS |
| MediaTek | Same view as HW. |

## CRs/TPs comments collection

Moderator: No CR/TP in this AI

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Status summary

Issue 2-1-1: extend to MR-DC cases Status: 10 companies support Option 2. 2 companies support Option 1, 1 company Option 3. For case a-1 and b-1 companies can be considered them in SA only. Tentative agreements: - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-1 in SA only if case a-1 is agreed as one of inter-RAT NR target scenario (issue 2-1-3) - Define capability and/or requirements to support inter-RAT E-UTRAN measurement without gap case b-1 in SA only if case b-1 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4) - ,FFS on : - Define capability and/or requirements to support inter-RAT NR measurement without gap case a-2 in in both SA and MR-DC if case a-2 shall be considered in as one of inter-RAT NR target scenario (issue 2-1-3). - Define capability and/or requirements to inter-RAT E-UTRAN measurement without gap case b-2 in both SA and MR-DC if case b-2 is agreed as one of inter-RAT LTE target scenario (issue 2-1-4) Recommendations for 2nd round: The open issues can be further discussed in WF for 2nd round discussion.

#### Issue 2-1-2: Numerology Status: all companies can support Option 1. Tentative agreements: N.A - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, the mixed numerology need to be supported. - FFS on whether the additional UE capability is needed Recommendations for 2nd round: No further discussion needed in this meeting.

#### Issue 2-1-3: inter-RAT NR target scenarios Status: No agreements. The position to support the target scenario can be summarized as below. - Case a-1 (Another spare RF chain is available for UE) supported by : - CMCC, Huawei, Ericsson, Apple, Qualcomm,Xiaomi, OPPO, Intel, CATT, Nokia, MTK - Case a-2 (The target RS to be measured is with UE’s active RF chain) supported by: - CMCC, Apple, Xiaomi, [OPPO], Intel, CATT Tentative agreements: - Case a-1 (Another spare RF chain is available for UE) can be considered as the inter-RAT NR target scenario. - FFS on Case a-2 Recommendations for 2nd round: Can be further discussed in GTW. From the moderator perspective, we thought the criteria to justify the target scenarios in this WI are based on: - Whether is there any new UE capability need to support this aspect needed in Ror - Whether is there any new or updated RRM requirements (e.g. measurement report delay, …) for UE to support this aspect

#### Issue 2-1-4: inter-RAT LTE target scenario Status: No agreements. The companies’ positions to support the target scenarios can be summarized as below. - Case b-1 (UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements) supported by : - CMCC, Xiaomi, - Case b-2 (The target CRS to be measured is with UE’s active BWP) supported by: - CMCC, Huawei, Ericsson, ZTE, Apple, Qualcomm, Xiaomi, OPPO, Intel, vivo, CATT, Nokia, MTK Tentative agreements: - Case b-2(The target CRS to be measured is with UE’s active BWP) shall be considered the inter-RAT LTE scenario under objective 2 of NR_MG_ehn2-core WI - FFS on Case b-1 Recommendations for 2nd round: Can be further discussed in GTW.

#### Issue 2-2-1: On top of which UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain avaiable (Case a-1) Status: Almost all companies can support Option 2. 1 company suggest to FFS. Tentative agreements: - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1) - The exact value to be reported can be FFS. Recommendations for 2nd round: The tentative agreements above can be discussed in WF in 2nd round.

#### Issue 2-2-2: On top of which UE capability to support the inter-RAT NR measurement without gap when the measurement reference signal can be contained within UE’s LTE channel bandwidth (Case a-2) Status: 10 companies can support Option 2. 2 companies suggest to be FFS. Tentative agreements: - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-2) - The exact value to be reported can be FFS. Recommendations for 2nd round: The tentative agreements above can be discussed in WF in 2nd round

#### Issue 2-2-3: On top of which UE capability to define the inter-RAT LTE measurement requirements when UE has vacant RF chain avaiable(Case b-1) Status: 10 companies can support Option 1. 2 companies proposed to use “NeedForGap”. 1 company suggest FFS.. Tentative agreements: - Reuse the existing capability. UE indicates ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement(case b-1) Recommendations for 2nd round: The tentative agreements above can be discussed in WF in 2nd round. But from moderator perspective, we thought Option 1 was agreed in last meeting.

#### Issue 2-2-4: On top of which UE capability to define the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2) Status: Companies have some diverse understanding on these options.. Tentative agreements: NA Recommendations for 2nd round: Further discussion on the candidate options below in WF in 2nd round - Option 1: - the capability of NeedForGaps for inter-frequency measurement wo gap in Rel16 can be taken as the baseline - Option 1a: - extend the capability of NeedForGaps - Option 2: - A new UE capability should be defined to indicate whether UE supports MG-less measurement in this scenario

#### Issue 2-3-1: General principle to define the requirements Status: The views on these proposals are quite variant. Tentative agreements: N.A Recommendations for 2nd round: Postponed to next meeting. The candidate options below can be captured in WF - FFS on the general principle to define the requirements - Proposal 1: CATT, - For the inter-RAT NR measurements without gap, the requirements frameworks can be based on NR inter-frequency measurement without gap - Proposal 2: CATT, Xiaomi - For the inter-RAT LTE gap-less measurement when LTE CRS to be measured is contained in UE’s active BWP, the requirements can be based on the measurement requirements for LTE inter-frequency measurement. - Proposal 3: Ericsson - RAN4 needs to further discuss where to perform the inter-RAT E-UTRAN measurement without gaps to reduce the performance degradation to legacy NR intra-frequency measurement without gap and L1-RSRP measurement. - Both NW and UE shall have the same understanding on the measurement occasions for Inter-RAT E-UTRAN measurement without gap. - Others are not precluded

#### Issue 2-3-2: Scheduling restriction Status: The views on these proposal are quite variant. Tentative agreements: N.A Recommendations for 2nd round: Postponed to next meeting. The candidate options below can be captured in WF - FFS on the scheduling restriction - Proposal 1: Intel - The existing scheduling availability specified for intra-frequency measurements in TS 38.133 section 9.2.5.3 can also be applied to the inter-RAT measurement without measurement gaps as a start point. - Proposal 2: Ericsson - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, no scheduling restriction is expected. - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, scheduling restriction is expected, such as UE performing measurements in TDD bands or with different SCS. - RAN4 to discuss how to apply the scheduling restriction based on LTE measurement RSs. - Others are not precluded

#### Issue 2-3-3: Searcher limitation Status: The technical proposal needs more clarifications. . Tentative agreements: N.A Recommendations for 2nd round: Postponed to next meeting. The candidate options below can be captured in WF - FFS on the searcher limiation - Proposal 1: Intel - The existing scheduling availability specified for intra-frequency measurements in TS 38.133 section 9.2.5.3 can also be applied to the inter-RAT measurement without measurement gaps as a start point. - Proposal 2: Ericsson - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, no scheduling restriction is expected. - When the target inter-RAT E-UTRAN frequency layers belong to an inter-band with the serving cells, scheduling restriction is expected, such as UE performing measurements in TDD bands or with different SCS. - RAN4 to discuss how to apply the scheduling restriction based on LTE measurement RSs. - . - Others are not precluded

#### Issue 2-3-4: CCSF Status:. 8 companies support P1. 1 company support P2. 1 company suggest to be FFS. But P1 and P2 are not exclusive indeed. As moderator we suggest to agree P1 as the high level principle. Tentative agreements: - The updates of CSSF requirements when these inter-RAT measurements without gap introduced (e.g. CSSF_outside_gap) is needed - FFS on: - The scaling factor for inter-RAT E-UTRAN measurement without gap equals to the total number of frequency layers for E-UTRAN measurement without gap. Recommendations for 2nd round: The tentative agreements above can be captured in WF for 2nd round email discussion.

#### Issue 2-3-5: Interruption requirements for inter-RAT measurement without gap. Status: Many companies suggest to be FFS.. Tentative agreements: NA Recommendations for 2nd round: Postponed to next meeting. The candidate options below can be captured in WF - FFS on whether interruption is allowed for inter-RAT NR measurements without MG - Others are not precluded

#### Issue 2-3-6: Effiective measurement window Status:. Many companies suggest to be FFS.. Tentative agreements: NA Recommendations for 2nd round: Postponed to next meeting. The candidate options below can be captured in WF - FFS on: RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. The effective measurement window can be defined based on measurement duration, measurement periodicity and offset. - Others are not precluded

#### Issue 2-3-7: Release independent requirements Status:. Many companies suggest to be FFS.. Tentative agreements: NA Recommendations for 2nd round: Postponed to next meeting. The candidate options below can be captured in WF - FFS on RAN4 to discuss whether to introduce the inter-RAT LTE measurements without gap case b-2 as release independent from Rel-17 - Others are not precluded

### CRs/TPs

Moderator: No CR/TP in this AI

## Discussion on 2nd round (if applicable)

The discussions in the WF are captured below

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
| TBD | WF on Measurement without gaps for UEs reporting NeedForGapsInfoNR | Intel |  |
| TBD | WF on inter-RAT measurement without gap | Intel |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents