3GPP TSG-RAN WG4 Meeting #104bis-e												R4-2216977

Electronic Meeting, 10–19 October 2022

Agenda item:			6.23.4

Source:	Moderator (Huawei)

Title:	Email discussion summary for [104bis-e][141] NR_cov_enh2_part1

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Collect views on proposals in each of the contribution and aim at making the scope of the study more concrete

  - Since this is the 1st meeting for Rel-18 CE, see if there are any agreements to be made.

- 2nd round: Continue the discussion on the 1st round if necessary based on the 1st round outcome so that further inquiries are provided to make the agreements more specific and detailed

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei, Hisilicon | Xiang Gao | gaoxiang74@huawei.com |
| Nokia | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| Samsung | Yuanyuan Zhang | Tina55.zhang@samsung.com |
| Apple | James Wang | fucheng_wang@apple.com |
| OPPO | Jinqiang | xingjinqiang@oppo.com |
| Xiaomi | Shengxiang Guo | guoshengxiang@xiaomi.com |
| Skyworks Solutions Inc. | Dominique Brunel | Domnique.brunel@skyworksinc.com |
| ZTE | Wubin Zhou | zhou.wubin@zte.com.cn |
| vivo | Sanjun Feng | fengsanjun@vivo.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Clarification on the ”Enhancements of increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on ”Increasing UE power high limit for CA and DC” in compliance with relevant regulations”

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215793 | Nokia, Nokia Shanghai Bell | This contribution briefly discussed a difference between inter band UL CA and intra band UL CA in terms of power high limit. If power high limit feature is extended to intra band UL CA, at least following aspects need to be considered. - Unlike inter band UL CA, the sum of PC per band within band combination cannot be directly used since there is only one band within intra band UL CA - Even if PC per power source, e.g., PA is clarified somehow, which power source covers which CC(s) may need to be clarified specifically for intra band non-contiguous UL CA - MPR/A-MPR evaluation would be needed. |
| R4-2215892 | ZTE Corporation | Observation 1. Only the eligible CA configuration with power configuration of 23dBm (TDD or FDD band) + 26dBm (TDD band) was supported after Rel-17 discussion. Observation 2. The 10 log10∑pPowerClass,c should exceed UE power class for uplink inter-band CA Proposal 1. To include more power configurations cases in Rel-18 power domain enhancements. Proposal 2. ‘PC2:26dBm(TDD)+ 26dBm(TDD) without UL-MIMO’ could be incorporated in the enhancement scope. Observation 3. Little RAN4 specification impacts are expected if more power configurations cases in Rel-18 power domain enhancements are introduced. |
| R4-2216120 | Vivo | Observation 1: Current “increasing UE power high limit” related first sub-bullet of RAN4 leading objective is still temporary and pending on revision. Observation 2: The tentative RAN4 leading objective discussed in RAN#96 seems either not needed in current stage. Proposal: Remove the following objective from the WID: “Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)”. |
| R4-2216149 | Xiaomi | Observation 1: if only 2PA is assumed, the possible need of the capability of HigherPowerLimitCADC is only for the PC configuration that PC3+PC2 with PPowerClass,CA=PC2. Observation 2: For PC configuration that PC3+PC2 with PPowerClass,CA=PC2, the capability of HigherPowerLimitCADC could not apply to PC2(FDD)+ PC3 (FDD/TDD) in R17. Observation 3: if PC3+PC1.5 is considered, 3PA architecture should be studied. |
| R4-2216441 | OPPO | Observation 1: In Rel-17, increasing UE power high limit feature is only for PC2 total power band combinations with PC3 TDD/FDD + PC2 TDD. Observation 2: The Rel-17 increasing UE power high limit feature only increase UE Tx power but keep the emission requirements unchanged. If UE cannot meet the emission requirements with higher Tx power then it shouldn’t indicate the higherPowerLimit-r17 capability. Proposal 1: Consider extending higherPowerLimit-r17 feature to more cases: - Case 1: PC3 band combinations with PC5 band1 + PC3 band2 - Case 2: PC2 intra-band UL CA band combinations with PC3 CC1 + PC2 CC2 - Case 3: PC2 single band UL MIMO with PC3 PA + PC2 PA - Case 4: 3Tx scenario in the future |
| R4-2216588 | Huawei, HiSilicon | Observation: There seems no other aspect can be enhanced since RAN4 have finished the work on increasing UE power high limit for CA and DC in Rel-17. Proposal 1: What could be further enhanced to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC” in Rel-18 should be clarified. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: The neccesity of this objective

Sub-topic description:

Further clarification to the objective on enhancements to realize increasing UE power high limit for CA/DC based on the outcomes of Rel-17 seems to be considered as a necessary action point among companies, while R4-2216120 (Vivo) proposes to remove this objective.

Open issues and candidate options before e-meeting:

- Issue 1-1-1: Whether to remove this objective

- Proposal:

  - Option 1: Yes, due to the following analysis on the tentative objectives: (Vivo)

    - PC 2, 23dBm + 23dBm: FDD+FDD, without UL-MIMO. It is now precluded in Rel-17 FDD HPUE basket WIs, and currently the WID basket for NR inter-band CA/DC with high power on FDD bands approved in RAN#97-e has not included the FDD HPUE in the uplink.

    - PC 1.5, 26dBm + 26dBm: FDD+FDD, without UL-MIMO. It is even more challenge than PC2 FDD HPUE, this is too early to be discussed.

    - Intra-band non-contiguous CA. Since there is quite limited band combinations (CA_n41(2A), CA_n77(2A) and CA_n78(2A)) for intra-band non-contiguous, and PC 1.5 has already been supported for band n41, n77 and n78, it seems that increasing CA power limit is not needed for this case.

  - Option 2: No, but further clarification is needed.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 2 At this stage, we don’t see the reason to remove the objective itself while at least 1st and 2nd sub-bullets are nothing related to this power high limit feature since the UEs supporting them just indicate PC2 and PC1.5, respectively. It’s noted that RAN4 requires specific non-spectrum related WIs to develop the requirements for PC2 and PC1.5 for FDD+FDD. Something new for the future must be intra band UL CA. It should be still kept as one of the options. |
| Samsung | Option1, since we do not see the necessity of enhancement or any higher power BC request from operators should be handled in this WI. We agree the observations from vivo, particularly for intra-band NC UL CA, since there are already many kinds of good performance PC2 PA for n41/n77/n78 available in the market, we agree directly define PC1.5 is more promising if in future there is operators’ request as well as the feasibility should be proven first. New PC is better to be handled in a new SI/WI rather than under this WI. |
| Apple | Option 1 The UE power high limit for CA and DC feature can be handled together with other HPUE WIDs for band combinations. It is not necessary to be included in this WID. Otherwise, all other HPUE related features can be included in coverage enhancement WID. |
| OPPO | Option 2. Change WID needs to be discussed in RAN plenary. In our view, at least the below cases need to be considered further:  Case 1: PC3 band combinations with PC5 band1 + PC3 band2  Case 2: PC2 intra-band UL CA band combinations with PC3 CC1 + PC2 CC2  Case 3: PC2 single band UL MIMO with PC3 PA + PC2 PA  Case 4: 3Tx scenario in the future |
| Xiaomi | From our understanding, the following potential PC2 +PC3 configuration may subject to further consideration in this WID, however there are still some issue as mentioned in the bracket. For other PC configuration without PC1.5, traditional PPowerClass,CA could be indicated if needed. Case 1: PC2 (FDD with) without TxD+ PC3 (FDD/TDD) inter-band CA (noted currently there is no such combination for this case in the spec) Case 2: PC2 (CC1)+ PC3 (CC2) intra-band CA (for this case, how to sum of the power class as there is no CC power class per band) Case 3: PC2 + PC3 UL MIMO (how to sum of the power class as there is no PA configuration reporting) For the PC2+PC1.5 or PC3+PC1.5, we think this should be deprioritized as 3Tx has not been discussed in RAN4. |
| Huawei | Option 1. Since the Rel-17 RAN4 WI has been finished, 23dBm + 26dBm has been enabled for the TDD/FDD + TDD inter-band combo, we don’t see any other combination that requires further discussion in Rel-18, |
| Meta | Option 1. We have similar view with vivo and Samsung. Our priority is to remove the unclear the objectives. If it is not possible, then it would be revised to update the objectives with the exact usage and power combinations in RAN plenary. |
| Skyworks | The WI is clear that the RAN4 should be based on the R17 increased power work which was only for inter-band cases. Intra band should not be in scope. R17 already covers PC2 increased power and increased power only applies to cases where a given power class is exceeded so in our view there is only a few inter-band cases in scope for R18: PC3 increased power based on PC3+PC5 of NR+NRU PC1.5 increased power based on PC1.5+any PC but this calls for 3 transmitters and thus could only apply in certain cases and only for 1CC for bath bands. |
| ### ZTE | ### If we only focus on the three cases mentioned in Option 1, then the first two cases could be done with HPUE WID, as commented by apple. And for the last case, the existing PC1.5 intra-band UL CA basket WID could cover. |
| ### vivo | ### Option 1. ### We do not think the current raised cases are quite reasonable to be considered in this WI: For PC5+PC3, currently we do not see a clear need; For Intra-band case, PC 1.5 is a much more straight forward extension; For single band UL-MIMO, there are power splitting issue and MPR and quite different from the R17 WI; ### Admittedly, removing a WID scope is strictly speaking a RAN level discussion. However, this objective is an unclear and not an implementable one. RAN4 can at least have a WF and/or other means to discuss this scope and express RAN4’s understanding on what is possible for the next stage. |
| ### Ericsson | Option 1. FDD + FDD with PC2 can be indicated with existing signaling, power-class fallback is the issue, but common for all HPUE FDD + FDD with PC1.5 appears less viable. For intra-band UL CA, the Pcmax,c per serving cell c is the same as the PCMAX for the band combination. This is a complication intra-band NC of PC1.5 that should be considered. We can focus on other objectives of the WID like Enhancements that achieve better utilization of UL power (RAN1, RAN4) |

Sub-topic 1-2: Inter-band CA/DC scenario for Rel-18 RAN4 study

Sub-topic description:

R4-2215892 (ZTE), R4-2216149 (Xiaomi) and R4-2216441 (OPPO) have proposals towards their understanding about the applicability of Rel-17 RAN4 work and the example cases that could be further considered in Rel-18 RAN4 study.

Open issues and candidate options before e-meeting:

- Issue 1-2-1: Whether ‘PC2: 26dBm (TDD) + 26dBm (TDD) without UL-MIMO’ needs to be considered

- Proposals:

  - Option 1: Yes, and limited spec impacts are expected since the only changes seems to be the updates for the NOTE 7 in Table 6.2A.1.3-1 of TS 38.101-1. (ZTE)

  - Option 2: No, since PPowerClass,CA =PC 1.5 could be requested for this case. (Xiaomi)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 2: we haven’t seen the reason to apply power high limit feature to this scenario, i.e., 26dBm (TDD)+26dBm (TDD). The UE just reports PC1.5. As we commented in Issue 1-1-1, in order to develop the requirements for this scenario, RAN4 needs a non-spectrum related WI. If there is no simultaneous UL and DL between two bands, i.e., so-called synchronized scenario, the required spec change may be very limited. Nevertheless, RAN needs to approve this new WI. If the corresponding WI is established, the WI would need a scenario where PC2(TDD)+PC2(TDD) for UL and PC2(TDD)+PC2(TDD)+PC3(FDD) for DL, where IMD due to TDD bands falls into the FDD DL. Then, we’ll see a new MSD requirement. |
| Samsung | Option 2 Currently we do not see the request from operators, if indeed have request in future, a new SI/WI is supposed to be established to fully discuss the feasibility and the impacted requirements, rather than under this WI. |
| Apple | Option 2 According to RAN4 consensus, 26dBm (TDD) + 26dBm (TDD) should be PC1.5 instead of PC2 with UE power high limit capability. |
| OPPO | Option 2. |
| Xiaomi | Option 2 |
| Huawei | Option 2 We also think PC 1.5 can serve this purpose. |
| Meta | Option 2. |
| Skyworks | Option 2 |
| ZTE | Our proposal is option 1. We understand the comments from other companies. However, the current HPUE basket WID only includes PC1.5 with single UL TDD band, it doesn't include PC1.5 with 26dBm TDD+26dBm TDD UL CA. And in our understanding, the CA band combination with sum power of each band exceed the UE power class can be seen as eligible CA band combination. So for PC2 UE with the CA combination of 26dBm TDD+26dBm TDD, if this UE reports the highpowerlimit signalling, then this UE can be up to 29dBm by lifting the PA output power, which is similar with PC2 23+26dBm discussed in Rel-17. But if there is HPUE basket WID for PC1.5 UL CA in future meeting, then we are fine with Option 2 either. |
| vivo | Option 2. |
| Ericsson | Option 2 |

- Issue 1-2-2: Any other candidate(s) can be considered

- Proposals:

  - Option 1: PC 3, 20dBm + 23dBm for inter-band CA/DC. (OPPO)

  - Option 2: Others.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | It can be one of the scenarios to apply power high limit to. However, in order to handle option 1, what we need to do is just develop MSD requirement for PC5+PC5 if we reuse what we agreed in Rel-17, where MSD for PC3+PC3 is used for MSD for PC3+PC2 with power high limit while the development of the MSD requirements itself is out of scope of this WI. |
| Samsung | Option2 We understand the intention of Option1 might be one constituent single band achieve PC3 through TxD, but we wonder does this kind of implementation (achieve PC3 through TxD) indeed exist in practice? Perhaps for low-end UE? If so, considering low-end UE is also not necessary to support UL CA, we still do not see the need to define 20+23 inter band UL CA. |
| Apple | The feature can be introduced when such UL configuration is proposed. But it is not necessary to be handled in this WID as we commented above. The 2UL MSD framework is no different from nominal PC3 UL combinations. |
| OPPO | Option 1. This can be one of the scenarios that increase high limit can work, and extend easily from current spec. |
| Xiaomi | The other possible PC configuration is PC2 (FDD) without TxD+ PC3 (FDD/TDD) inter-band CA (noted currently there is no such combination for this case in the spec) |
| Huawei | Option 2. We also think it is not necessary to be discussed here since such UL configuration has not been proposed. |
| Meta | Option 2. In LTE, the PC combinations are discussed, but UE vendor did not have the specific PC combinations in real market. So we think that RAN4 do not need to define 20+23 inter band UL CA. |
| Skyworks | PC3 increased power based on 23+20dBm should be covered as CA already exist for NR+NRU inter-band cases |
| ZTE | Option 1 could be one of the scenarios. |
| vivo | Option 2. No candidate is currently deemed necessary. As commented in issue 1-1-1. |
| Ericsson | Option 2. The UE would also have to include PC3 for the band combination (if not included then PC3 is implicit since the default power class) |

Sub-topic 1-3: Intra-band CA/DC scenario for Rel-18 RAN4 study

Sub-topic description:

R4-2215892 (ZTE), R4-2216149 (Xiaomi) and R4-2216441 (OPPO) have proposals towards their understanding about the applicability of Rel-17 RAN4 work and the example cases that could be further considered in Rel-18 RAN4 study.

Open issues and candidate options before e-meeting:

- Issue 1-3-1: Whether intra-band CA/DC scenario needs to be considered

- Proposals:

  - Option 1: Yes.

    - Example case could be PC2: 23dBm (on CC1) + 26dBm (on CC2). (OPPO)

  - Option 2: No, and the reasons could be...

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 1. |
| Samsung | For Intra-band NC UL CA, as we commented in subtopic 1-1, we think directly define PC1.5 is more promising if requested/feasible. New PC is better to be handled in a new SI/WI to fully discuss the feasibility/impacted requirement rather than under this WI. |
| Apple | Option 2 For intra-band UL CA, the cells are expected to be collocated and PSD for the two UL CCs should be maintained equal. If the UL CA is composed by 2 PAs, each per CC, then PC3+PC3 may not be sufficient support PC2. With PC2+PC3, the PAs may need to be swapped based on the UL configurations. On the other hand, with PC2+PC3, there may not be much total output power gain as the PC3 PA would be the linearity bottleneck where the MPR due to RIMD could render diminishing return in total output power gain. |
| OPPO | Option 1. This is for 23+26 case which is same as inter-band UL CA introduced in Rel-17. It is understood that equal PSD is expected for the two CCs but with different RBs allocated for each CC the power of each CC still can be different. There is no harm to consider this case in the power enhancement. |
| Xiaomi | When defining PC2 intra-band CA requirements in R17, 23 dBm+26 dBm is considered, we are ok to discuss this example scenario. However, how to sum of the power class as there is no CC power class per band. |
| Huawei | Option 2. We think the typical deployment for intra-band UL CA is co-located, thus the gain seems questionable and it is not necessary to be considered here. |
| Meta | Option 2. However, this issue can be discussed in this WI based on update the objectives in this WI. |
| Skyworks | Intra band is out of scope in our view as it is not part of R17 and has impact on MPR/A-MPR which is not in scope. |
| ZTE | Option 1 could be one of the scenarios. However, the existing R17 approaches (sum power of each band) could not be reused. |
| vivo | Option 2. PC 1.5 is a more natural choice. |

Sub-topic 1-4: Other candidate(s) for Rel-18 RAN4 study scope

Sub-topic description:

R4-2216149 (Xiaomi) and R4-2216441 (OPPO) have proposals about further considering 3PA/3Tx/UL-MIMO cases in Rel-18 RAN4 study.

Open issues and candidate options before e-meeting:

- Issue 1-4-1: Whether ‘PC 2 single band UL-MIMO with PC3 PA + PC2 PA’ needs to be considered

- Proposals:

  - Option 1: Yes. (OPPO)

  - Option 2: No, and the reasons could be...

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 1 under the condition that intra band UL CA is also included in the objective. The reason is as follows. “PC 2 single band UL-MIMO with PC3 PA + PC2 PA” is a simple scenario in a way that the sum of the power sources(PA) is 27.8 dBm per cell. As far as the power sources and its delivered power are clarified in the spec, the sum is possible, and the sum can be used as PHR, though MPR/A-MPR re-evaluation is essential. “PC2: 23dBm (on CC1) + 26dBm (on CC2) for intra band CA” is also easy to apply power high limit in terms of power as intra band UL CA. However, as our paper mentioned, the achievable power per CC is different and these shall be visible to network since PHR is conducted per cell basis. Finally, the first example is just part of the 2nd example meaning that each of the power sources belong to the same CC. Hence, it is essential to discuss the 2nd example (intra band UL CA) as an enhancement. |
| Samsung | Option 2 For single band UL MIMO (without TxD), with current code book we assume the power is splitted equally between two Chains, if Option 1 is considered then new code book is also needed if my understanding correct. In addition, currently no FDD band support PC2 with MIMO, while for TDD band, we think it more promising to directly define PC1.5, just as Intra-band NC UL CA. |
| Apple | Option 2 As commented above, the PC3 PA would be the linearity bottleneck where the MPR due to RIMD could render diminishing return in total output power gain. On the other hand, if companies do expect the benefit of such feature, the work can be handled in FR1 UE RF enhancement WID instead of under this coverage enhancement WID as the feature not only involves capability signaling, but also new UE RF requirements. |
| OPPO | Option1, Share similar view as Nokia on the motivation of this enhancement. Currently there is no requirement on each Tx chain for UL MIMO instead the total is considered. Meanwhile, we understand that the power control is per CC, and are open to hear more views on this UL MIMO case. |
| Xiaomi | Similar comments as Issue 1-3-1. The question is how to sum of the power class as there is no PA configuration reporting. |
| Huawei | Option 2 We don’t think UL-MIMO should be considered in this coverage enhancement-related WI. |
| Meta | Option 2. However, this issue can be discussed in this WI based on update the objectives in this WI. |
| Skyworks | Increased power for this case is unclear as it would impact the power split and again MPR/A-MPR would be affected |
| vivo | Option 2 As commented in previous issue, UL-MIMO is quite different from what R17 increase higher power WI had impacted, and not suitable here. |
| Ericsson | Further clarification of this scenario is needed. If non-CA this appears similar to full-power UL-MIO mode 2. |

- Issue 1-4-2: Whether 3PA/3Tx architecture needs to be considered

- Proposals:

  - Option 1: Yes. (OPPO)

    - The 3PA architecture needs to be considered if PC 3 + PC 1.5 or PC 2 + PC 1.5 will be considered for inter-band CA/DC. (Xiaomi)

  - Option 2: No, and the reasons could be...

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 2: Clarification is needed. 3Tx architectures are used but the number of bands for simultaneous UL is still two? Is it common understanding that 3Tx architectures for PC3(1Tx) + PC2(2Tx) are covered by the existing specifications given that it is not proposed here? |
| Samsung | Option 2 3Tx/3PA work simultaneously is not in Rel-18 work scope as of now. If there is indeed practical UE implementation supporting 3Tx/3PA working simultaneously in current market, it shall comply with the 2Tx requirements. |
| Apple | Option 2 The 3Tx feature may not directly link to coverage enhancement. The feature may involve more UE RF requirements beyond coverage enhancement. Therefore, it should be considered in a separate WID. |
| OPPO | To clarify, our original proposal is for future release not Rel-18. The scenario includes: - 3Tx for two bands can be one of the scenarios that power enhancement can be applied, and is the straight forward extend of Rel-17 raising power feature with two inter-band UL CA. For example: PC3@band A + (PC3+PC3) @band B PC3@band A + (PC3+PC2) @band B PC3@band A + (PC2+PC2) @band B - 3Tx for single band, and the three PAs can be either PC3 or PC2 which forms the cases like below which cannot be covered by current power class definition: PC3+PC3+PC3 PC3+PC2+PC2 |
| Xiaomi | Prefer Option 2 |
| Huawei | Option 2 The 3Tx feature is beyond the scope of this WI, whether to consider this case shall be up to RAN plenary discussion. |
| Meta | Option 2. Similar view with Samsung. 3TX/PA shall be discussed in other SI/WI. |
| Skyworks | PC1.5 (one CC only with MIMO/TxD)+23 or 20dBm could be considered if 3Tx was allowed only for 1CC per band. |
| Nokia | To: OPPO Thank you for the clarification. Though companies would not admit, but RAN4 spec doesn’t prevent UE vendors from implementing the top case below. For example, if Band B supports PC2 thanks to TxD, and also if the UE reports PC3@band A and PC2@band B for single carrier/band, the UE just reports PC2 as inter band UL CA together with power high limit feature. The remaining cases are not covered by Rel-17 and in order to cover the 2nd case, power source handling needs to be addressed in a scenario of PC 2 single band UL-MIMO with PC3 PA + PC2 PA in advance. PC3@band A + (PC3+PC3) @band B PC3@band A + (PC3+PC2) @band B PC3@band A + (PC2+PC2) @band B And are UE with PC3+PC2 implementation in the fields? That is a question. At least we discussed e.g., ul-FullPwrMode2-TPMIGroup-r16 as if a UE supports this capability might implement at least one of the PAs can deliver 26 dBm and the other perhaps, 23 dBm. But in reality, do UEs implement two PAs with the same rated power? |
| ZTE | Prefer Option 2. Similar view with Apple, more Txs/PAs means more power can be summed, it is a separated topic. |
| vivo | Option 2. It is clear out of the possible scope, at lease in current stage. |

Companies views’ collection for 1st round

Open issues

- Sub topic 1-1

| Company | Comments |
| --- | --- |

- Sub topic 1-2

| Company | Comments |
| --- | --- |

- Sub topic 1-3

| Company | Comments |
| --- | --- |

- Sub topic 1-4

| Company | Comments |
| --- | --- |

CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1-1 Issue #1-1-1 | In total, 11 companies have provided their comments. While there are 6 companies (Samsung, Apple, Huawei, Meta, Vivo and Ericsson) agree on removing this objective from the WID and focus on another objective for MPR/PAR reduction, 3 companies point out their interested cases that could be further discussed. - Inter-band case: - PC3 BC with PC3 band 1 and PC5 band 2 (OPPO, Skyworks) - PC1.5 BC with PC1.5 band 1 and any PC band 2, but this calls for 3 transmitters and only for 1CC for both bands (Skyworks) - PC2 FDD band without TxD + PC3 TDD/FDD band (Xiaomi) - Intra-band case: - non-contiguous UL CA with different high power limit on each CC (Nokia, OPPO, Xiaomi) - contiguous UL CA with different high power limit on each CC (OPPO, Xiaomi) - UL-MIMO case: - PC2 PA + PC3 PA (OPPO, Xiaomi) Tentative agreements: TBA Candidate options: Recommendations for 2nd round: Check if this objective can be removed based on most companies’ view. |
| Sub-topic #1-2 Issue #1-2-1 | In total, 11 companies have provided their comments. ZTE think that Rel-17 higher power limit scheme shall be applied for this case since the current HPUE basket WID only includes PC1.5 with single TDD band, while all other 10 companies think that UE can report PC1.5 for such band combo, which is the usual way that already supported. Tentative agreements: ‘PC2: 26dBm (TDD) + 26dBm (TDD) without UL-MIMO’ does not need to be considered in this WID. Candidate options: Recommendations for 2nd round: Agree on the tentative agreement. |
| Sub-topic #1-2 Issue #1-2-2 | In total, 11 companies have provided their comments. For the PC3 inter-band BC with PC3 band 1 and PC5 band 2 case, 5 companies think it is not necessary to be considered, while 4 companies think in the other way. Main reasons from objectors can be listed as below: - It seems that this is for low-end UE (PC5), but low-end UE is not needed to support UL CA. (Samsung) - In LTE, the PC combinations are discussed, but UE vendor did not have the specific PC combinations in real market. (Meta) - There is no such kind of band combination in the current spec (Apple, Huawei) For the other two cases mentioned in the issue#1-1-1, further discussion seems needed. Tentative agreements: TBA Candidate options: - Case 1: PC3 band combination with PC3 band 1 and PC5 band 2 - Case 2: PC1.5 band combination with PC1.5 band 1 and any PC band 2 - Note: this calls for 3 transmitters and only for 1CC for both bands - Case 3: PC2 FDD band without TxD + PC3 TDD/FDD band Recommendations for 2nd round: If no conclusion can be reached for the first sub-topic, then discussion is needed for whether inter-band scenario should be included. |
| Sub-topic #1-3 Issue #1-3-1 | In total, 10 companies have provided their comments regarding the necessity for including intra-band scenario: - Unnecessary: 5 companies (Samsung, Apple, Huawei, Skyworks, Vivo) - Necessary: 3 companies (Nokia, OPPO, Xiaomi) - FFS: - Meta: Not necessary. However, this issue can be discussed in this WI based on update the objectives in this WI. - ZTE: Could be considered. However, the existing R17 approaches (sum power of each band) could not be reused. Main reasons from objectors can be listed as below: - Since for intra-band scenario the co-located deployment is assumed, the gain from different MOP for each CC might be questionable due to equalizing PSD which should be expected for that assumption. (Apple, Huawei) - PC1.5 could be a more natural choice (Samsung, Vivo) - Different MOP for each CC can lead to MPR/A-MPR change, which is out of the scope. (Skyworks) Tentative agreements: TBA Candidate options: - Case 1: non-contiguous UL CA with different high power limit on each CC - Case 2: contiguous UL CA with different high power limit on each CC Recommendations for 2nd round: If no conclusion can be reached for the first sub-topic, then discussion is needed for whether intra-band scenario should be included. |
| Sub-topic #1-4 Issue #1-4-1 | In total, 10 companies have provided their comments regarding the necessity for including UL-MIMO single band scenario: - Unnecessary: 5 companies (Samsung, Apple, Huawei, Skyworks, Vivo) - Necessary: 3 companies (Nokia, OPPO, Xiaomi) - FFS: - Meta: Not necessary. However, this issue can be discussed in this WI based on update the objectives in this WI. - Ericsson: Need clarification, seems full-power UL-MIMO mode 2 can be used for this scenario. Comments can be listed as below: - Current assumption on the equal splitting of transmission power between two chains will be violated (Samsung) - PC3 PA would be the linearity bottleneck where the MPR due to RIMD could render diminishing return in total output power gain (Apple) - MPR/A-MPR requires review, which is out of the scope (Skyworks) - PC1.5 for single band (Samsung) or full-power UL-MIMO mode 2 (Ericsson) can serve the purpose of increasing transmission power. - UL-MIMO should not be included in this coverage enhancement WI (Huawei) Tentative agreements: TBA Candidate options: - Case: single band UL-MIMO with 23dBm PA + 26dBm PA Recommendations for 2nd round: If no conclusion can be reached for the first sub-topic, then discussion is needed for whether single band UL-MIMO scenario should be included. |
| Sub-topic #1-4 Issue #1-4-2 | Most companies think this 3Tx/PA case is out of the scope. Tentative agreements: The ‘3PA/3Tx architecture’ does not need to be considered in this WID. Candidate options: Recommendations for 2nd round: Agree on the tentative agreement. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on enhancements of increasing UE power high limit | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2215793 |  | Power high limit and intra band UL CA | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215892 |  | Discussion on enhancement of increasing UE power high limit for CA and DC | ZTE Corporation | Noted |  |
| R4-2216120 |  | Discussion on enhancement of increasing UE power high limit for CA and DC | vivo | Noted |  |
| R4-2216149 |  | initial discussion on enhancement of increasing UE maximum power high limit | Xiaomi | Noted |  |
| R4-2216441 |  | R18 Discussion on power domain enhancement | OPPO | Noted |  |
| R4-2216588 |  | On further enhancement for NR UL coverage | Huawei, HiSilicon | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents