3GPP TSG-RAN WG4 Meeting #104-bis-e 						R4-2217185

E-meeting, 10th Oct. – 19th Oct., 2022

Title:	WF on performance part for NR NTN

Source:	Xiaomi

Agenda Item:	4.2.6

Document for:	Approval

- Introduction

According to the email discussion summary in [1], this document is to capture the WF on performance part for NR NTN RRM.

- Measurement accuracy requirements for NTN RRM

- Issue 1-1: Measurement accuracy

  - Agreement

    - Reuse the legacy TN measurement accuracy requirements for NTN.

      - Add brackets to the numbers

- Test case coverage, design and configuration

- Issue 2-1: Test cases for NTN specific requirements

  - Agreement:

    - The following TCs are considered for NTN specific requirements

      - 1-2 – 1-4, 1-6 – 1-8

      - 2-3 – 2-6

      - 4-1

      - 10-1 – 10-9, 11-1 – 11-9

      - 13-1 – 13-8

- Issue 2-2: Serving and Neighbour Satellite configurations.

  - Agreement:

    - Add a sub-clause in Annex to generate the reference for the motion trajectory for the virtual satellite used in the test cases.

  - Open issues:

    - High-level configuration for serving and neighbour Satellite are listed in Table 1.

    - FFS the detail model for the reference motion trajectory, and the inputs from satellite companies are needed.

Table 1: High-level configuration for serving and neighbour Satellite

|  | Qualcomm |  | CATT |  | Nokia |  | Xiaomi |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Characteristics | GSO satellites | Non-GSO satellites | GSO satellites | Non-GSO satellites | GSO satellites | Non-GSO satellites | GSO satellites | Non-GSO satellites |
| Altitude | 36,000km | 600km | 35786km | 600km | 35786km | 600km | 35786km | 600km |
| Motion trajectory | Cube (typically size based on satellite companies input) | circular orbit | Stationary (velocity = 0 ) | circular orbit | Cube (typically size based on satellite companies input) | circular orbit | Cube (typically size based on satellite companies input) | circular orbit |
| Elevation angle | (relative to UE position) > 30 deg during a test | (relative to UE position) > 45 deg during a test | > 30 deg | > 45 deg | (relative to UE position) > 30 deg during a test | (relative to UE position) > 30 deg during a test | (relative to UE position) > 30 deg during a test | (relative to UE position) > 45 deg during a test |

- Issue 2-3: Test case for earth-fixed cell and earth-moving cell

  - Agreement:

    - No differentiation of earth fixed cell or moving cell is made in the test cases

- Issue 2-4: Acquisition of UE location in RRM TCs

  - Agreement:

    - At least for test cases other than UE Tx timing, UE GNSS location is set by TE via “Update UE Location Information” procedure. No need to involve GNSS test setup in the NTN tests

- Issue 2-5: Reference propagator model

  - Open issue:

    - Option 1:

      - Introduce the reference propagator model;

    - Option 2:

      - No need to introduce the reference propagator model;

- Test case design for Cell reselection

- Issue 3-1: SMTC setup and scaling factor K_multi” in cell reselection tests

  - Agreement:

    - Use the following SMTC configurations for TC 1-1 – 1-4.

      - TC 1-1: serving cell in SMTC1, neighbor cell in SMTC2, SMTC1 and SMTC2 non-overlapping

      - TC 1-2: serving cell in SMTC1, neighbor cell in SMTC2, SMTC1 and SMTC2 overlapping

      - TC 1-3 and 1-4: serving cell in SMTC1, neighbor cell in SMTC1

      - Scaling factor “K_multi” is taken into account in the testing requirement for TC 1-1 – 1-4.

    - Use the same SMTC configuration as in A.6.1.1.2 for TC 1-5- 1-8, and scaling factor “K_multi” is not taken into account in the testing requirement.

    - The requirements in TC 1-3 and 1-4 should be based on UE capability: parallel measurements on more than one NGSO satellite within one SMTC; and different requirements are applied to different UE capabilities.

- Issue 3-2: ‘t-Service’ configuration in test case

  - Agreement:

    - ‘t-Service’ is configured in TC 1-3 and 1-7, but not configured in other TCs.

- Issue 3-3: Test setup for intra/inter-frequency cell reselection with timer trigger

  - Agreement:

    - TC 1-3 and 1-7 consist of two time periods T1 and T2:

      - Before test: UE camps in cell1, and t-Service is included in SIB19 of cell1

      - T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

      - T2: cell2 is powered on, T2 is [40]s, t-Service is pointed to the time point (start of T2 + [36s])

      - UE should reselect to cell2 before t-Service

- Issue 3-4: Test setup for intra/inter-frequency cell reselection with location trigger

  - Agreement:

    - TC 1-4 and 1-8 consists two time periods T1 and T2:

      - Before test: UE camps in cell1, and Ref-location is included in SIB19 of cell1

      - T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

      - T2: cell2 is powered on, T2 is [34]s, UE location is changed at the start of T2

      - UE should reselect to cell2 before time point (start of T2 + [34]s), assuming [2]s for the GNSS response time for the AT command.

        - FFS on the GNSS location change in the test shall be not less than distanceThresh + 50m.

- Issue 3-5: SIB19 reading time in test requirement

  - Agreements:

    - SIB19 reading time is included in TSI-NR.

    - FFS on the SIB19 scheduling period.

- Test case design for handover

- Issue 4-1: Test setup for CHO TC

  - Agreement:

    - It is proposed that same way as in current specification for TN system should be used in test cases for conditional handover for NTN, i.e. the network shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2

- Issue 4-2: Test case for CHO with time/location-based condition

  - Agreements:

    - Replace the original test cases with the case in which setting fulfil power based events and time/location based events at different time instances, choose 1 or 2 cases to apply this update.

- Test case design for UE timing requirements

- Issue 5-1: UE timing TC for 30 kHz SCS scenario

  - Agreements:

    - No need to define UE timing test configuration for FDD 30kHz SCS scenario for UE timing test cases.

      - 30 kHz test cases can be added when there is operator demand in the future.

- Issue 5-2: Acquisition of UE location in UE timing test cases

  - Open issue:

    - Option 1:

      - UE location is acquired by GNSS positioning, and the test parameter for GNSS signal power levels defined in B.4.1 is reused.

    - Option 2:

      - Use AT command approach to acquire UE location

        - AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates.

      - The exact UE position should be defined in such a way that the smallest elevation angle between the UE and satellite(s) is not smaller than 45 deg.

- Issue 5-3: Reference timing for uplink transmission in test cases

  - Open issue:

    - Option 1:

      - RAN4 to capture the following clarification on the reference timing for uplink transmission in Annex for RAN5 and TE vendors reference.

        - In RRM test cases, when a test equipment adjusts downlink transmission frame boundary/Doppler shift and UL reception timing, asymmetric propagation delays on DL and UL for the same slot index shall be taken into account. To model the round trip delay over service link (N_{TA,UE-specific}), the following definitions of reference slot for S3 and S4 (based in Fig. 3) are adopted.

          - for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

          - for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

    - Option 2:

      - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ±T_e_NTN

        - For the NTA,common and NTA,UE-specific  in the test requirement, the description should at least contain the clarification that UE GNSS estimation error and satellite positioning error from UE calculation are not involved.

    - Option 3:

      - For NTN UE timing testing, it is suggested to define a reference orbit for the serving satellite, and the DL timing shall be adjusted according to the distance change between serving satellite and UE.

      - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ±T_e_NTN

        - The UE specific TA for reference timing can be derived from the satellite location and UE location. The common TA for reference timing is calculated based on the definition in TS38.211.

- Test case design for L3 measurement requirements

- Issue 6-1: SMTC configuration for measurement delay TCs

  - Agreement:

    - RAN4 not to define the SMTC/satellite configuration with 2 SMTC per MO and each SMTC contains 2 SSB/Satellites.

    - For intra-frequency TCs (10-x),

      - Config.1: 2 SMTC per MO, each SMTC contains 1 SSB/Satellites

        - Config.1a: two SMTCs are overlapping

        - Config.1b: two SMTCs are non-overlapping

      - Config.2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites

    - For inter-frequency TCs (11-x):

      - Config.0: 1 SMTC per MO, each SMTC contains 1 SSB/Satellites

        - Config.0a: the SMTC for serving carrier and the one for neighbour carrier are overlapping

        - Config.0b: the SMTC for serving carrier and the one for neighbour carrier are non-overlapping

- Issue 6-2-1: SMTC configuration for FNO gaps

  - Agreements:

    - For FNO gaps, the associated SMTCs should be pattern 1 and Y defined in clause A.3.11.

- Issue 6-2-2: Whether to define test case for FO gaps if the core requirements are defined

  - Open issue:

    - Option 1:

      - Yes

    - Option 2:

      - No

- Issue 6-3: TC list for intra-frequency measurement delay

  - Agreement:

    - Update the TC list for intra-frequency measurement delay as follows

| 10-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 10-2 | Event triggered reporting tests without gap under DRX |
| 10-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 10-4 | Event triggered reporting tests with single measurement gap under non-DRX |
| 10-5 | Event triggered reporting tests with FNO concurrent measurement gap under DRX |
| 10-6 | Event triggered reporting tests with PPO concurrent measurement gap under non-DRX with SSB index reading |

- Issue 6-4: TC list for inter-frequency measurement delay

  - Agreement:

    - Update the TC list for inter-frequency measurement delay as follows

| 11-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 11-2 | Event triggered reporting tests without gap under DRX |
| 11-4 | Event triggered reporting tests with measurement gap under non-DRX |
| 11-5 | Event triggered reporting tests with measurement gap under DRX |
| 11-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading |
| 11-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 11-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

- Reference

[1]	R4-2217135,	Email discussion summary for [104-bis-e][202] NR_NTN_solutions_RRM_2