3GPP TSG-RAN WG4 Meeting # 104bis-e 												R4-2217214

Electronic Meeting, 10 – 19 October, 2022

Agenda item:			4.6.3, 4.6.3.1 and 4.6.4

Source:	Ericsson

Title:	WF on RedCap RRM requirements

Document for:	Approval

# Introduction

This way forward contains all agreements from RAN4#104-e in email thread: [104-bis-e][207] NR_redcap_RRM_1.

# Agreements from discussions on ‘General’

## Cell-specific RSRP offset

Cell-specific RSRP thresholds and their values:

| No | Type of threshold | Threshold [dB] |
| --- | --- | --- |
| 1 | rsrp-ThresholdSSB, | + 1 |
| 2 | msgA-RSRP-ThresholdSSB, | + 1 |
| 3 | msgA-RSRP-Threshold | + 1 |
| 4 | absThreshSS-BlocksConsolidation | + 1 |
| 5 | sdt-RSRP-Threshold | + 1 |
| 6 | s-SearchDeltaP-r16 | -1 |
| 7 | s-SearchDeltaP-Stationary-r17 | -1 |
| 8 | s-SearchThresholdP-r16 | + 1 |
| 9 | s-SearchThresholdQ-r16 | + 1 |
| 10 | s-SearchThresholdP2-r17 | + 1 |
| 11 | s-SearchThresholdQ2-r17 | + 1 |
| 12 | Qrxlevmin and Qqualmin | - 1 |

How to capture RSRP offset in specification

The application of the offset to the signaled cell-specific RSRP threshold is specified under each relevant requirement in TS 38.133 defined for the procedure using the the corresponding cell specific threshold(s).

## Small data transmission for RedCap

Clarification of SDT FR2 requirements

Given that R4-2214626 was agreed at RAN4#104-e meeting, no further updates needed.

Clarification of SDT FR1 requirements

No consensus to add further clarification to eDRX-RAN.

# Agreements from discussions on ‘Mobility requirements’

## Handover

Requirements for HO to a BWP which has different SSB with the one used for measurement

  - Option 1 (Ericsson, HW, Nokia, Apple, Intel, CMCC, OPPO, Xiaomi, HW): No additional delay is needed when UE handover to a BWP which has different SSB type with the one used for measurement.

  - Option 2 (QC, MTK): The scenario when handover is performed to a BWP which has different SSB than the one used during measurement should be considered as handover to an unknown cell. Capture the above condition as a note in the Handover related section in TS38.133

  - Option 2a (QC): When handover is performed to a known cell where the SSB associated with the first active BWP is different from the SSB used during intra-frequency or inter-frequency measurements of the target cell, then Tsearch = Trs  ms if the target cell is intra-frequency cell and Tsearch = 2* Trs  ms if the target cell in inter-frequency cell.”

  - Compromise (new) Option 3 (Ericsson, vivo, Apple): An additional delay is allowed if some conditions are met, such as the frequency difference between NCD-SSB and CD-SSB is larger than a threshold.

Option 4 (new) (Ericsson): Additional delay is allowed if the frequency difference between NCD-SSB and CD-SSB is larger than a threshold.

## RRC re-establishment

RRC reestablishment delay with RedCap specific initial BWP

When the Redcap specific initial BWP is configured for RA, extend RRC connection release with re-direction delay by X ms.

- TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH+X

- X = TBD

## RRC Connection release with redirection

RRC connection release with redirection delay with RedCap specific initial BWP

When the Redcap specific initial BWP is configured for RA, extend RRC connection release with re-direction delay by X ms.

- Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH + X

- X = TBD

# Agreements from discussions on ‘Timing requirements’

Requirements for the case SSB and MG overlapping

No consensus to update the existing Tx timing requirements to cover the scenario of active BWP without SSB and need for measurement gap for monitoring CD-SSB in case of MG sharing and large MGRP/SMTC.

Timing requirements when SSB is not in the active BWP

# Agreements from discussions on ‘Signaling characteristics’

Whether to define further relaxation (relaxed RLM/BFD) based on Rel-17 UE power saving WI for RedCap in Rel-17

No consensus to define further relaxation (relaxed RLM/BD) based on Rel-17 UE power saving WI for RedCap UE in Rel-17.

# Agreements from discussions on ‘Measurement procedure’

### Use of NCD-SSB for CONNECTED mode measurements

Serving cell threshold associated SSB

No need to clarify in the spec that serving cell thresholds of s-MeasureConfig for Connected mode should be checked based on reference SSB measurement.

# Agreements from discussions on RRM performance part

Whether to define test cases for NCD-SSB

RAN4 to define RRM test cases with NCD-SSB.

Agreement:

SSB Test configuration

- Intra-frequency measurement

| Test Index | Test | Test Configuration |
| --- | --- | --- |
| 1 | SA event triggered reporting tests for NCD-SSB without gap under non-DRX | d |
| 2 | SA event triggered reporting tests for NCD-SSB with per-UE gaps under non-DRX |  |
| 3 | SA event triggered reporting tests for NCD-SSB without gap under non-DRX with SSB index reading |  |
| 4 | SA event triggered reporting tests for NCD-SSB with per-UE gaps under non-DRX with SSB index reading |  |

- HO

| Test Index | Test | Test Configuration |
| --- | --- | --- |
| 1 | Intra-frequency handover from FR1 CD-SSB to FR1 CD-SSB; known target cell for 1 Rx UE | D by replacing NCD-SSB with CD-SSB |
| 2 | Intra-frequency handover from FR1 NCD-SSB to FR1 NCD-SSB; unknown target cell for 1 Rx UE | d |
| 3 | Inter-frequency handover from FR1 CD-SSB to FR1 NCD-SSB; unknown target cell for 1 Rx UE | D on the neighbour cell carrier |

- BWP switching

| Test Index | Test | Test Configuration |
| --- | --- | --- |
| 1 | DCI-based and Timer-based Active BWP Switch: NR FR1 DL active BWP switch of Cell with non-DRX in SA (1Rx, 2Rx) | c |
| 2 | RRC-based Active BWP Switch: NR FR1 DL active BWP switch of Cell with non-DRX in SA (1Rx, 2Rx) | Follow legacy BWP switch configuraitons but 40MHz BWP does not apply |