3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2217391

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.3.7.1, 4.3.7.2

Source:	Qualcomm

Title:	WF on FR2-2 UE demodulation requirements

Document for:	Information

# Way Forward

## General and PDSCH Demodulation Requirements

Maximum Testable SNR

Updated version of Table 7.2.3-2 in “TR 38.884 - Study on enhanced test methods for FR2 NR UEs v18.2.0”, which contains the Maximum DL testable SNR.

Table 7.2.3-2: Maximum DL testable SNR preliminary extension for band n263 CBW (MHz); Test method. IFF. Single band UE; 100; [9.8]. 400; [2.6]. 800; [-2.3]. 1600; < -20 (NOTE 1). 2000; < -20 (NOTE 1). Multi band UE; 100; TBD. 400; TBD. 800; TBD. 1600; TBD. 2000; TBD. NOTE 1: Result does not converge.

Extended maximum Testable SNR for reduced allocation

- Option 1:

|  | CBW (MHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 | [9.8] |
|  |  | [32] | [13.2] |
|  | 400 | 66 | [2.6] |
|  |  | [32] | [6.6] |
|  |  | [20] | [8.9] |
|  |  | [16] | [10] |

- Option 2:

|  | CBW (MHz)/SCS(kHz) | Num RBs | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100/120 | 66 | [9.8] |
|  |  | 32 | [13.2] |
|  | [400/120] | [TBD] | [TBD] |
|  | 400/480 | 66 | [2.6] |
|  |  | 32 | [6.6] |
|  |  | 20 | [8.9] |
|  |  | 16 | [10] |

Whether to introduce requirements with 400MHz CBW for 120kHz SCS for single CC

- Option 1: Yes

- Option 2: No;

Whether to introduce requirements with 800MHz CBW for 480kHz SCS

No;

Whether to introduce requirements with 960kHz SCS

RAN4 focus on 120kHz and 480kHz SCS for introducing UE demodulation requirements in FR2-2. It’s not precluded to discuss and introduce requirements for 960kHz SCS in future releases.

Whether to introduce SDR Test for FR2-2 in Rel. 17

Agreement: Not define SDR test for FR2-2 in Rel-17. It’s not precluded to further discuss in future releases.

Whether to define PDSCH requirements with 30% of peak throughput

Add one test case for 120kHz case only with 30% peak throughput

Whether to introduce requirements with 650Hz Doppler Frequency:

Yes;

Design Margin for PN models in TR 38.808 Set 1.

According to the PN for UE, Example 1 in 38.808, use: BS Margin = 0dB; UE margin 5dB;

## PDSCH Requirements

\MCS Choice for 120kHz/100MHz for 70% requirements

MCS 4, 13, 17 are agreed; Further discuss for MCS 20;

Whether to introduce PDSCH requirements with MCS20 in FR2-2 for 120kHz/100MHz

- Option 1: Yes;

- Option 2: No;

Whether to introduce PDSCH requirements with MCS17 in FR2-2 for 480kHz/400Mhz

No;

MCS Choice for 480kHz/400MHz for 70% requirements

MCS 4, 13 are agreed;

RB allocation for 120kHz/100MHz for MCS 4, 13 for 70% requirements

- For 120kHz/100MHz, agree on 66RBs (full) allocation for MCS at least up to 13;

RB allocation for 120kHz/100MHz for MCS 17 for 70% requirements

- Option 1: 32RBs;

- Option 2: 66RBs;

RB allocation for 120kHz/100MHz for MCS 20 for 70% requirements

- Option 1: 16 RBs;

- Option 2: 32/33 or 66RBs;

- Option 3: Don’t include MCS 20 for 120kHz;

RB allocation for 480kHz/400MHz for MCS 4 for 70% requirements

66 RBs;

RB allocation for 480kHz/400MHz for MCS 13 for 70% requirements

- Option 2: 16 RBs;

- Option 3: 20 RBs;

RB allocation for 480kHz/400MHz for MCS 17 for 70% requirements

Don’t include MCS 17 for 480kHz;

Propagation Channels for PDSCH Requirements for 120kHz/100Mhz:

TDLA30-650 for MCS 4 and MCS 13 (30%);

TDLA30-200 for MCS 13 (70%);

TDLD30-200 for MCS 17 and above;

Propagation Channels for PDSCH Requirements for 480kHz/400Mhz:

For PDSCH Requirements for 480kHz/400Mhz: following channel model adopted

- TDLA10-200 for MCS 4;

- TDLD10-200 for MCS 13 and above;

MCS and RB allocation choice for 120kHz/100MHz for 30% Requirements

Recommended WF: MCS 13, 66RBs;

PDSCH Requirements Table

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB |
| --- | --- | --- | --- | --- | --- |
| 120kHz/100MHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | [66, 32] |
|  | MCS20 | TDLD30-200 | 2x2 Low | 70% | [32] |
| [120kHz/400MHz] | [TBD] | [TBD] | 2x2 Low | [TBD] | [TBD] |
| 480kHz/400MHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | [16,20,32] |

## PDSCH Requirements for CA

Bandwidth choice and test setup for FR1+FR2-2 CA requirements

- Option 1:

  - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

  - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC).

  - CQI requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is 66RBs

- Option 2:

  - 100MHz/120KHz

    - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

    - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC).

    - CQI requirements: FR1(40MHz,single CC)+FR2-2(100MHz,single CC). The RB allocation for FR2-2 CC is 66RBs

  - 400MHz/480KHz

    - PDSCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is aligned with single CC requirements configuration.

    - PDCCH requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC).

    - CQI requirements: FR1(40MHz,single CC)+FR2-2(400MHz,single CC). The RB allocation for FR2-2 CC is 66RBs

Applicability rule for FR1+FR2-2 CA requirements

- Option 1:

  - Copy all cases of single CC to CA and define applicability rules as follows:

  - If UE support standalone operation, UE can only be tested cases with single CC.

  - If UE don’t support standalone operation, UE can only be tested cases with CA

Configurations for FR1 CC for FR1+FR2-2 CA requirements

- Companies can provide configurations for FR1 CC for FR1+FR2-2 CA requirements next meeting.

Test cases for FR1+FR2-2 CA requirements

- Option 1: Reuse all test cases of single CC to CA requirements

How to define FR1+FR2-2 CA requirements

- Only define the requirements for FR2-2 CC

- Option 1: Apply all the requirements of single CC to CA requirements

- Option 2: Define separate requirements for CA

- Companies can provide simulation results or analysis next meeting to support your views

## PDSCH Simulation Assumptions

Whether the TE transmits OCNG on PDSCH slots during PDSCH testing

No;

Whether to schedule PDSCH in slots that contain TRS symbols during PDSCH testing

Tentative agreement for simulation alignment: No, pending feedback from TE vendors;

Whether the TE transmit TRS in the full CBW during PDSCH testing

- Option 1: Yes, further discuss the impact on TRS SNR;

- Option 2: Transmit TRS in unallocated PRB only if PDSCH is not multiplexed with TRS (see Issue 1-2-2), or otherwise if there is no impact on maximum SNR;

- Option 3: Schedule same RB size for TRS and PDSCH;

- Option 4: Refer to TE recommendation;

Partial bandwidth allocation position in the CBW

Center;

Choice of Noh PRB for TBS calculations.

NOHPRB = 6;

70% Peak Throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz, 480 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM). |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

30% Peak throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [KHz] | 120 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |
| NOH | 6 |
| Number of HARQ Processes | 16 |

# Topic #2: PDCCH and PBCH Requirements

## PDCCH Requirements Table

Channel Model for PDCCH Requirements using 120kHz SCS

- Option 1: TDLA30-200;

- Option 2: TDLA30-650;

Interested companies are encouraged to provide alignment and impairment results for both options in RAN4#105.

| Test number | CBW/ SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | [TDLA30-200] | 1_0 | 40 | 1x2 Low |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | [TDLA30-200] | 1_1 | 56 | 1x2 Low |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | [TDLA30-200] | 1_1 | 56 | 2x2 Low |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | [TDLA30-200] | 1_0 | 40 | 2x2 Low |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low |

## PBCH Requirements Table

Channel Model for FR2-2 PBCH Requirements Test 1-1 (120kHz SCS);

- Proposals

  - Option 1: TDLA30-200;

  - Option 3: TDLA30-650;

Interested companies are encouraged to provide alignment and impairment results for both options in RAN4#105.

| Test number | SCS/CBW | Antenna configuration | Propagation condition | PBCH SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120kHz/100MHz | 1x2 Low | [TDLA10-200] | New | unknown |
| 1-2 | 480kHz/400MHz |  | TDLA10-200 | New | unknown |

# Topic #4: CSI Reporting Requirements

Whether to define CQI requirements including 16QAM

Include 16QAM;

Rank to use for the definition of CQI Requirements

- Option 1: Rank 1;

- Option 2: Rank 2;

Whether to define CQI requirements including 64QAM

- Option 1: Yes;

- Option 2: No

SNR Test points for 64QAM

- Option 1: 8/9 dB with Rank 1;

- Option 2: Use Rank 2 as in FR2-1 and do not consider 64QAM;

SNR Test points for 16QAM

- Option 1: 8/9 dB;

- Option 2: 2/3 dB;

SNR Test points for QPSK

- Option 1: -2/-1 dB;

- Option 2: 0/1 dB;