3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216031

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results for RedCap RI requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.6.5.2.2

Document for:	Discussion

Background

As per [1], RAN4 agreed to define the requirements for RedCap RI requirements.

Agreement: Whether to define RI reporting requirements for RedCap 2Rx UEs - Define RI reporting requirements for RedCap 2Rx UEs - Apply Test 2 only - Replace fading CQI test for 2 Rx UE (high SNR point) by RI test case (Test 2)

In this paper, we provide our simulation results.

Simulation results

The simulation assumptions are captured in follows:

- FR1

- FDD:

- CSI-RS periodicity and offset is 10/1,

- CSI reporting periodicity and offset is 10/9,

- CSI delay is 14ms.

- CQI table 1

- Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1

- TDD:

- CQI table 1

- CBW/SCS: 20MHz/30kHz

- Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1

- FR2: Reuse the requirements

The simulation results are captured in Table 2-1:

Table 2-1: Simulation results for FR1 RI test

| SNR(dB) | TP ratio: Follow RI/Fixed RI=1 |  |
| --- | --- | --- |
|  | FDD/HD-FDD | TDD |
| 16 | 1.36 | 1.50 |
| 17 | 1.30 | 1.43 |
| 18 | 1.33 | 1.56 |
| 19 | 1.41 | 1.58 |
| 20 | 1.46 | 1.63 |

Conclusion

In this paper, we provide our simulation results for RedCap RI test.

Reference

[1]   WF on RedCap demodulation and CQI reporting requirements. Ericsson