3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217472

Electronic Meeting, 10 ‒ 19 Oct, 2022

Title: 	WF on BS RF requirements for LTE based broadcast

Agenda Item:	     7.4.5

Source: 	ZTE

Document for:	Approval

GTW agreement

Issue 1-1: BS type

- Agreement: Option 1 agreed

Issue 1-2: BS classes

- Agreement: Postpone the decision of BS classes pending on the investigation of RF requirements and scenario assumptions for regulatory requirements.

Issue 1-4: BS ACLR requirement

- Agreement: Further discuss based on the analysis on the regulatory requirements.

Issue 1-5: Regulatory requirements

- Agreement: Baseline assumption is using reference approach instead of explicitly list the regulatory requirements into the spec; further work on the details together with the discussion on the RF requirements.

Other RF requirements

|  | ZTE R4-2216550 | Nokia R4-2216735 | Recommended WF |
| --- | --- | --- | --- |
| BS output power | up to the declaration and no need to define any requirements | No change is expected except some regulatory requirements might be referenced in this Clause | No change is expected except some regulatory requirements might be referenced in this Clause |
| Output power dynamics |  |  |  |
| RE power control dynamic range | N/A, we don’t expect to have the RE power control to alleviate the co-channel interference by this approach | RE power control dynamic range to be further checked if it can be applied for LTE based 5G terrestrial broadcast band(s) | FFS |
| Total power dynamic range | N/A, we don’t expect the vision/sound signal of broadcast would be transmitted in the partial PRBs. | Total power dynamic range to be defined for new channel bandwidths | FFS |
| Transmit ON/OFF power | N/A | N/A | N/A |
| Transmitted signal quality |  |  |  |
| Frequency error | to reuse the same requirement defined in TS 36.104 |  | FFS |
| Modulation quality | The supported the modulation order up to 256QAM and reuse the EVM requirement in TS 36.104. 1024QAM should not be applicable for LTE based broadcast. | At least EVM window length to be addressed for new channel bandwidths and relevant sub-carrier spacings (Annex E in 36.104) | FFS |
| Time alignment error | FFS, this depend whether MIMO scheme is supposed to be supported for LTE based broadcast. If not, then this requirement is also not needed. |  | FFS |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | to reuse the same requirement defined in TS36.104 which is following ITU-R Recommendation SM.328 |  | to reuse the same requirement defined in TS36.104 which is following ITU-R Recommendation SM.328 |
| Adjacent Channel Leakage Power Ratio | this depends on the outcome of coexistence study. | Confirm and define ACLR for new channel bandwidths | Further discuss based on the analysis on the regulatory requirement |
| Operating band unwanted emissions | this depends on the outcome of coexistence study and ITU/regional regulation which was captured in [4]. Further consideration how to accommodate these requirement into the TS 36.104 | Define/clarify OBUE for new channel bandwidths, clarify which Category and BS classes would apply Address regulatory requirements in additional OBUE requirements | Further discuss based on the analysis on the regulatory requirement |
| Transmitter spurious emissions | FFS Coexistence requirement for the protection of other TN BS receiver should be considered. The legacy value is not applicable due to its aggressor transmitter power and potential MCL assumption. | Include new LTE based 5G terrestrial broadcast band(s) if co-location with existing 3GPP bands is considered Pending band definition decision, confirm existing 3GPP bands will be protected by new LTE based 5G terrestrial broadcast band(s) in Clause 6.6.4.3, and 6.6.4.4 if co-location with existing 3GPP bands is considered Address regulatory requirements in additional spurious emissions requirements | Include new LTE based 5G terrestrial broadcast band(s) if co-location with existing 3GPP bands is considered |
| Transmitter intermodulation | Not applicable for MPMT and HPHT since it’s not expected to have surrounding interfering gNB. FFS for LPLT. If the practical deployment, there are also no surrounding interfering gNB next to the victim BS, then this requirement should be also not applicable. | No change is expected except some regulatory requirements might be referenced in this Clause | FFS |
| 7.6.2 Co-location with other base stations |  | Include new LTE based 5G terrestrial broadcast band(s) if co-location with existing 3GPP bands is considered | Include new LTE based 5G terrestrial broadcast band(s) if co-location with existing 3GPP bands is considered |