3GPP TSG-RAN WG4 Meeting #104-bis-e									                 R4-2217513

Electronic Meeting, 10– 19 October 2022

Agenda item:			6.17.4

Source:	Qualcomm Inc

Title:	WF on SBFD feasibility study and RF impact: UE aspects

Document for:	Approval

# Scope

This WF discusses UE feasibility aspects from 310

# UE TX modelling aspects

This WF discusses UE feasibility aspects from 310

## Adjacent channel model

### UE TX aggressor toward adjacent channel victim (FR1)

What base value for ACLR1 in TX model for FR1 power class 3?

Agreement:

- 30 dB is the total distortion power on either side of a fully allocated uplink sub-band. The ACLR1 distortion PSD is modeled as flat over that range (From the agreement below)

- FFS whether we need to consider whether we need to model allocations that are less than fully  allocated uplink sub-bands

What base value value should the model use for FR1 PC3 ACLR2?

Agreement:

- Follow Ericsson suggestion and evaluate the effect of UE-UE CLI with ACLR1 only.

- Revisit the discussion on ACLR2 if UE-UE CLI becomes significant

Do we need to model TX power classes other than FR1 PC3?

Agreement: power class 3 only

What is the frequency resolution (granularity) of the model?

Agreement: Distortion is modeled as a flat power spectral densitity across the frequency range of the distortion

Should the ACLR-based interference be scaled with backoff?

Agreement:

- Do not model improved ACLR with backoff

- Revisit the discussion on backoff-dependent ACLR if UE-UE CLI becomes significant

### UE TX aggressor toward adjacent channel victim (FR2-1)

Should the model use ACLR or OBW as the base value?

Agreement: For FR2-1 use OBW as basis (23 dB)

Should the FR2-1 model include an ACLR2-type aspect, similar to FR1?

Agreement: ACLR-2 model aspect is precluded for FR2-1

## Co-channel model

### UE TX aggressor toward co-channel victim (FR1)

ACLR or IBE based model

  - Agreement: Use  IBE-based model for co-channel

What is the frequency domain granularity of the  IBE-based model

Agreement: IBE-based model granularity is 1 RB.

IBE-based model inclusion of image and LO location assumption

Agreement: The IBE-based model should Include the image aspect of IBE and assume the LO is in the middle of the channel to allow for correct placement of the image frequency.

### UE TX aggressor  toward co-channel victim (FR2-1)

Agreement: Use the same approach as in adjacent channel aggressor model for FR2-1

# UE RX modeling aspects

## Adjacent channel model

### UE RX victim from adjacent channel aggressor (FR1)

ACS value as one performance point in the model

Agreement: agree 33 dB value (33 dB comes from ACS) as performance point in the RX model

RX model with adjacent channel blocker over the RX dynamic range

Agreement: If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss)

### UE RX victim from adjacent channel aggressor (FR2-1)

ACS value as one performance point in the model

Agreement: agree 23 dB value (from ACS) as performance point in the FR2-1 model

RX model with adjacent channel blocker over the RX dynamic range

Agreement: For FR2-1 Use the same method as in FR1, with changes being related to the parameters of ACS value, REFSENS, and maximum input power level

## Co-channel model

### UE RX victim from co-channel aggressor (FR1)

Receiver sub-band selectivity

Agreement:

- FFS with below candidate options for further consideration:

  - Option 1: 0 dB without any rejection/attenuation on interference in adjacent sub-band

  - Option 2: Something based on 33 dB FR1 ACS but the details are not clear

  - Option 3: Typical performance model

  - Other options not precluded

- FFS for the sub-band definition from UE perspective for SBFD operation

- Further discuss the definition of sub-band selectivity

Configuring the UE channel bandwidth to be equal to a sub-band for selectivity

Agreement: FFS whether UE channel bandwidth be configured to equal the sub-band BW for SBFD operation from UE perspective

Effect of power contained in uplink sub-band on receiver model (blocker)

Agreement:

FFS for the effect of power contained in uplink sub-band on receiver model

- One proposed model from company as following in R4-2216794:

  - x axis is total power in the channel at the receiver input, so signal + any uplink jammer or blocker power. Let’s call it Pin.

  - y axis is the ratio of total input channel power to noise, so it is Pin/noise power

  - The receiver performance breaks down above the maximum input power level, so the receiver would not be able to demodulate the signal at all in this regime

- Other models not precluded

### UE RX victim from co-channel aggressor (FR2-1)

Agreement: For FR2-1 use the same method as in co-channel RX victim for FR1. Note that the co-channel RX victim method for FR1 has not been agreed yet.