3GPP TSG-RAN WG4 Meeting # 104bis-e                         R4-2217708

Electronic Meeting, October 10 – October 19, 2022

Title: 	WF on CA_n5-n8

Source: 	Xiaomi

Agenda item:	6.2.2

Document for:	Approval

- Introduction

This WF is to capture the agreements and remaining issues for CA_n5-n8.

- Way forward

Issue 2-1: Whether 2 antenna architecture is down selected for the requirements study

Issue 2-2: Which architecture is based for the UE RF requirement if two architectures are allowed?

GTW Agreement:

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 2-3: RF parameters for requirements analysis

- Proposals

The following parameters are needed for the feasibility study:

    - n5 Tx filter attenuation at n8 Rx frequency range

    - n5 Rx filter attenuation level at n8 Tx frequency range

    - n8 Tx filter attenuation at n5 Rx frequency range

    - n8 Rx filter attenuation level at n5 Tx frequency range

    - antenna ISO

    - RF front end loss

- Agreements

    - Companies are encouraged to provide the RF parameters when the feasibility study is conducted.

    - The parameters are not limited to above.

Issue 2-4: Whether the filter can be dedicated, based on the restricted frequency range.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Agreements

    - Both single band filter and dedicated filter can be used in the study.

Issue 2-5: Possible solutions for CA_n5-n8

- Proposals

  - Option 1: n8 Tx restricted RBs

  - Option 2: n8 TX power reduction

  - Option 3: non-simultaneous Rx/Tx between n5 DL and n8 UL

  - Option 4: restrict to n5 UL only for 1UL/2DL NR CA_n5-n8

- Agreements

    - All of the solutions can be candidates at current stage. The detail solution for the corresponding RF architecture can be analyzed and discussed in future meetings.

    - The above solutions are for full filters of n5/n8.

Issue 2-6: IMD for 2UL CA_n5-n8

- Proposals

  - Option 1: No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

- Agreements

    - No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

    - MSD due to n5 DL and n8 UL overlapping is further studied in both 1UL and 2UL cases.

    - This does not prevent from agreeing on any solutions given in Issue 2-5.

- Reference

- R4-2216961, Email discussion summary for [104-bis-e][125] FS_NR_700800900, Moderator (CATT)

- R4-2215662, On CA_n5-n8 architectures, Apple, RAN4#104bis-e

- R4-2216340, Considerations on CA_n5-n8, Qualcomm, RAN4#104bis-e

- R4-2215445, CA_n5-n8 Feasibility and assumptions for UE-UE RF Requirements, Murata, RAN4#104bis-e

- R4-2215896, Feasible on CA band combination of n5-n8, ZTE Corporation, RAN4#104bis-e

- R4-2216039, Discussion on feasibility study for CA_n5-n8, Xiaomi, RAN4#104bis-e

- R4-2216136, Further discussion on feasibility aspects for 700800900MHz CA band combinations, vivo, RAN4#104bis-e

- R4-2216679, CA_n5-n8 with restricted frequency range, Skyworks, RAN4#104bis-e

- R4-2215897, RF requirements on CA band combination of n5-n8, ZTE Corporation, RAN4#104bis-e

- R4-2216072, TP and discussion on CA_n5-n8, Huawei, HiSilicon, RAN4#104bis-e