3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217744

Electronic Meeting, 10 ‒ 19 October, 2022

Agenda item:			6.23.4

Source:	Huawei, HiSilicon

Title:	WF on enhancements of increasing UE power high limit

Document for:	Approval

Introduction

In this meeting, RAN4 discussion focuses on identifying what specific scenarios could be studied regarding the following highlighted objective that belongs to the Rel-18 WID on further NR coverage enhancements:

- Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

Conclusion can be found as in the rest of this WF.

Way forward

- Sub-topic #1-1: The neccesity of this objective

#### Issue 1-1-1: Whether to remove this objective

Agreement:

FFS on the other issues and check whether if the discussion for this objective in RAN4 level could be stopped based on companies’ view in the next meeting.

- Sub-topic #1-2: Inter-band CA/DC scenario for Rel-18 RAN4 study

#### Issue 1-2-1: Whether ‘PC2: 26dBm (TDD) + 26dBm (TDD) without UL-MIMO’ needs to be considered

Agreement:

- ‘PC2: 26dBm (TDD) + 26dBm (TDD) without UL-MIMO’ does not need to be considered in this WID.

#### Issue 1-2-2: Any other candidate(s) can be considered

- Case 1: PC3 inter-band BC with PC3 for band 1 and PC5 for band 2

Agreement:

- Postpone the discussions for PC3 inter-band BC with PC3 for band 1 and PC5 for band 2 until receiving the demand from operators

- Case 2: PC1.5 band combination with PC1.5 band 1 and any PC band 2

  - Note: this calls for 3 transmitters and only for 1CC for both bands

Agreement:

- The case ‘PC1.5 band combination with PC1.5 band 1 and any PC band 2’ does not need to be considered in this WID.

- Case 3: PC2 FDD band without TxD + PC3 TDD/FDD band

Agreement:

- The case ‘PC2 FDD band without TxD + PC3 TDD/FDD band’ does not need to be considered in this WID.

- Sub-topic #1-3: Intra-band CA/DC scenario for Rel-18 RAN4 study

#### Issue 1-3-1: Whether intra-band UL CA scenario needs to be considered

Agreement:

- Further study on the case ‘intra-band contiguous/non-contiguous UL CA with PC2 on CC1 and PC3 on CC2’.

- Sub-topic #1-4: Other candidate(s) for Rel-18 RAN4 study scope

#### Issue 1-4-1: Whether ‘PC 2 single band UL-MIMO with PC3 PA + PC2 PA’ needs to be considered

Agreement:

- Further study on the case ‘PC 2 single band UL-MIMO with PC3 PA + PC2 PA’.

#### Issue 1-4-2: Whether 3PA/3Tx architecture needs to be considered

Agreement:

- 3Tx architecture should not be considered in this WID.