3GPP TSG-RAN4 Meeting #104Bis-e	R4-2217749

Oct 2022

Title:	TP for NB-IoT UE Frequency Error for 36.102

Source:	Qualcomm Incorporated

Agenda item:	7.5.5

Release:	Rel-18

WI: 	LTE_NBIOT_eMTC_NTN_req-Core

Lead WG: 	RAN4

Document for:	Approval

# 1.	Introduction

For IoT NTN, the specification skeleton was approved [1]. This paper proposes frequency error in accordance with the moderator summary after 1st round for thread [143] in RN4#104Bis-e.

# 2. 	Discussion

This

# Conclusion

Proposal: Use suffixes on 2nd level in the specification 36.102 and approve the TP in appendix.

# References

[1]	R4-2215118, “Draft spec for TS 36.102”, Mediatek India Technology Pvt., RAN4#104e, Aug 2022

[2]	R4-2215779, “Discussion on UE RF requirements for IoT NTN”, MediaTek, RAN4#104Bis-e, Electronic Meeting, Oct 10 – Oct 19, 2022

# Text proposal

### 6.4.1B	Frequency error for UE category NB1 and NB2

For UE category NB1 and NB2, the UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 clause 23.21.2.2).

The UE pre-compensated modulated carrier frequency shall be accurate to within the limits in Table 6.4.1B-1, observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to [the ideally pre-compensated reference uplink carrier frequency signalled to the UE by the SAN].

When a repetition period is configured on the uplink for which repetition period (R ) >1, the UE shall not change Doppler pre-compensation during an ongoing repetition period except in the gaps or in between segments as defined in clause 10.1.3.6 of TS 36.211.

Table 6.4.1B-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency (GHz) | Frequency error (ppm) |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

### 6.4.2	Transmit modulation quality