3GPP TSG-RAN WG4 Meeting # 104Bis-e	 R4-2217739

Electronic Meeting, October 10 ‒ 19, 2022

Title: 	[DRAFT] WF on expanded and improved NR positioning – UE RF aspects

Agenda Item:	6.18.5

Source: 	Intel Corporation

Document for:	Approval

# Accuracy improvement study based on PRS/SRS bandwidth aggregation

## RF architecture

Agreement:

- Prioritize the single RF chain (Tx/Rx) for BS and UE in the study.

## PRS/SRS bandwidth aggregation scenario

Agreements:

- Prioritize intra-band contiguous CA with simultaneous PRS/SRS transmission for the RF and RRM impacts study.

- CA configurations with 2, 3 and 4 CCs should be investigated and the configuration with 2 CCs should be prioritized over 3 and 4 CCs.

- The TAE and group delay need be studied. In addition, phase noise needs be studied for FR2.

## RF impairment model and assessment

Tentative Agreement:

- Proposal 1: RAN4 to evaluate the impact of group delay on the performance of PRS/SRS aggregation covering PRS/SRS resources with both same PRS bandwidth and different bandwidths

For further discussion in GTW:

  - Incorporating the RRM agreement (R4-2217257) on aggregation of PFLs with different bandwidths to Proposal 1

    - PRS resources to be aggregated for MC positioning measurements from different PFLs can have different bandwidths (i.e., different number of PRS RBs)

  - Studying TAE for single RF chain architecture

    - For single RF chain architecture TAE between PFLs/carriers transmitted from same antenna is negligible and therefore no need to define TAE requirement

    - For single RF chain architecture TAE between PFLs/carriers transmitted from different antennas is FFS

    - Confirm whether no additional study of relative timing error or frequency errors between signals/carriers is needed

## Achievable accuracy gain study for intra-band contiguous CA

For further discussion in GTW:

- Whether to study non-coherent aggregation as a second priority

Agreement/WF:

- TBD

## Baseline assumption for FFT processing

Agreement:

- The FFT assumption should be discussed in RRM part.

## Notifying RAN1 of UE transmit power limitation

Agreement:

- Notifying RAN1 on UE transmit power limitation due to prioritization of PCell over SCell is not needed at this point in time.

# Accuracy improvement study based on carrier phase measurements

## RAN4 study on RF requirement

Agreement:

- RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement.

## Scope of RAN4 study

Agreement:

- No further discussion is needed in RAN4 at this time

# References

- R4-2217790, “Email discussion summary for [104-bis-e][138] FS_NR_pos_UERF,” Moderator (Intel Corporation), RAN4 #104Bis-e, October 2022