3GPP TSG-RAN WG4 #105	R4-2218243

Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.2.6.3.1

Source:	MediaTek Inc.

Title:	Simulation results for NR-NTN PDSCH requirements

Document for:	Discussion

- Introduction

In RAN4#104-bis-e, RAN4 agreed on the propagation channel model for NR-NTN PDSCH demodulation requirements [1]. In this contribution, we provide our simulation results for NR-NTN PDSCH demodulation requirements based on the agreed channel model.

- Discussion

The simulation results in Table 3 are based on the propagation channel model defined in Table 1 and 2.

Table 1: NTN-TDLA100 (DS = 100 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table 2 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

Table 3: SNR requirements for NR-NTN PDSCH Demodulation

| Prop. Channel | MCS | HARQ Config | Alignment SNR (dB) |
| --- | --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ | -1.0 |
|  | MCS4 | 16 HARQ Proc | -1.7 |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc | -2.7 |
|  | MCS13 | 16 HARQ Proc | 5.0 |
| Note: The Doppler shift for LOS Path in NTN-TDLC5 Channel is (0.7*fD), with AoA of 45 degree. |  |  |  |

- Conclusion

In this contribution, we provided our simulation results for NTN PDSCH based on the agreed propagation channel model.

- Reference

- R4-2217344, “WF for NTN demodulation requirements - general and PDSCH”, Qualcomm